Ribble Valley Borough Council

### DATE INSPECTED: 28 April 2010

DELEGATED ITEM FILE REPORT - APPROVAL

| Ref: CS/CMS |  |
| --- | --- |
| Application No: | 3/2010/0254/P |
| Development Proposed: | Proposed extension to east elevation approximately 5.45m long x 1.9m wide x 4.34m high at Terra Cottage, Kayley Lane, Chatburn |
| # CONSULTATIONS: Parish/Town Council |  |
| Parish Council – Has viewed this application and has no observations to make. |  |
| # CONSULTATIONS: Highway/Water Authority/Other Bodies |  |
| Environment Directorate (County Surveyor) – N/A. Environment Agency – N/A. |  |
| ## CONSULTATIONS: Additional Representations. |  |
| No representations have been received. |  |
| RELEVANT POLICIES: |  |
| Policy G1 - Development Control. Policy H10 - Residential Extensions. Policy SPG – “Extensions and Alterations to Dwellings” |  |
| COMMENTS/ENVIRONMENTAL/AONB/HUMAN RIGHTS ISSUES/RECOMMENDATION: |  |
| Terra Cottage is a bungalow on the inside of a bend on the north-west side of Kayley Lane close to its junction with Downham Road within the Chatburn Conservation Area. The main front elevation (on which there is a porch) is the south elevation. The eastern side elevation, however, also faces Kayley Lane. This side elevation ranges from 2.8m to 6m away from the eastern boundary of the site to Kayley Lane. The boundary is presently defined by a stone wall above which there is a timber fence giving a maximum height of approximately 1.8m. The proposed extension would be on to the eastern side elevation. It would measure 5.45m long and would project 1.9m out from the side elevation. This front wall would be set in 0.5m from the front wall of the existing building and its rear wall would be 1.7m in from the rear wall of the bungalow. It would have a gabled pitched roof with the same eaves height and angle as the main roof but a ridge height approximately 0.5m lower than the main roof. The external materials would comprise rendered walls and a grey slate roof to match the existing dwelling. At its closest, the extension would be approximately 1.3m away from the boundary to Kayley Lane. In view of the existing boundary wall/fence, however, I do not consider that it would have any seriously detrimental effects upon the appearance of the Conservation Area locality. It would also not have any detrimental effects upon the amenities of any nearby residents. Overall, therefore, I can see no objections to this application. |  |
| SUMMARY OF REASONS FOR APPROVAL: |  |
| The proposal has no significant detrimental impact on nearby residential amenity nor would it have an adverse visual impact. |  |
| RECOMMENDATION: That conditional planning permission be granted. |  |