Ribble Valley Borough Council

DATE INSPECTED: 19/01/2011

DELEGATED ITEM FILE REPORT – APPROVAL

| Ref: RH |  |
| --- | --- |
| Application No: | 3/2010/0952 |
| Development Proposed: | Application for a Lawful Development Certificate for a proposed replacement garage at 10 Higherfield, Langho. |
| # CONSULTATIONS: Parish/Town Council |  |
| N/A |  |
| # CONSULTATIONS: Highway/Water Authority/Other Bodies |  |
| N/A |  |
| ## CONSULTATIONS: Nearby Residents |  |
| N/A |  |
| RELEVANT POLICIES: |  |
| N/A |  |
| COMMENTS/ENVIRONMENTAL/AONB/HUMAN RIGHTS ISSUES/RECOMMENDATION: |  |
| This is a Certificate of Lawfulness for a proposed replacement garage to the northern corner of the applicant’s property, 10 Higherfield, at the end of the cul-de-sac and within the main settlement of Langho. Under the provisions of Schedule 2, Part 1, Class E of the Town and Country Planning (General Permitted Development) (Amendment) (No. 2) (England) Order 2008 the criterion below must be met. I will consider each part in turn. Class E Permitted Development E. The provision within the curtilage of the dwellinghouse of – - any building or enclosure, swimming or other pool required for a purpose incidental to the enjoyment of the dwellinghouse as such, or the maintenance, improvement or other alteration of such a building or enclosure; or - a container used for domestic heating purposes for the storage of oil or liquid petroleum gas. Development not permitted E.1 Development is not permitted by Class E if – - the total area of ground covered by buildings, enclosures and containers within the curtilage (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); No more than 50% of the applicants curtilage will be covered by buildings as a result of the development. - any part of the building, enclosure, pool or container would be situated on land forward of a wall forming the principal elevation of the original dwellinghouse; The proposal is to the side elevation of the property. - the building would have more than one storey; The proposal is single storey. - the height of the building, enclosure or container would exceed – - 4 metres in the case of a building with a dual-pitched roof, - 2.5 metres in the case of a building, enclosure or container within 2 metres of the boundary of the curtilage of the dwellinghouse, or - 3 metres in any other case; The height of the proposed garage is to be 2.7 metres and is over 2 metres from the rear boundary of the curtilage of the dwellinghouse. - the height of the eaves of the building would exceed 2.5 metres; The proposed garage at eaves height is no more than 2.5 metres. - the building, enclosure, pool or container would be situated within the curtilage of a listed building; The property is not listed. - it would include the construction or provision of a veranda, balcony or raised platform; No veranda, balcony or raised platform is proposed. - it relates to a dwelling or microwave antenna; or The proposal is not a microwave antenna. - the capacity of the container would exceed 3,500 litres. The proposal is not for a container. E.2 In the case of any land within the curtilage of the dwellinghouse which is within – - a World Heritage Site; - a Naitonal Park - an area of outstanding natural beauty, or - the Broads development is not permitted by Class E if the total area of ground covered by buildings, enclosures, pools and containers situated more than 20 metres from any wall of the dwellinghouse would exceed 10 square metres. The property is in none of the above locations. E.3 In the case of any land within the curtilage of the dwellinghouse which is article 1(5) land, Development is not permitted by Class E if any part of the building, enclosure, pool or container would be situated on land between a wall forming a side elevation of the dwellinghouse and the boundary of the curtilage of the dwellinghouse. The property and its curtilage is not on article 1(5) land. For the reasons stated above, the application can be classed as permitted development and does not require formal planning permission as stated in the General Permitted Development Order 2008. |  |
| SUMMARY OF REASONS FOR APPROVAL: |  |
| The proposed works constitute permitted development under the Schedule to Part 1, Class E of the Town and Country Planning (General Permitted Development) (Amendment) (No. 2) (England) Order 2008. |  |
| RECOMMENDATION: That a Certificate of Lawfulness for a proposed use is granted. |  |