Ribble Valley Borough Council

DATE INSPECTED: 25/02/2011

DELEGATED ITEM FILE REPORT - APPROVAL

| Ref: RH |  |
| --- | --- |
| Application No: | 3/2011/0050 |
| Development Proposed: | Single storey extension to front elevation at 20 Greenfield Avenue, Chatburn. |
| # CONSULTATIONS: Parish/Town Council |  |
| Chatburn Parish Council – No Objection. |  |
| # CONSULTATIONS: Highway/Water Authority/Other Bodies |  |
| N/A |  |
| ## CONSULTATIONS: Nearby Residents |  |
| No observations received. |  |
| RELEVANT POLICIES: |  |
| Policy G1 - Development Control Policy H10 – Residential Extensions Policy SPG – “Extensions and Alterations to Dwellings” |  |
| COMMENTS/ENVIRONMENTAL/AONB/HUMAN RIGHTS ISSUES/RECOMMENDATION: |  |
| The application relates to a terraced property on Greenfield Avenue off Ribble Lane within the main settlement of Chatburn. Permission is sought to replace the existing bay window to the property with a single storey lean-to extension projecting 1.4 metres from the front elevation, 3.2 metres in length and 2.9 metres in height. I consider that the scale, size and design of the proposal is appropriate and as it will compliment the existing lean-to extension to the front elevation of the property and other extensions of similar type on this row of terraces, its visual impact upon the property and the immediate locality will be minimal. Any impact of the proposal upon the amenity of neighbouring residents will be minimal. The extension would abut the bay window of the adjacent property and therefore I have assessed potential loss of light to this window using the BRE methodology detailed in the Councils SPG ‘Extensions and Alterations to Dwellings’. The lean-to roof and the projection from the front elevation of 1.4 metres ensures that no more than 50% of the bay window to the adjacent property will be impacted by loss of light. As such the proposal is considered to have minimal impact upon neighbouring residential amenity. Therefore, I do not consider this application would cause a significant detrimental impact on the visual amenity of the area or on the residential amenity of the occupiers of the surrounding properties. As such, the application is recommended accordingly. |  |
| SUMMARY OF REASONS FOR APPROVAL: |  |
| The proposal has no significant detrimental impact on nearby residential amenity nor would it have an adverse visual impact. |  |
| RECOMMENDATION: That conditional planning permission be granted. |  |