Ribble Valley Borough Council

DATE INSPECTED: 01/12/2011

DELEGATED ITEM FILE REPORT - APPROVAL

| Ref: GT |  |
| --- | --- |
| Application No: | 3/2011/0887/P |
| Development Proposed: | Proposed conversion of a barn to a single dwelling at Abbot Barn, Back House Lane, Chipping, Lancashire, PR3 2NR. |
| # CONSULTATIONS: Parish/Town Council |  |
| Chipping Parish Council – No observations or comments received within the statutory 21-day consultation period. |  |
| # CONSULTATIONS: Highway/Water Authority/Other Bodies |  |
| LCC Traffic and Development Engineer – No objection to this proposal. LCC Planning Officer (Archaeology) – Abbot Barn is recorded on the 1st edition Ordnance Survey, 1:10560 map, surveyed in 1844 (Lancashire Sheet 54) which shows a building of similar size and location as that proposed for conversion. The stables are not shown on this map, but can be seen on the 1st Edition OS 1:2500 (Lancashire Sheet 46.09) surveyed 1891. The building should therefore be considered to be of some historical interest, most likely dating from the latter half of the 18th Century and the stable from the mid-late 19th Century, having undergone a number of alterations in response to changes in agricultural practices and economics. The proposed conversion will have a significant impact on the historic character and appearance of the building, and may result in the loss of some historic fabric, therefore should the LPA be minded to grant permission, the LAS would recommend that the recording of the buildings prior to conversion be secured. |  |
| ## CONSULTATIONS: Additional Representations. |  |
| No additional representations received. |  |
| RELEVANT POLICIES: |  |
| Policy G1 - Development Control. Policy G5 - Settlement Strategy. Policy ENV1 – Area of Outstanding Natural Beauty. Policy ENV7 – Species Protection. Policy ENV14 - Ancient Monuments and Other Important Archaeological Remains. Policy H2 – Dwellings in the Open Countryside. Policy H15 - Building Conversions - Location. Policy H16 - Building Conversions - Building to be Converted. Policy H17 - Building Conversions - Design Matters. PPS3 – Housing. PPS5 – Planning for the Historic Environment. Wildlife & Countryside Act 1981 as amended. The Conservation [Natural Habitats & c.] Regulations 1994. |  |
| COMMENTS/ENVIRONMENTAL/AONB/HUMAN RIGHTS ISSUES/RECOMMENDATION: |  |
| This application details the proposed conversion of a redundant agricultural building, Abbot Barn, into a single dwelling. The building is a roadside barn that sits adjacent to the existing farmhouse on site, on Blackhouse Lane, Chipping. The building is situated some 1.5km west of the village boundary of Chipping, and some 5km north of Longridge, and is surrounded by land in agricultural use. The landscape is relatively open in character, and lies within the Forest of Bowland Area of Outstanding Natural Beauty. Abbot Barn comprises a traditional stone built former agricultural barn, albeit with a corrugated iron roof at present. A number of date stones are evident at the property – 1752 (or 1732) on the farmhouse, 1775 on the barn (with initials ‘TR’ – possibly Thomas Rogerson) and 1914 on the stonewall on the road frontage. A range of other documents/sources of information have been researched which all reveal that the farmhouse, barn and outbuildings existed at the time of survey for the 1847 OS map, with subsequent historic OS maps (1991, 1912 and 1930) showing the same arrangement. The barn has been constructed in two phases, with a single bay added to the northern end, evidenced by stone quoins part way along its length. The main cart entrance is on the east side of the building (towards the yard), with a now blocked up smaller cart entrance to the roadside of the building. The barn also has a number of other openings at ground and upper floor levels, and a flight of stone steps providing entry to a door at first floor level in the north gable. The building is not suitable for modern day agricultural practices and is now redundant. The application seeks permission for the conversion of the larger barn in question into a dwelling, with the smaller barn being used as a garage. The garden curtilage area for the property will be created to the northwest of the larger barn by removing the existing portal frame building attached to the building, and using an enclosed parcel of land behind the proposed garage building (to the east). These two portions are currently enclosed by the existing boundary treatments (hedgerows) and will be fully enclosed by inserting a simple timber post and rail fence. The residential curtilage for the site is that enclosed within the red edge shown on the amended plan received on the 8th of December 2011. The site also includes the brick built shippon within the scheme, however there are no alterations proposed to this building. The main issues with this application relate to the principle of the development, what affect the proposed change of use and the external/internal alterations may have with regards to its visual impact on the barn, any potential impact on the residential amenity of the adjacent property, any potential impact on habitats and any potential impact on highway safety. There will be no impact on the residential amenity of the occupiers of the adjacent property. With regards to the creation of one residential dwelling within the barn, guidance is provided within Policy H2 of the Local Plan, which notes “Outside the settlement boundaries, residential development will be limited to the appropriate conversion of buildings to dwellings, provided they are suitably located and their form, bulk and general design are in keeping with their surroundings. Also, that they structurally sound and capable of conversion without the need for complete or substantial reconstruction”. Additional advice is also provided by Policy H15 of the Local Plan, which notes that “The conversion of appropriate buildings within settlements or which form part of already defined groups is acceptable”, however this is providing that there would be no materially damaging effects on the landscape qualities of the area, and Policy H16 which notes that “the building must be structurally sound and capable of conversion, without the need for extensive or major alterations which would adversely affect the character or appearance of the building”, and that “the character of the building and its materials are appropriate to its surroundings and the building is worthy of retention”. Finally, Policy H17 discusses the finer points of the conversion of a building, noting that it must of a high standard and in keeping with the local tradition, and it also notes that “Too many doors and windows, the insertion of dormers, roof lights and chimneys and the alterations of roof trusses will devalue the character of traditional farm buildings and that of the surrounding environment.” Given the age of the building, as indicated by both the Agent and the LCC Planning Officer (Archaeology), the building is also considered to be a Heritage Asset (see PPS5) and have historical interest, showing the development of the surrounding farm in response to changing agricultural practices. The Agent has submitted a substantial Heritage Statement regarding the proposed conversion and its effect on the historic character of the building, in line with PPS5: Planning for the Historic Environment, and this is an important material consideration when assessing this application. With respect to PPS5, the following Policies and their considerations are relevant, - Policy HE7.1 states that ‘such identification and assessment of the particular significance of each element of the historic environment is fundamental to decision making’, - Policy HE7.4 requires consideration of the sustaining and enhancement of the significance of heritage assets and of the consideration of the positive role of heritage assets in place-shaping, - Policy HE 9.1 states that: “there should be a presumption in favour of the conservation of designated heritage assets and the more significant the designated asset, the greater the presumption in favour of its conservation should be. Once lost, heritage assets cannot be replaced and their loss is a cultural, environmental, economic and social impact. Significance can be harmed or lost through alteration or destruction of the heritage asset or development within its setting. Loss affecting any designated heritage asset should require clear and convincing justification”, - Policy HE9.4 states that “Where a proposal has a harmful impact on the significance of a designated heritage asset which is less than substantial harm, in all cases local planning authorities should; (i); weigh the public benefit of the proposal (for example, that it helps to secure the optimum viable use of the heritage asset in the interests of its long term conservation) against the harm; and recognise that the greater the harm to the significance of the heritage asset the greater the justification will be needed for any loss”, and. (ii); recognise that the greater the harm to the significance of the heritage asset the greater the justification will be needed for any loss”.. Therefore, taking into account all the above Policies and guidance, the principle of the conversion of the building in question is considered acceptable given, - the location of the building adjacent to the main dwelling on site, - that the structural survey provided with the application notes it is structurally sound and capable of conversion without the need for substantial reconstruction, - that there is an existing access into the site, and - that given the significant number of openings already within the building, the design proposed for the conversion is considered acceptable. With specific regarding to Policy HE9.4 of PPS5, there must be consideration of PPS3, particularly Para. 71, which states that in the absence of a five year supply of deliverable sites, which is the current position within the Borough, planning applications for housing should be considered favourably having regard to the wider policies within the PPS and including criteria in PPS3 para 69. Paragraph 69 states that in deciding planning applications, Local Planning Authorities should have regard to: - achieving high quality design, - ensuring developments achieve a good mix of housing, - the suitability of a site for housing, including its environmental sustainability, - using land effectively and efficiently; and - ensuring the proposed development is in line with planning for housing objectives. Bullet point 3 above relates to the need for Local Planning Authorities to have regard to the suitability of a site for housing, including its environmental sustainability. Therefore, given the proposals compliance with Local Plan Policies, and that this will, - help meet the Borough wide need for additional housing, - help to secure the optimum viable use of the heritage asset in the interests of its long term conservation, and - that given the actual limited impact on the heritage asset given the design proposed, it is considered that the site meets the stipulations of paragraph 69 of the PPS3 and PPS5 criteria. Therefore, having assessed the scheme in regards to Local Plan Policies G1, G2, H2, H15, H16 and H17, and National Guidance contained within PPS3 and PPS5, I am satisfied that the principle of the scheme is acceptable. With more specific regard to the proposed design, the existing building benefits from a significant number of existing openings in useful locations within its elevations, which help in respect to this proposal to convert it into residential accommodation. The proposed layout of the property utilise all the existing openings, with only one additional window being proposed within the stonework on the east facing (yard facing) elevation. This window has been moved from its previous, more visible, location on the south facing elevation, and as it is only visible from within the site, is therefore considered acceptable. The scheme also proposes additional conservation type roof lights on the east and west (roadside and yard facing) facing elevations of the main barn, four on each, however as the roof requires replacing in its entirety and that they will be conservation type roof lights, there are not considered to be significantly visible from any public viewpoints, and would therefore not be to the detriment of the character of the building. The scheme also includes two flues for wood burning stoves, however these too have been moved onto the yard facing (east) roof elevation so that they are not significantly visible. The scheme also includes the renovation and restoration of the existing stable/outbuilding to the eastern corner of the site, and I have no objections to this element of the scheme. As noted earlier, the existing openings within the building have been kept as they are, and the materials to be used (timber) for the both the openings themselves, and the windows and doors, are considered sympathetic. On this basis, the design of the scheme is now considered to comply with Local Plan Policies H16 and H17, and the guidance provided within PPS5, as it will have an acceptable impact on the character and setting of the existing barn, and will not have a detrimental impact on the location. In respect of the visual impact on the A.O.N.B. and the adjacent open countryside, the character and form of the building has been retained by virtue of the sympathetically proposed conversion scheme and by virtue of the use of existing access points into the site, and as such there is very little visual change to the site. The proposed garden and parking areas will be screened from public view, and will be within existing boundaries on site. Permitted Development Rights will be removed in order to retain control over further development of this site and to safeguard the character and setting of this rural location, and as such the scheme is considered visually acceptable. In respect of the potential impacts on existing habitats at the site, I have discussed the bat survey submitted with the application with the Countryside Officer, and we are satisfied that conditions can be placed upon this proposal to enable the safe control of the future development of this site as per the proposed scheme. The application is therefore recommended accordingly. |
| SUMMARY OF REASONS FOR APPROVAL: |  |
| The proposal represents an appropriate form of development and given its design, size and location would not result in visual detriment to the surrounding countryside, nor would its use have an adverse impact on highway safety. |  |
| RECOMMENDATION: That conditional planning permission be granted. |  |