| Report to be read in conjunction with the Decision Notice. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Application Ref: | 3/2016/0577 |  |  |  |
| Date Inspected: | 20/07/2016 |  |  |  |
| Officer: | AB |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  | APPROVAL |
| Development Description: |  |  | Proposed demolition of 14No. garages and erection of 4No. 3 bedroom dwellings including alterations/improvements to existing vehicular access. |  |
| Site Address/Location: |  |  | Land off Ribblesdale View Chatburn Clitheroe |  |
| CONSULTATIONS: |  |  | Parish/Town Council |  |
| Chatburn Parish Council has the following observations to make: - assurance that there will be adequate parking facilities for residents, particularly for the 4 bedroomed detached dwellings - there are concerns that vehicles will exit on to a very narrow road - the provision of a footpath is essential from which residents can safely access the Ribblesdale View path. |  |  |  |  |
| CONSULTATIONS: |  |  | Highways/Water Authority/Other Bodies |  |
| LCC Highways: |  |  |  |  |
| No objections in principle subject to conditions. Recommends a bin collection point is provided for plot 1 and that the applicant should provide a 2m wide footpath for the full frontage of the site in the interest of highway safety for pedestrians and the protection of the sight lines from the new access point. |  |  |  |  |
| United Utilities: |  |  |  |  |
| United Utilities will have no objection to the proposal subject to conditions relating to foul and surface water drainage. |  |  |  |  |
| Network Rail: |  |  |  |  |
| Network Rail advise that the applicant/developer should liaise directly with Asset Protection and provide conditions and advisory notes accordingly. |  |  |  |  |
| CONSULTATIONS: |  |  | Additional Representations. |  |
| Two letters have been received in relation to the application; one in support and one in objection. The objection relates to a strip of land that would be left between the application site and the adjacent garage site and the future management of this land. In support of the application, the proposals would improve the visual appearance of the site so long as the new buildings are in keeping with the village. |  |  |  |  |
| RELEVANT PLANNING HISTORY: 3/2000/0189/P – Outline application for residential development. Refused. 3/1985/0095/P – Proposed erection of two pairs of semi-detached dwellings with integral garages. Approved. |  |  |  |  |
| RELEVANT POLICIES: |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1 – Development Strategy Key Statement DS2 – Sustainable Development Key Statement EN5 – Heritage Assets Key Statement H1 – Housing Provision Policy DMG1 – General Considerations Policy DMG2 – Strategic Considerations Policy DMG3 – Transport and Mobility Policy DME1 – Protecting Trees and Woodlands Policy DME3 – Site and Species Protection and Conservation Policy DME4 – Protecting Heritage Assets National Planning Policy Framework National Planning Policy Guidance Planning (Listed Buildings and Conservation Areas) Act 1990 Chatburn Conservation Area Appraisal and Management Guidance |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |
| Proposed Development for which consent is sought: This application seeks consent for the demolition of 14no. garages and the erection of 4 no. 3 bedroom dwellings (one pair of semi-detached and two detached) and alteration /improvements to existing vehicular access at land off Ribblesdale View, Chatburn. The application site is triangular in shape and is located on the south side of Ribblesdale View and near the junction and south east side of Bridge Road. There is an existing access off Ribblesdale View with land rising into the site from the highway. Along the western boundary of the site are eleven lock up garages which at the northern end attach to a vacant stone built commercial building. A further three lock ups are located north east of the site entrance. The development site is located within the settlement of Chatburn and on the edge of the Chatburn Conservation Area. The surrounding area is predominantly residential although as noted there is a stone built commercial building close to the site on the corner of the Ribblesdale View and Bridge Road junction. A rail line is located directly to the east of the site. The proposal involves the demolition of the existing garages and construction of four two-storey, three bedroom dwellings consisting of a pair of semi-detached property and two detached. The proposed dwellings on plots 1-3 would face north-east with the rear gardens backing onto an adjacent garage site and plot 4 would be located on the north-eastern side of the site adjacent to the rail line and would face south-west. The properties would be served by a new access that would terminate in the southern corner of the site. The semi-detached dwellings would have a combined footprint of 12.3m x 10.2m and would have eaves and ridge heights of 5.4m and 8.8m respectively. Dwelling no.1 would be stepped back slightly and would have a canopy across the front elevation below the cills of first floor windows whilst dwelling no.2 would have a front porch with a height of 3.5m. Properties no.3 and 4 would measure 6m x 9m and would have a height of 8.8m. The proposed dwellings would be faced with coursed natural stone to the front and rendered to the sides and rear. They would have stone dressings in quoins, cills and lintels and window frames would be hardwood. Each dwelling would have a piked wall dormer on the front elevation. |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: In determining the application it is important to consider the principle of the development, its impact of the proposals on the setting of the adjacent Conservation Area, the relationship of this site with surrounding uses, matters of highway safety and the impact of the proposal on neighbouring residential amenity. The application site lies within the settlement boundary of Chatburn. In terms of assessing the principle of residential development in this location, the Development Strategy put forward in Key Statement DS1 of the Core Strategy seeks to direct the main focus of new house building to the Strategic Site, the Principal Settlements of Clitheroe, Longridge and Whalley and Tier 1 villages which are considered the more sustainable of the 32 defined settlements. Chatburn is identified as a Tier 1 Village and Policy DS1 sets a residual number of 18 houses to be provided in the village during the plan period 2008-2028. The latest position is contained in the Council’s Housing Land Availability Schedule April 2016 which indicates a residual of 18 houses. Therefore, the provision of four dwellings within the settlement of Chatburn would contribute to the supply of housing with the Borough and would accord with Core Strategy Policy DS1. Turning to matters of visual amenity and heritage impacts, Section 72(1) of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires that special attention is paid to the desirability of preserving or enhancing the character or appearance of the area but these general duties apply to land or buildings within a Conservation Area. However, it is normally accepted that designation should have an effect beyond the boundaries of the Conservation Area itself as reflected by Policy DME4 of the Core Strategy which applies not only to development within Conservation Areas but also those affecting views into or out of, or affecting the setting of Conservation Areas. There would be some limited views of the proposals afforded from Bridge Road and I consider the proposed dwellings to be of appropriate scale, design and external materials such that they would enhance the appearance of the site. As such, it would have a positive impact of the setting of the Conversion Area and is considered acceptable. The proposed development would provide an appropriate level of residential amenity for future occupants. In terms of the impact of the development on the residential amenities of neighbouring occupiers, the nearest dwelling to the south is no.18 Downham Road with its rear elevation located around 13m away. There are no first floor windows on the north facing elevation of this property and there would be an angle of 90° between first floor habitable room windows on the east side elevation. The occupants of this dwelling would not suffer any unacceptable loss of light, privacy or outlook by virtue of the proposals to justify refusal of the application. The proposals would not result in any harm to amenities of any other neighbouring occupants of dwellings on the opposite side of Ribblesdale View or at no.4 The Croft to the west. In terms of the relationship of the site to other uses, there is a vacant industrial (B2) unit that adjoins the site to the north west. Whilst the intention of its owner is to convert the building to residential use in the future, the building is not in the ownership of the applicant and therefore there would be no guarantee that the building would be converted in the near future. The building in question has been vacant for a number of years but there is the possibility that it could be used for industrial uses without restriction. Of additional concern is the location of the site adjacent to the rail line from Clitheroe to Settle and Carlisle. The line is cut into a deep ravine which would provide a barrier between the rail line and the development site. The applicant is agreeable to the incorporation of noise attenuation measures within the build to ensure that internal noise in kept to an acceptable level. Following amendments to the layout of the site it has been agreed that, should consent be granted, a pre-commencement condition will be attached to the decision notice requiring an assessment of noise and vibration to be submitted to the Council. If the assessment indicates that noise and/or vibration is likely to affect the proposed dwellings then a detailed scheme of mitigation measures shall be submitted to and approved in writing by the Local Planning Authority prior to the commencement of the development. A protected species survey has been submitted which found no evidence of bats using the garage buildings and concludes that the proposed works are unlikely to cause disturbance to bats, result in the loss of a bat roost or cause injury or death to bats. A Tree Report and Constraints Plan submitted with the application indicates that there are 8 trees on the western edge of the site between the lock-up garage to be demolished and the adjacent garage site to the west. Six of the trees are relatively small and do not contribute significantly to the amenity of the area. Tree nos. 2 and 6 are the dominant trees on site and can be clearly seen from Ribblesdale View and Bridge Road; they have some amenity value and contribute to the character of the area. Tree no.2 would be retained, however; the report states that tree no.6 has long term structural damage and requires removal before structural failure occurs. The Council’s Countryside Officer has agreed to the removal of tree no.6. The proposed site plan includes an open plan landscaped area on the northern side of plot no.3 which would serve to soften the appearance of the development site and would also function as a barrier to noise from the vacant industrial (B2) unit should it be brought back to use. Subject to further details, the proposals are considered acceptable and in keeping with surroundings. According to the County Surveyor (Highways) the proposals would have a negligible impact on highway capacity. The applicant provides a 1.8m wide footpath for the frontage of the site to the west of the new access point in the interest of highway safety for pedestrians which the County Surveyor has approved verbally. The protection of the sight lines with the footpath works are to be carried out under a section 278 agreement with Lancashire County Council. The stone wall along the frontage of the site to the east of the access is to be retained in the development. Having carefully considered the above, I am of the opinion that the scheme would be acceptable in terms of its visual appearance, impact of the Conservation Area, effect on the residential amenities of neighbouring occupants and its impact on trees, protected species and highway safety. Subject to the use of appropriate conditions, the Council is satisfied that any noise in the locality can be reduced to an acceptable level through the use of mitigation measures. Accordingly, it is recommended that the application be approved. |  |  |  |  |
| RECOMMENDATION: |  | That planning consent be approved. |  |  |