| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Application Ref: | 3/2017/0250 |  |  |  |  |
| Date Inspected: | 31/03/2017 |  |  |  |  |
| Officer: | RH |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  | APPROVAL |  |
| Development Description: |  |  | Application for a lawful development certificate for a rear dormer |  |  |
| Site Address/Location: |  |  | 100 Chatburn Road Clitheroe BB7 2AT |  |  |
| RELEVANT POLICIES: |  |  |  |  |  |
| Schedule 2 Part 1 Class B of the Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO). |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |
| Site Description and Surrounding Area: The application property is located on the south-east side of Chatburn Road opposite Clitheroe Cricket Ground and forms one of a pair of stone built semi-detached properties. There is a side street to the side of the dwellinghouse which provides access to the backstreet of properties 98-82 Chatburn Road and a pedestrian footway to Tower Hill with associated grass verge which adjoins the south-western site boundary. |  |  |  |  |  |
| Proposed Development for which consent is sought: This is a Certificate of Lawfulness for proposed development comprising a rear dormer extension at 100 Chatburn Road, Clitheroe. The application property has not had any permitted development rights removed. The proposed partially angle dormer extension project 3.4m from the rear roofslope and has a width of 1m. The face of the dormer would project to a height of 1.8m above the plane of the main roof of the application dwelling. The proposed dormer will include the insertion of a Juliet balcony to the rear elevation and a window to the south east side elevation. |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The merits of the proposals are not under consideration as the application is seeking formal confirmation that the proposal is lawful under the terms of the Town & Country Planning (General Permitted Development) (England) Order 2015. The rear extension is assessed against Class B of Schedule 2 Part 1 of the GPDO against all of the relevant criteria of Class B below: Development is not permitted by Class B if— - permission to use the dwellinghosue as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); Permission for use as a dwellinghouse was not granted by virtue of Class M, N, P or Q of Part 3. (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; The proposed rear dormer would not exceed the height of the highest part of the existing roof. (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; The proposed rear dormer would not extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway. (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than— (i) 40 cubic metres in the case of a terrace house, or (ii) 50 cubic metres in any other case; The cubic content of the resulting roof space would not exceed the original roof space by more than 50 cubic metres. (e) it would consist of or include— (i) the construction or provision of a veranda, balcony or raised platform, or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; or (f) the dwellinghouse is on article 2(3) land. The proposed rear dormer does not include the installation, alteration or replacement of a chimney, flue or soil and vent pipe nor is the dwellinghouse located within a World Heritage Site, a National Park, an area of outstanding natural beauty or the Broads. The proposal includes the insertion of a Juliet balcony to the face of the rear dormer. A balcony is understood to be a platform with a rail, balustrade or parapet projecting outside an upper storey of a building. A ‘Juliet’ balcony has no platform and therefore no external access, so does not constitute a balcony. The proposal must also meet the requirements of the Conditions B.2: B.2 Development is permitted by Class B subject to the following conditions— - the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; The proposed development would be constructed using external materials that are of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse, the proposal will include no first floor windows and the proposed development would not have more than one storey. - the enlargement must be constructed so that— (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension— (aa) the eaves of the original roof are maintained or reinstated; and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; and The proposed rear dormer accords with all aspects of the above criteria. - any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be— (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. The proposed rear dormer includes the insertion of a window on the side elevation of the proposed dormer. Drawing Number – Proposed Plans and Sections – Drawing Number: CJH2016-03 REV C states that the aforementioned window will be obscured glazed. A condition will be attached to this permission to ensure the use of obscure glass in order to reduce the harm the window would have to an acceptable level. Taking into account all of the above, I consider that the proposed development accords with the requirements of the GPDO and constitute permitted development. Accordingly, I recommend that the Lawful Development Certificate for a Proposed use be issued. |  |  |  |  |  |
| RECOMMENDATION: |  | Approve Certificate of Lawfulness |  |  |  |