| Report to be read in conjunction with the Decision Notice. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Application Ref: | 3/2018/0826 |  |  |  |
| Date Inspected: | 27/9/18 and 2/11/18 |  |  |  |
| Officer: | AD |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  | REFUSAL |
| Development Description: |  |  | Listed building consent for various internal and external alterations to dwelling including demolition of existing garden room extension and erection of a new single storey extension. |  |
| Site Address/Location: |  |  | Wiswell Eaves House Pendleton road Wiswell BB7 9BZ |  |
| CONSULTATIONS: |  |  | Parish/Town Council |  |
| No objections. |  |  |  |  |
| CONSULTATIONS: |  |  | Highways/Water Authority/Other Bodies |  |
| LAAS: |  |  |  |  |
| Date stone of 1766 - probably built significantly earlier and altered at the date above. Generally agree with the assessment of the impact of the scheme on the extant building in Heritage Statement. The HS suggests a record of the building 'as existing' prior to commencement - agree. There is no assessment of the impact of the proposals on buried archaeology, but consider that these are likely to be minor and not to merit any additional archaeological mitigation. Recommend that a photographic survey as set out in 'Understanding Historic Buildings' (Historic England 2016) is required by planning condition. Historic amenity societies: Consulted, no representations received. |  |  |  |  |
| CONSULTATIONS: |  |  | Additional Representations. |  |
| None received. |  |  |  |  |
| RELEVANT POLICIES: |  |  |  |  |
| Planning (Listed Buildings and Conservation Areas) Act 1990. ‘Preservation’ in the duties at sections 16 and 66 of the Act means “doing no harm to” (South Lakeland DC v. Secretary of State for the Environment [1992]). National Planning Policy Framework (NPPF) National Planning Policy Guidance (NPPG) Ribble Valley Core Strategy: Key Statement EN5 – Heritage Assets Key Statement EN2 – Landscape Policy DME4 – Protecting Heritage Assets Policy DMG1 – General Considerations Policy DMG2 – Strategic Considerations Policy DME2 - Landscape and Townscape Protection |  |  |  |  |
| Relevant Planning History: No pre-application advice has been sought in respect to the proposals. 3/1996/0076 – Variation to internal layout, conservatory, extension amendment to vehicular access to improve facility. LBC granted 23 May 1996. 3/1992/0528 – Conversion of barn and stable to dwelling (resubmission). PP granted 13 October 1992. 3/1991/0768 - Conversion of barn to dwelling. PP refused 3 March 1992. 3/1976/1371 – Proposed rear porch. LBC granted 24 February 1977. 3/1976/0446 – Provision of bathroom, alteration of windows on front elevation and additional windows on side elevations. LBC granted 27 May 1976. |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |
| Site Description and Surrounding Area: The list description identifies a Grade II listed (13 February 1967) “House, c.1700, altered 1766 … Double- depth plan. 2 storeys with attic, 2 bays”. Wiswell Eaves House is prominently sited on the lower west facing slopes of the Pendle Hill, Forest of Bowland AONB, outlier. Public Footpath FP13 runs from Pendleton Road to Parker Place Farm (Grade II listed) and then south immediately to the east of both listed buildings. The historic farmstead is linear and incorporates a barn (now converted to residential) immediately to the south. The submitted Heritage Statement identifies: “a relatively symmetrical composition to its front west facing elevation … typical and in keeping with the architectural trends and fashions of the mid-18th Century … building was altered in 1766 indicating at its earlier origins … Wiswell Eaves House has a double fronted, double pile plan layout consisting of two rooms to the front west side of the property and two rooms to the east side” (2.2). The Heritage Statement map regression shows very little change to the site from 1848 to 1912. In this period a field boundary is shown to run east-west incorporating the house north gable wall and enclosing a garden/orchard to the west of the house. “Assessment of significance … Evidential Value … The plan form of Wiswell eaves House also adds evidential value; its double pile plan form layout is typical of this age and type of building and is commonplace throughout many Georgian farmhouses. However past internal alterations have slightly changed the plan form of the building and have subdivided some areas to provide new rooms and uses; however the plan form is still legible and interpretable. The evidential value of Wiswell Eaves House is also derived from physical historic fabric, of which much remains … the principal front elevation of the building, including its date stone” (7.1). “Assessment of significance … Historical Value … In terms of illustrative historical value, Wiswell Eaves House is illustrative of a typical upland Georgian farmhouse and is a relatively good example” (7.2). “Assessment of significance … Aesthetic Value … The aesthetic Value of Wiswell Eaves House is derived from a number of aspects. It is a building of vernacular construction and would have been built using local materials and craftsmen. Although a vernacular building, Wiswell Eaves House incorporates a certain amount of ‘’politeness’ such as the attempt at a symmetrical double fronted façade and proportions which is common amongst many Georgian farmhouses. This suggests that some conscious design was incorporated in to the alteration of the building in 1766 … Apart, from the building itself, the local landscape adds to the aesthetic appeal of the building. Its location provides attractive and vast views westwards through the Ribble Valley and across through the Forest of Bowland. Its rural setting and location at the base of Wiswell Moor further enhances its aesthetic value” (7.4). Wiswell Eaves House is prominent in views (little obstruction) from Parker Place Farm and is within its setting. |  |  |  |  |
| Proposed Development for which consent is sought: LBC is sought for a replacement extension at the north elevation, further removal of interior historic walling (see 3/1996/0826) at Ground Floor to widen doorways and enlarge break-throughs, install 6 rooflights in the rear (east) roofslope and enclose the stairwell below Ground Floor to form WC. |  |  |  |  |
| Impact upon the special architectural and historic interest of the listed building, the setting of listed buildings and the cultural heritage of the Forest of Bowland AONB: Whilst the existing extension is unsympathetic, the proposed extension is conspicuous, incongruous and harmful to the special architectural and historic interest of the listed building (particularly the distinct and intrinsically important square plan and the symmetric neo-classical west front), the setting of Parkers Place and the cultural heritage of the Forest of Bowland AONB because of its scale (almost as wide as the historic west front), complex form (including gables, rooflights and a wrap-around wing) and the incorporation of large expanses of glazing. The proposed rooflights are a conspicuous (including formalised arrangement) and incongruous intrusion into the unblemished and prominent (including FP13) rear roofslope. The agent suggested that rooflights were required in part to provide escape windows but no further information has been submitted in consideration to this matter. Consideration to 3/1996/0826 indicates substantial alteration to interior plan form including significant loss of the Ground Floor walls identifying the distinct double-pile plan. The proposed door widening and removal of the stub providing a memory of removed walling is of further harm to the historic fabric and plan form of the ostensibly Georgian building. Heritage is an important contributor to the character of the AONB (see the Forest of Bowland AONB Management Plan, April 2014 - March 2019 and NPPF paragraph 172) and the submitted Heritage Statement identifies in respect to Wiswell Eaves House that “Its location provides attractive and vast views westwards through the Ribble Valley and across through the Forest of Bowland. Its rural setting and location at the base of Wiswell Moor further enhances its aesthetic value” (7.4). The incongruity of proposed development is also harmful to the character of the AONB. NPPG states that “In general terms, substantial harm is a high test, so it may not arise in many cases” – the harm to the special architectural and historic interest of Wiswell Eaves and the setting of Parkers Place is considered to be ‘less than substantial’. No evidence has been submitted to suggest that the continued residential use of Wiswell Eaves House would be in jeopardy if the proposed works were not implemented. The public benefit of construction employment does not outweigh the harm to the designated heritage assets (NPPF paragraph 196). |  |  |  |  |
| Landscape/Ecology: The submitted bat survey concludes that no mitigation or timing constraints is required. |  |  |  |  |
| Residential amenity: The proposal does not result in any significant overshadowing, overbearing or overlooking of neighbours. |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: Therefore, in giving considerable importance and weight to the duties at section 16 and 66 of the Planning (Listed Buildings and Conservation Areas) Act 1990 and in consideration to NPPF (2018) and Key Statement EN5 and EN2 and Policies DME4, DMG1 and DMG2 of the Ribble Valley Core Strategy, it is recommended that listed building consent is refused. |  |  |  |  |
| RECOMMENDATION: |  | That listed building consent be refused for the following reason: The proposed works are harmful to the special architectural and historic interest of the listed building because: the proposed extension is conspicuous and incongruous resulting from its scale, complex form (including gables, rooflights and a wrap-around wing) and the incorporation of large expanses of glazing; the proposed rooflights are conspicuous and incongruous resulting from their number, formal arrangement and intrusion to an otherwise unblemished roof slope and the loss of important historic fabric and planform (door widening and removal of the wall stub between existing Kitchen Area and Family Room). |  |  |