| Report to be read in conjunction with the Decision Notice. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Application Ref: | 3/2019/0091 |  |  |  |
| Date Inspected: | 15/05/18 and 16/05/19 |  |  |  |
| Officer: | AD |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  | APPROVAL |
| Development Description: |  |  | Change of use of existing workshop into holiday let and construction of five additional log cabins for holiday let accommodation. |  |
| Site Address/Location: |  |  | Greendale View Downham Road Chatburn BB7 4DL |  |
| CONSULTATIONS: |  |  | Parish/Town Council |  |
| Concern about access onto narrow and busy road. Poor visibility and no footpath. |  |  |  |  |
| CONSULTATIONS: |  |  | Highways/Water Authority/Other Bodies |  |
| LCC Highways: |  |  |  |  |
| (24/04/19) Further information required. Site plan - proposed bridleway running alongside Downham Road. Remote from any other similar facility; reason for inclusion? PROW colleagues unaware. Issues with bridleway: entry and exist points onto Downham Road; width available to the front of the property and site plan suggests that between existing hedge and a new hedge. If the existing hedge is retained in the vicinity of the access to the proposed pods then this would create visibility issues. How does bridleway fit into wider picture? (16/05/19 and following the forward of information from the agent) Proposal closely linked to the current application for a permissive bridleway between Downham and Chatburn. Unhappy with the sight lines to the left of the entrance to the Pod parking area and the hedges will need to be removed to ensure a proper visibility splay. Need a revised plan showing the achievable sight lines. Bridleway – the proposed route across the front of Greendale is unacceptable. The proprietor is adamant that this area will be utilised for disabled parking - as such it cannot accommodate parking and equine through traffic. Awaiting feedback on suggestion of taking pedestrians along original route with horse riders diverted around the back (have also commented on the Downham end of the scheme). (5/06/19 and following revised plans from the agent 29/05/19) Considering as a separate application to the bridleway application (although there is reference to it on the plans). Acceptable in principle but need to see a visibility splay dimension quoted on the plan for clarification purposes. The plan also refers to the "lowering of the existing wall and hedge to provide the visibility splay" - preference for the hedge within the visibility splay to be completely removed ensuring that new growth doesn't imped visibility over time. This would then be backed up by condition (suggested). (3/07/19 and following revised plans from the agent 6/06/19) Treating the Greendale View (0091) as a standalone application. Principle is acceptable but still unhappy with the visibility splay proposed. The applicant has measured 65m to the centreline of the road - preference for measurement to the nearside edge of the carriageway which would further extend the visibility distance,. This will require the removal of a greater section of hedging but should allow the 2 mature trees to remain. Need to check to see if the degree to which the trees are impinging on the visibility splay is acceptable. (22/07/19 and following revised plans from the agent 19/07/19 in respect to use of existing café car park) The revised parking proposals are acceptable. There would still be the possibility of guests using the area adjacent to the pods for parking (as previously proposed) without the sightline improvements. If this practice became the norm then there would be a safety hazard (as previously identified) at the site entrance. Access to the rear courtyard area would need to be physically restricted and signage erected adjacent to No. 2 to direct guests to the designated parking area. (9/08/19 and following revised plans from the agent 8/08/19) The revised proposals with the signage and gates to mitigate against the likelihood of guests using the area to the front of the pods would be acceptable. (13/08/19 and following case officer reference to visibility splay and bridleway) Since the parking for the proposed pods has been moved to the existing car park, there will not be an increase in vehicle movements to and from the existing access over and above current levels so for the purpose of the development the improvements would be unnecessary. However there is a visibility issue for the existing users and from a highway safety perspective, the retention of the proposed visibility splay would be beneficial. Re: bridleway, its inclusion on the plan is more of an information item than an element of the application. No bearing on LCC consideration. Forest of Bowland AONB: No significant landscape or visual effects impacting on the AONB. The proposed cabins/pods are sited within a field behind the buildings at Greendale, and it appears they will be largely screened from views from the Chatburn - Downham Road due to the topography and intervening buildings/vegetation. Views from the north of Greendale (principally from the public right of way just outside the AONB) do not appear to be significant either. The public right of way is 300 – 350m away and with a combination of distance, intervening vegetation and size/low roof height of the cabins, the landscape and visual effects are not likely to be significant. United Utilities: Note suggested (drainage; water supply; UU property/assets/infrastructure) to facilitate sustainable development. RVBC Countryside: Recommend that work carried out during the hibernation period (Sept – March) because of building type and location (excellent connectivity to high value foraging potential). An activity survey to further assess the situation will be required (condition), if the work is carried out during the activity period (April- September). Site inspection 14/8/19 (impact of proposed visibility splay) – acceptable impact – trees which may be lost are not the main landscape focus. Proposed planting species are acceptable. |  |  |  |  |
| CONSULTATIONS: |  |  | Additional Representations. |  |
| None received. |  |  |  |  |
| RELEVANT POLICIES : |  |  |  |  |
| Key Statement DS1 – Development Strategy Key Statement DS2 – Sustainable Development Key Statement EN2 – Landscape Key Statement EC1 – Business and Employment Development Key Statement EN5 – Heritage Assets Policy DMG1 – General Considerations Policy DMG2 – Strategic Considerations Policy DMG3 – Transport and Mobility Policy DME4 – Heritage Assets Policy DME2 – Landscape and Townscape Protection Policy DMB1 - Supporting Business Growth and the Local Economy Policy DMB3 – Recreation and Tourism Development Planning (Listed Buildings and Conservation Areas) Act 1990 National Planning Policy Framework (NPPF) National Planning Practice Guidance (NPPG) |  |  |  |  |
| Relevant Planning History: 3/2019/0331 – Creation of a new off-road, multi-use link between the villages of Downham and Chatburn. The one kilometre connection would accommodate walkers, cyclists, horse-riders and those with mobility issues including chair users. Resubmission of planning application 3/2018/0754. Under consideration. Pre-application advise provided 15 June 2018: “proposed ‘safari tents’/conversion of outbuildings and stables … The site is within the Pendle Hill outlier of the Forest of Bowland Area of Outstanding Natural Beauty and is inter-visible with the main body of the AONB. It is also within the open countryside … opinions of the Lancashire County Council Forest of Bowland AONB Principal Officer. Following your site meeting, he comments that the proposals are unlikely to have a significant landscape and/or visual impact on the character of the AONB. He has asked that I forward the attached extract from the Forest of Bowland AONB Landscape Character Assessment, which offers advice and guidance on managing landscape change within particular landscape character types found within the AONB. The proposed development is situated in landscape character type 'Undulating Lowland Farmland, E8 Rimington' (sic). I note from this extract that: “This is an intimate and scenic landscape, where there is a relatively strong sense of tranquillity in many places” (1.1.3); “the erosion and loss of vernacular building styles through introduction of cheaper alternatives will reduce the distinctive characteristics of this area” (1.1.12: Development: Current and future landscape changes and opportunities); “The overall strategy for the Undulating Lowland Farmland Landscape Character Type is to manage the impact of changes in land and building use, conserve or restore neglected landscape features and encourage the retention and restoration of historic and vernacular building materials and details and the careful design of new buildings. There is also a need to conserve the network of dry stone walls, hedgerows and hedgerow trees which contribute to a diverse landscape pattern. The network of lanes, together with the rich roadside verges should be maintained and enhanced. Open views towards the Unenclosed and Enclosed Moorland Hills, and the Moorland Plateaux Landscape Character Types should be conserved” (1.1.14: Guidelines for managing landscape change); “Encourage conservation of significant historic features and buildings … Encourage sympathetic new uses for disused farm buildings to ensure that they remain a viable and contributory feature within this landscape … Encourage the use of local building materials, in particular gritstone and limestone” (1.1.15 Specific guidelines include: Cultural and historic character: Guidelines for managing landscape change). … concur that the principle of development is likely to be acceptable. In my understanding, appropriate site landscaping will be very important to a successful development and it is suggested that you discuss this with the Borough Council’s Countryside Officers … little detail has been submitted with the pre-application proposal (e.g. no elevation plans of proposed new build or conversions) and the comment of Lancashire County Council Highways has not been sought (please note that LCC Highways has its own pre-application advise service). The submission is also unclear in respect to site extent, neighbouring uses and residential amenity (dwellings adjoining Greendale View Kitchen?) and proposed use of new build stables. Mindful of the recent development of a car park to the west of and removed from the existing building footprint, the sympathetic conversion of existing buildings for use in conjunction with the Safari Tents is welcomed. I would be concerned at any significant extension of the existing building footprint”. 3/2018/0754 - Creation of a new off-road, multi-use gravel track 2.5m wide between the villages of Downham and Chatburn. The one kilometre connection would accommodate walkers, cyclists, horse-riders and those with mobility issues including chair users. Withdrawn. 3/2017/0854 – Change of use of first floor rooms over the existing cafe to four en-suite letting bedrooms for bed and breakfast use. Certificate of Lawfulness approved 18/12/2017. 3/2016/0442 - Change of use of agricultural land to car park for Greendale View Cafe. PP granted 16/09/2016. There is no planning record of a change of use of the site to a café suggesting the long-standing operation of the business. |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |
| Site Description and Surrounding Area: Greendale View is an isolated building complex lying at the top of a north-west facing escarpment in the open countryside between Chatburn and Downham (outside of both village boundaries shown in the 1998 Local Plan). The escarpment forms the north-western boundary of the Forest of Bowland AONB Pendle Hill outlier; local topography results in strong inter-visibility with the main body of the AONB to the north. Footpath FP10 follows the valley to the north. The submitted Heritage Appraisal identifies: Large two storey house with basement, the latter approached from the rear where ground level is lower, and adjoining domestic outbuildings which form the other two sides of a back yard. The house currently combines residential use with a restaurant and guest accommodation. Established as a pair of semi-detached houses in 1875 (No buildings are shown on the 1847 OS map). Between 1875 and 1884, extended at the west end which continued to the north as outbuildings, forming two sides of a courtyard - carried out as a temperance hotel. Outbuildings - trap house with stable and hay loft; front appears unaltered. Lean-to at rear of both trap houses; three rooms; easternmost room was probably a privy for use by staff; L-shaped addition seems to be of a single main phase, with minor alterations, and to date from the early 20th century. Visually interesting group of late 19th and early 20th century historic structures. It is visually prominent from some viewpoints, particularly Chatburn Road, but the rear is generally more concealed. The small group of outbuildings to the rear date from a number of phases and include a wash house, a main trap house with stable and hay loft, together with a second, later trap house or motor garage. There are also two privy buildings within the group. Rear outbuildings illustrate the provision of service and transport accommodation outside the main structures. Group contributes to the visual appearance and historic interest of the AONB. The site faces the continuous stone wall boundary to the gardens of Downham Hall (Grade II* listed; ‘Country house, 1835, by George Webster’- list description) which also forms the northern boundary to Downham Conservation Area (the Conservation Area boundary extends to the west just beyond the application site before it returns to the south). |  |  |  |  |
| Proposed Development for which consent is sought: Planning permission is sought for: - the conversion of the later trap house/motor garage and removal of the easternmost room/privy from its rear lean-to (proposals to alter the opening treatment of the adjoining early trap house have been deleted). - Five log cabins in the rear paddock to the north of the courtyard. - Timber clad donkey stable (shown on site plan but no elevations originally submitted). Car parking and access arrangements have been the subject of significant modification (see LCC Highways above). The latest plans (8/8/19) refer to parking for log cabin users in the existing café car park and a visibility splay at the existing access adjacent 1 and 2 Greendale View. On 17 May 2019 the case officer requested further information: “Details of the curtilage walling (including gate pillars) to be lost in provision of the bridleway/access sight lines; Proposed log cabin north elevation plans - indicating any glazing (reflectivity of materials) and internal and external lighting; Details of proposed stables – elevation plans and how to be used and Details of current residential amenity areas associated with 1 and 2 Greendale View. The existing opening treatment for the south elevation of the outbuilding is traditional, uniform and pleasing. I am concerned from the submitted plan that character is to be lost from the removal of painted timberwork (including historic door to middle unit) and replacement with large expanses of glazing and incongruous fenestration styles (horizontally emphasised lights to right hand unit)”. On 14 August 2019 the agent confirmed “the annotation and reference to the 'Proposed 2.5m bridleway' was placed upon the drawings for information purposes only … matter is being dealt with via another application”. |  |  |  |  |
| Highway safety: The revised plans of 13/08/19 are acceptable to LCC Highways (without conditions – see response of 13/08/19). Impact upon AONB (including cultural heritage): Mindful of the comments of the Forest of Bowland AONB officer, the proposals have an acceptable impact on landscape character. Scheme revisions result in the sensitive conversion of the non-designated heritage assets. |  |  |  |  |
| Impact upon the setting of Downham Conservation Area and listed buildings: There is no harm to either the setting of Downham Conservation Area or listed buildings (principally Downham Hall). |  |  |  |  |
| Impact Upon Residential Amenity: The proposals would have a significant adverse impact on the residents of 1 and 2 Greendale View. However, mindful of site ownership and the Greendale business related tenancy of residents this would be tolerable subject to a personal permission. |  |  |  |  |
| Ecology: A bat survey condition is necessary related to the possible timing of works implementation. |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: Negotiation has resulted in a scheme which supports the local economy whilst safeguarding non-designated heritage assets, AONB character and highway safety. |  |  |  |  |
| RECOMMENDATION: |  | That planning consent be granted. |  |  |