| Report to be read in conjunction with the Decision Notice. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Application Ref: | 3/2019/0936 |  |  |  |
| Date Inspected: | 12/11/19 |  |  |  |
| Officer: | AD |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  | APPROVAL |
| Development Description: |  |  | Internal alterations including construction of partition wall and re-positioning of kitchen; installation of shower room and utility room; repairs to ceilings and windows; replacement doors; replacement drain pipes and gutters; new radiators, new lighting and re-wiring; replacement of tiled floor; removal of fireplace; replacement bathroom suite; re-pointing; installation of external security lights. |  |
| Site Address/Location: |  |  | The Old Toll House Longsight Road Copster Green BB1 9EX |  |
| CONSULTATIONS: |  |  | Parish/Town Council |  |
| No comments received. |  |  |  |  |
| CONSULTATIONS: |  |  | Highways/Water Authority/Other Bodies |  |
| LCC Highways: |  |  |  |  |
| No objection no conditions Historic amenity societies: Consulted, no representations received. LCC Archaeology: No objections. The scale of the work is not sufficient to merit an archaeological response. RVBC Countryside: (5 /11/19) Proceed with caution. However, in event that any evidence of bats is found, discuss with Countryside Officer. |  |  |  |  |
| CONSULTATIONS: |  |  | Additional Representations. |  |
| None received. |  |  |  |  |
| RELEVANT POLICIES : |  |  |  |  |
| Planning (Listed Buildings and Conservation Areas) Act 1990. ‘Preservation’ in the duty at section 66 of the Act means “doing no harm to” (South Lakeland DC v. Secretary of State for the Environment [1992]). National Planning Policy Framework (NPPF) National Planning Policy Guidance (NPPG) Ribble Valley Core Strategy: Key Statement EN5 – Heritage Assets Policy DMG1 – General Considerations Policy DME4 – Protecting Heritage Assets |  |  |  |  |
| Relevant Planning History: Pre-application advice was sought in September 2019. Repair/replacement windows, guttering and painting/central heating/re-wiring likely to be acceptable. However, proposed removal of chimney breast of concern. 3/1985/0432 – Replacement of existing window frames and door, cleaning exterior stone, installation of central heating, two new fireplaces and new floor in sitting room. LBC granted 5/9/85. |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |
| Site Description and Surrounding Area: ‘Oaks Bar’ is a Grade II listed (24/11/66) toll house of the early to mid C19 prominently sited (innately) adjoining the A59. The list description does not refer to the interior. LCC Archaeology suggest that the Old Toll House was probably built 1808 for the Trustees of the Clitheroe and Mellor Brook Turnpike Trust. The 1845 and 1893 OS maps appear to show only the north range (with kitchen?). The 1912 OS map shows the existing extent of the toll house (the kitchen is shown as an extension). The initially submitted Heritage Impact Statement provided generic information on toll houses. A revised statement (13/11/19) identifies: “It is understood the property was constructed in at least two if not three stages. The original build is known to have been where the current sitting room and lounge areas are formed and we understand built early to mid-1800’s. The current owner has looked at deeds dated 1902 which identifies the land to the side and right of the property which was not part of the original demise and therefore it is considered the property was extended post 1902 where the current three bedrooms are located. There is evidence in the property to support the property was built at different times with the different chimney stacks and changes in finishes and ceiling heights between the two areas. It is also felt the “lean to” at the rear of the property (now a kitchen) was also extended post 1902 … Toll House ceased to operate … around 1890 and coincides with the property being extended after 1902 to form larger and possibly residential accommodation from that time”. |  |  |  |  |
| Proposed Development for which consent is sought: LBC is sought for interior planform and fabric alterations (door blocking; new corridor to south range; re-location of kitchen and bathrooms) and external cleaning, re-painting and provision of lights and security cameras. On 16 December 2019 the case officer advised/requested: “build date of the existing kitchen – the 1845 OS map appears to show only the north range of the toll house but this appears to include the kitchen. Does the 1880s OS map or your client’s 1902 deeds provide clarity? Whilst the kitchen lean-to ceiling superficially appears modern, I am concerned it could be older. If this is traditional fabric, what impact will proposed insulation have on the building environment (see IHBC ‘Retrofitting of traditional buildings)? … advise of any alteration (e.g. holes in walls) to the historic fabric in provision of a kitchen in the existing sitting room. … proposed stone cleaning methods … Ribble Valley Conservation Area Management Guidance that “Stone cleaning: All stone cleaning techniques have an inherent risk of damaging the stone and must be selected and executed with care. Cleaning may sometimes be desirable to prevent the harm caused by corrosive dirt or to reveal where problems are hidden by encrustations. However, cleaning is less justifiable for aesthetic reasons alone, and consideration must be given to its impact on the historic character of the building (e.g. loss of 'the patina of age')”. … proposed wall re-painting … Ribble Valley Conservation Area Management Guidance that “External painting: Previously unpainted surfaces should not normally be painted over. There is rarely a good reason to paint historic stonework or brickwork … Historically, some buildings in Ribble Valley may have been limewashed, sometimes called ‘whitewashed’. Limewash can be used to protect exposed rubblestone walls or to conceal rough stonework. Limewash is a simple type of matt paint made from lime and water, with or without additives. Impurities in early lime commonly produced an off-white colour, not today’s startling white which is a 20th century development that is inappropriate on a historic building”. … probably built 1808 for the Trustees of the Clitheroe and Mellor Brook Turnpike Trust. ‘Making changes to heritage assets’ (Historic England, 2016, paragraph 52) identifies “new services, both internal and external, can have a considerable, and often cumulative, impact on the significance of a building and can affect significance if added thoughtlessly”. Please submit details of the proposed security cameras. Would a modern light fitting be more appropriate than a Victorian design?”. The agent (18 December 2019) makes no comment on the date of the kitchen or impact of insulation and reiterates painting proposals. However, there is confirmation of: No significant holes in walls proposed; just limited fixings back to the wall for the units. A light clean of the stone is required to prevent long term damage to the building from the obvious modern-day traffic pollutants. Could look to position a couple of cameras away from the building and possibly one to the underside of the eaves. |  |  |  |  |
| Impact upon the special architectural and historic interest and setting of the listed building: |  |  |  |  |
| The proposed works are acceptable in principle and details can be considered by condition. The proposed corridor insertion will result in some ‘less than substantial’ harm to historic planform (room proportions; fireplace juxtaposition). However, the public benefit of ensuring the practical usage of the listed building as a three bedroomed dwelling by installation of potentially reversible partitions (no loss of important fabric) outweighs this harm; the corridor is to the post-1902 build which is not at the core of the special interest of the toll house. |  |  |  |  |
| Landscape/Ecology: The suggested note by RVBC Countryside is appropriate. |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: Therefore, in giving considerable importance and weight to the duty at section 16 of the Planning (Listed Buildings and Conservation Areas) Act 1990, in giving ‘great weight’ to the conservation of the designated heritage asset (NPPF paragraph 193) and in consideration to NPPF paragraph 192 (development sustaining and enhancing the significance of heritage assets and positively contributing to local character and distinctiveness) and Ribble Valley Core Strategy Key Statement EN5 and Policies DME4 and DMG1, it is recommended that listed building consent be granted subject to condition. |  |  |  |  |
| RECOMMENDATION: |  | That listed building consent be granted. |  |  |