| Report to be read in conjunction with the Decision Notice. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Application Ref: | 3/2019/1079 |  |  |  |
| Date Inspected: | 4/10/18, 21/3/19, 25/3/19, 11/7/19, 10/12/19 and 11/2/20 |  |  |  |
| Officer: | AD |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  | REFUSAL |
| Development Description: |  |  | Alterations to the door and window openings to the rear elevation only. |  |
| Site Address/Location: |  |  | 6 Downham Road Chatburn BB7 4AU |  |
| CONSULTATIONS: |  |  | Parish/Town Council |  |
| No comments received. |  |  |  |  |
| CONSULTATIONS: |  |  | Highways/Water Authority/Other Bodies |  |
| LCC Archaeology: |  |  |  |  |
| The development site is considered to have negligible archaeological potential. Historic amenity societies: Consulted, no response received. |  |  |  |  |
| CONSULTATIONS: |  |  | Additional Representations. |  |
| Three letters of concern received which request that the impact of the unauthorised rear outbuilding also be considered. |  |  |  |  |
| RELEVANT POLICIES : |  |  |  |  |
| Planning (Listed Buildings and Conservation Areas) Act 1990. ‘Preservation’ in the duties at sections 16 and 66 of the Act means “doing no harm to” (South Lakeland DC v. Secretary of State for the Environment [1992]). Downham Conservation Area Appraisal Ribble Valley Core Strategy: Key Statement EN5 – Heritage Assets Policy DME4 – Protecting Heritage Assets Policy DMG1 – General Considerations National Planning Policy Framework (NPPF) National Planning Policy Guidance (NPPG) |  |  |  |  |
| Relevant Planning History: The application has been submitted following consideration to enforcement complaints. 3/2019/0044 & 0047 Demolition of existing outbuilding and roof garden and construction of new detached annex and roof garden. PP and LBC refused 26 March 2019. Pre-application advise provided 5 October 2018: “Application 3/1976/0582 relates to the outbuilding. File correspondence (12 August 1976) refers to the unfortunate existing impact of modern window openings and modern rendering at the rear elevation of the historic building. The approved plans show a mono-pitched roof to the outbuilding in matching blue slate. I can find no reference to a first floor door at the eaves onto a bridge to a flat roof terrace above the outbuilding. In my opinion, this work has had a detrimental impact upon the fabric, planform and appearance of the listed building. The works now proposed include loss of important historic fabric from the drop-down of openings (including an original C17 window). Important and distinguishing elements of significance (C17 doorway and window) are shown to be encapsulated within the link extension. The proposed link extension would be visible from Sawley Road and I am concerned that the proposed development would exacerbate the domination of the listed building and its immediate setting by incongruous modern additions (bridge and outbuilding roof terrace). … also noted the potentially (no draft elevation drawings have been submitted) harmful impact of proposed side windows on residential amenity (i.e. possible significant overlooking of neighbours)”. 3/1976/0582 - Utility room, store and workshop in rear yard. LBC granted 13 September 1976. 6/10/1257 – Alterations to existing dwelling house. PP granted 27 April 1964. ‘Existing’ plans show that the major alterations to front elevation windows (mullion and transom windows installed) referred to in the list description were implemented at this time. |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |
| Site Description and Surrounding Area: ‘6 Downham Road’ is a Grade II listed (13 February 1967) house of the C17, prominently sited within Chatburn Conservation Area. The list description states: “House, C17th. Limestone rubble with sandstone dressings and slate roof. 2 storeys, 2 bays. The windows are mullioned and transomed, of 6 lights and chamfered on the 1st floor, of 8 lights and rebated and chamfered on the ground floor. The left-hand ground-floor window appears to be original, but the other windows were once simply mullioned and have been enlarged. The chamfered doorway, between the bays, has a Tudor-arched head. The right-hand gable has a massive projecting chimney stack with offsets, a later cap, and a chamfered fire window in its front wall. Inside, the right-hand room has a wide chamfered fireplace with segmental head. The left-hand room has a chamfered fireplace with flat head”. The Chatburn Conservation Area Appraisal identifies: Downham Road buildings adjoining to the east and facing to be Buildings of Townscape Merit deserving Article 4 Direction restriction; the former tollhouse at the end of this row to be a Focal Building (Townscape Appraisal Map). “Chatburn’s earliest buildings, which date from 17th century, have mullioned or mullioned and transomed windows of the same limestone as the walls (Nos 6, 55 and 57 Downham Road, for example)” (Architectural Qualities). “considered to be good, relatively unaltered examples, of their type. Among buildings of note are … Downham Road … the Reading Room/Parish Hall at 21 Downham Road, Nos 1 and Nos 7/9 … Several houses have distinctive architectural features that make them stand out from other dwellings … The old toll house (No. 2 Downham Road): has a semi octagonal plan, a flat-roofed sandstone porch supported by a column of turned sandstone and a moulded cushion capital, and a corner squinch of sandstone supporting a projecting upper story. It also has lead lined stone gutters, a roof of Welsh slate, and a gable end with coping and shaped corbels. Overall this represents a late 19th-century reworking of what was probably a utilitarian toll house into a picturesque cottage, which is now an eye-catching focal building” (Buildings of Townscape Merit). “roof conversions that result in very large dormer windows on the rear roofs of some cottages” (SWOT analysis: Weaknesses). “Continuing loss of original architectural details and use of inappropriate modern materials or details” (SWOT analysis: Threats). “The incremental loss of original building materials and detailing has been noted on many of the historic buildings within the Chatburn Conservation Area … The buildings which are proposed for inclusion within the Article 4 Direction are … Downham Road north side: Nos 2 (old toll house), 8 to 12 … The kinds of work that it is proposed to control include … alterations to the roof, including changing the roof materials and installing rooflights … the erection of sheds and other outbuildings … the erection or alteration of gates, fences or walls … It is proposed that the restrictions will only relate to development visible from a public highway” (Article 4 Direction). |  |  |  |  |
| Proposed Development for which consent is sought: It is proposed to remove the unauthorised rear elevation first floor link bridge (to outbuilding roof) and repair the door opening created for this access. Alterations to other modern rear elevation openings are proposed. Extended negotiation has been undertaken in respect to proposed alterations to the ground floor kitchen and first floor stair windows. The only surviving evidence for the form of these openings is provided by the block plans for 6/10/1257 (27 April 1964) which shows narrower (lighting only one flight of the dog-leg staircase in respect to the stair window) openings in both cases. The case officer confirmed 27/1/20 “It is recognised that your clients are addressing unauthorised works to the rear of the listed building which they have inherited from previous property owners. In large respect the proposed amelioration works are acceptable and welcomed and my concerns relate only to the width of the proposed stair and kitchen windows”. The applicant has confirmed acceptance of the concerns of the case officer in respect to the width of the kitchen window (1 March 2010). The outstanding matter is the width and form of the stair window. In acknowledging that the proposed stair window “is rather dominant on the rear elevation”, revised plans were received 18/12/19. However, there was little reduction in the overall width of the window and negotiation continued. Consideration has been made to the applicant’s wish to introduce an escape window. The case officer stated 27/1/20 “You have suggested that the stair window is a ‘70s window (post – listing and unauthorised? Correspondence on 3/1976/0582 suggests the windows may have been installed by August 1976). I note from your email of 16 January 2020 that whilst Part B of the Building Regulations is not necessarily applicable for this building, it is absolutely essential for the present and future occupants of this dwelling and that Part B of the Building Regulations states a fire escape window should be at least 450 x 450mm. Can such fire escape windows be achieved within narrower stair and kitchen window openings?”. The applicant does not wish to revisit the revised plans in respect to the stair window. |  |  |  |  |
| Impact upon the special architectural and historic interest of the listed building and the character and appearance of Chatburn Conservation Area: The existing stair window, central in the rear elevation, is prominent and conspicuous in views of the listed building from Sawley Road. The date of its substantial and harmful widening has not been established. It is fortunate that the plans for 6/10/1257 provide some information to guide restoration considerations. Restoration work is contentious even when evidence survives to guide it and ensure authenticity and integrity. The SPAB Approach identifies: “The SPAB Approach therefore stands against Restorationist arguments that it is possible and worthwhile to return a building to its original — or imagined original — form” (page 8 and 9). “REPAIR NOT RESTORATION: Restoration of the kind opposed by Ruskin and Morris sets out to turn back the clock or to recreate the past. Its often a destructive process and may leave a building without the signs of age or evidence of its past interaction with people. Knowledge of an original design is not sufficient reason for erasing later change, particularly where this change has added positively to a building’s historic interest. Also, the Society believes that damaged or missing elements of a building do not necessarily need to be replaced, except where there is a functional need. Then, small-scale, localised reinstatement may be justified, but only if carried out for well-considered, practical reasons. Reinstatement for the sake of tidiness, or to recreate historic design or detail is at odds with the SPAB Approach” (page 13). ‘Conservation Principles, Policies and Guidance for the Sustainable Management of the Historic Environment’ (Historic England, 2008) identifies: “The concept of authenticity (paragraph 91) demands that proposals for restoration always require particularly careful justification. Reinstating damaged elements of work directly created by the hand of an artist normally runs counter to the idea of authenticity and integrity. However, the reinstatement of damaged architectural or landscape features in accordance with an historic design evidenced by the fabric of a place may not do so, if the design itself was the artistic creation, intended to be constructed by others, and the necessary materials and skills are available” (paragraph 128). “Any restoration inevitably removes or obscures part of the record of past change to a significant place, and so reduces its evidential value, as well as potentially affecting its historical and aesthetic values. Restoration may, however, bring gains by revealing other heritage values, such as the integrity and quality of an earlier and more important phase in the evolution of a place, which makes a particular contribution to its significance. Careful assessment of the values of the elements affected is essential” (paragraph 130). “Evidence of the evolution of the place, and particularly of the phase to which restoration is proposed, should be drawn from all available sources – from study of the fabric of the place itself (the primary record of its evolution), any documentation of the original design and construction process, and subsequent archival sources, including records of previous interventions. The results of this research and the reasoned conclusions drawn from it should be clearly set out” (paragraph 131). ‘Making Changes to Heritage Assets’ (Historic England, 2016) identifies: “If convincing evidence is available it may be appropriate to take opportunities to reinstate missing architectural details, such as balustrades and cornices or missing elements of a decorative scheme, using traditional methods and materials” (paragraph 31). “Restoration is likely to be acceptable if … The work proposed is justified by compelling evidence of the evolution of the heritage asset and is executed in accordance with that evidence … The work proposed respects previous forms of the heritage asset. No archaeological interest is lost if the restoration work could later be confused with the original fabric”. The proposed stair window design (width) is not in accordance with the evidence available indicating its historic form. The combination of C17 style stone mullion window and an arbitrarily chosen width confuses historic development and the ability to ‘read’ building evolution (the extracts below identify the importance of window detailing to building understanding and interest). This is detrimental to building integrity and is harmful to the listed building’s special architectural and historic interest. The Downham Conservation Area Management Guidance identifies: “Windows … Over time, the thickness and moulding of glazing bars, the size and arrangement of panes and other historic window details varied. Care is therefore needed in the repair and replacement of historic windows to ensure works are ‘honest’ and not historically misleading. Details should be appropriate to the date of the building or to the date when the window aperture was made”. ‘Traditional windows: their care, repair and upgrading’ (Historic England, April 2015) states: “Windows are the eyes of a building - they let in light and give views out - and profoundly affect its appearance. In addition, traditional windows bear witness to the artistic, social, economic and technological developments of past ages. Their design and detailing were influenced by contemporary architectural fashion, and reflected the status of a dwelling (and sometimes the individual rooms within it)” (page 3). ‘Listing Selection Guide: Domestic 1: Vernacular Houses’ (Historic England, 2011) states: “the formation and detailing of door and window openings and the application of decorative detail to stone and brickwork can provide evidence of date, the chronology of construction and alteration and the hierarchy of importance or status of different parts of a house ” (page 6). ‘Energy Efficiency and Historic Buildings: Application of Part L of the Building Regulations to Historic and Traditionally Constructed Buildings’ (Historic England, 2010) states: “The Importance of Windows in Older Buildings: Window openings and frames give a building’s elevation its character. They should not be altered in their proportions or details, as they are conspicuous elements of the design”. ‘Period House Fixtures and Fittings: 1300-1900’ (Hall L, 2007) states: “Stone mullions were used in areas of better quality stone such as the Pennines ... in the Pennine areas of Lancashire and Yorkshire a symmetrical facade seems to have commanded far less importance in the C17, and as late as c.1700 there are houses with unbalanced facades. This area also boasts some very distinctive local types of window (page 73-4). NPPG states that “substantial harm is a high test, so it may not arise in many cases” and the harm to the special architectural and historic interest of the listed building and the character and appearance of Downham Conservation Area is ‘less than substantial’. NPPF paragraph 196 requires that ‘less than substantial’ harm be weighed against any public benefits of the proposal. Contractor employment is a public benefit. Whilst the overall approach to works is welcomed, it is also acknowledged that proposals are to address unauthorised and damaging works and NPPF Paragraph 191 states “Where there is evidence of deliberate neglect of, or damage to, a heritage asset, the deteriorated state of the heritage asset should not be taken into account in any decision”. Harm from the prominent and inauthentic design of the stair window is not outweighed by scheme public benefits. |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: Therefore, in giving considerable importance and weight to the duties at section 16 and 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990 and in consideration to NPPF (2018) and Key Statement EN5 and Policies DME4 and DMG1 of the Ribble Valley Core Strategy it is recommended that listed building consent be refused. |  |  |  |  |
| RECOMMENDATION: |  | That listed building consent be refused for the following reason: The proposed stair window is harmful to the special architectural and historic interest of the listed building because of its design (principally width; not substantiated by the available evidence for restoration in the plan submitted with application 6/10/1257) and consequent impacts on authenticity and an understanding of building historical development. |  |  |