| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: |  |  |  |  | Date: |  | Manager: |  |  |  | Date: |  |
| Site Notice displayed |  | Photos uploaded |  |  |  |  |  |  |  |  |  |  |  |
| Application Ref: |  |  | 3/2020/0728 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  | 07/12/2020 |  |  |  |  |  |  |  |  |  |  |
| Officer: |  |  | RB |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  | Decision |  | APPROVAL |  |  |
| Development Description: |  |  |  |  | Proposed single storey rear extension and proposed entrance porch |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 8 Grindleton Road, West Bradford, BB7 4TE |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |
| No objections but had concerns that the materials for porch were unclear. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |
| No representations received in respect of proposed development |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: DMG1: General Considerations DMH5: Residential and Curtilage Extensions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Location: The application property is mid terrace dwelling located in Grindleton. The application site benefits from a garden to the front and private garden to the area. The application site is not within any areas of designated interest. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for the erection of a single storey extension to the rear and porch to the front. The porch will project forwards of the princpal elevation by 1.4m measuring 1.67m in width with a pitch roof measuring 2.2m at the eaves and 3m at the ridge. The porch will be faced with render with a slate roof to match the main dwelling. The rear extension will project beyond the rear wall of the dwelling by 2.7m measuring 5.7m in width. The extension will have a flat roof measuring 2.39m in height with two roof lanterns taking the total height to 2.6m. The extension will be faced with render to match the main dwelling with a fiberglass roof. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Residential Amenity: The porch to the front of the property will have minimal impact on no 6 and no 10 Grindleton Road due to the minimal forward projection and the separation distance between the windows on the front of both neighbours. The extension to the rear will project beyond the rear wall of 2.7m and both neighbours benefit from single storey extensions. The proposed extension will project between 0.5 m – 1m further north of these adjoining extensions. As such the development would have minimal impact on these neighbours due to the minimal rearward projection of the extension. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity: The proposed rear extension will not be visible on approach from the nearest highway and will therefore have minimal impact on the visual amenity of the area. The porch to the front of the property will have limited impact on the visual amenity of the area as the porch will remain subservient to the main dwelling in terms of its scale and design. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposed development will not result in any significant impact on the residential or visual amenity of the area. With all the above taken into consideration it is recommended accordingly. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  | That planning consent be granted |  |  |  |  |  |  |  |  |  |