| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: |  |  |  |  | Date: |  | Manager: |  |  |  | Date: |  |
| Site Notice displayed |  | Photos uploaded |  |  |  |  |  |  |  |  |  |  |  |
| Application Ref: |  |  | 3/2020/0740 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  | 11/12/20 |  |  |  |  |  |  |  |  |  |  |
| Officer: |  |  | AD |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  | Decision |  | Approval |  |  |
| Development Description: |  |  |  |  | Essential structural repairs following vehicular collision including partial rebuild of the south east corner including part of the roof coverings to make safe. |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | Hark to Bounty Inn Townend Slaidburn BB7 3EP |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |
| No objections. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Need Streetworks Licence for the repairs - works to rebuild will need to be carried out from the highway - need safe working zone for both the builder and the highway users. Builder will need to devise a safe working scheme so that the street works team can produce a suitable permit (Standard note suggested). LCC Archaeology: Scheme of repairs appropriate and impact assessed in the Heritage Statement as acceptable, although there is no mitigation - suggest a rapid photographic survey of portion of the building impacted by the works, prior to the repairs commencing, and a watching brief during excavations for new foundations. The former may well also be of use in the rebuilding works. Condition suggested. Historic England: Do not wish to offer any comments. Suggest seeking views of the RVBC specialist conservation adviser. RVBC Countryside: Protected species condition suggested. Ancient Monuments Society: Support. Agree with Heritage Impact Statement conclusions and efforts to salvage and reuse masonry, slate tiles, lintels, etc in order to retain and restore as much historic building fabric as possible. Some material will be lost through the reconstruction and repairs but like-for like and is essential for the long-term conservation of the building. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |
| None received. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Planning (Listed Buildings and Conservation Areas) Act 1990. ‘Preservation’ in the duties at sections 16, 66 and 72 of the Act means “doing no harm to” (South Lakeland DC v. Secretary of State for the Environment [1992]). Ribble Valley Core Strategy: Key Statement EN5 – Heritage Assets Key Statement EN2 – Landscape Policy DMG1 – General Considerations Policy DME4 – Protecting Heritage Assets Policy DMG2 – Strategic considerations NPPF NPPG |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: The Borough Council was informed of the vehicle impact in April 2020 and has advised the applicant in respect to the compilation and processing of the current application. 3/2017/0079 - Proposed structural works to west facing gable end of building to include lateral restraint and associated remedial building works. LBC granted 29/6/2017. 3/2001/0219 - Alterations to doorway between bar lounge and dining room. LBC granted 4/5/2001. 3/1982/0643 – Installation of bathrooms in bedroom nos. 2 & 4. LBC granted 3 January 1983. 3/1978/1153 – Extension. PP granted 24 January 1979. 3/1977/0438 – Proposed external fire escape. PP granted 26 May 1977. 3/1975/1314 – Proposed additional residential accommodation. PP granted 13 February 1976. Plans show: front elevation end bay first floor windows to be new (and that a previous window has been infilled); rear elevation first floor window and former garage opening treatment to be new; end bay divided by wall but open to roof? 3/1974/0921 – Proposed internal alterations to provide additional dining accommodation. LBC granted 2 December 1974. BO1709 – Break out 2 new window openings at front elevation, new toilet accommodation and form tap room. PP granted October 1971. BO1650 – Convert building into shop. PP granted 2 July 1971. Plans show that relates to end bay. No windows shown to gable. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The Hark to Bounty is a Grade II listed (16 November 1954; amended 16 November 1983), probably early C17 and late C18, public house prominently sited within Slaidburn Conservation Area. The list description identifies: “Public house, probably early C17th and late C18th. Sandstone rubble with slate roof. The earliest part, to the right, has 3 chamfered doorways with Tudor-arched heads. Each has a chamfered window surround to its right, the left-hand one having its sill lowered. A continuous drip course terminates beyond the right-hand door … On the 1st floor is a door with plain stone surround and 5 windows with plain stone surrounds and 6-pane sashes. The right-hand gable has a blocked doorway (now a window) with chamfered surround and Tudor-arched head. On the 1st floor are the remains of a rebated and chamfered surround to a 6-pane sash. Above is a chamfered attic window. Adjoining to the left (west) is the C18th portion … Interior: On the 1st floor at the east end is a large room open to the roof, having 4 trusses with ties, light collars and queen struts. At the east end, dividing it from a further room beyond, is a 5th truss of slightly steeper pitch, having a king post from tie to ridge and 2 braces from tie to principals. Beneath, with the muntins tenoned into the tie, is a wooden screen of vertical panels with deeply moulded borders”. The Hark to Bounty is within the setting of a number of listed buildings including ‘5, 7 and 9 Townend’, ‘3 Townend’, ‘1 Townend’ and ‘Youth Hostel’ opposite (all Grade II). Pevsner ‘The Buildings of England: Yorkshire West Riding’ (1967) states: “The seat of the courts of the Forest of Bowland. The Court Room is preserved” (page 489). The Slaidburn Conservation Area Appraisal (The Conservation Studio consultants 2005; subject to public consultation) identifies: The Hark to Bounty to be a Focal Building; the adjacent No. 7 Townend to be a Building of Townscape Merit positively contributing to the conservation area. “Architectural and historic interest of the conservation area’s buildings, including 39 listed buildings”; “Remarkable surviving historic appearance with almost complete absence of 20th century alterations and accretions”; “Two particularly fine ensembles of historic buildings … The Hark to Bounty, King’s Youth Hostel and Waterloo Building in the centre of the village” (Summary of special interest). “The Hark to Bounty Inn, overlooking a wide road junction, holds a commanding position as one approaches from the south” (Key views and vistas). “The Hark to Bounty Inn was also built in vernacular style in the early 17th century although it has much 18th century fabric” (Architectural and historic character). “The Hark to Bounty is an inn of 16th century origin, the upstairs of which was used as a courtroom until the 1930's, following the demolition of the Moot Hall in the mid-19th century” (Key listed buildings). |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: It is proposed to repair (including rebuilding) the south-east corner of the listed building following a vehicle impact in March 2020. The submitted Heritage Impact Assessment identifies at 4.1.1: Proposed development seeks to repair and restore the presently damaged south-eastern corner of the building, which will involve: • Removal of existing roof covering • Repairs of failed components • Occasional replacement of components beyond repair • Taking down and rebuilding the structurally compromised south-eastern corner of the building The roof covering will be carefully stripped over the end rooms, including the front and rear pitches, with salvageable ridge and slate tiles retained for reinstatement. Those slates which are damaged and therefore, unusable, will be disposed of. The removal of the roof will also involve the removal of the front eaves and gable verge at the south-eastern corner of the building. Should the verge detailing require replacement due to damage or poor condition, templates will be taken of the detailing to ensure accurate like-for-like replacements. Certain elements of the repairs of failed components, such as the damaged slate tiles and gable verge detailing, will result in some inevitable loss of original fabric. However, such loss will be as minimal as possible in order to retain this element of the building’s architectural and artistic interest. The proposed development also seeks to take down the damaged masonry at the south-eastern corner of the building from the eaves to below ground level, tapered down from retained sections of walling, which is necessary to make safe and restore its structural integrity. This will involve the removal of the both windows at ground and first-floor levels on the south-facing façade as well as the three southernmost windows on the east-facing gable elevation. Key stone features, including sills, lintels, mullions, and quoins will be annotated and stored separately in order to ensure their correct original placement during reconstruction. The existing windows will be carefully reinstated in their original position; however, the presently cracked glazing is to be replaced with matching plate glass. The external masonry will be rebuilt as close to existing as possible, with additional structural support for this vulnerable part of the building, strengthening it in order to limit extensive damage from potential future incidents. The masonry will be rebuilt using a lime mortar (1-part hydraulic NHL3.5 lime and 3 parts sand) subject to approval. This corner, which forms part of the earliest phase of the building, is considered to hold a great deal of architectural and historic illustrative interest as a contributor to its overall significance. Although there is potential for some of this historic illustrative interest to be lost as a result of this intervention, the proposed development, in reusing the existing materials to rebuild the south-eastern corner, seeks to limit the scale of this loss. As the work is considered to be essential for the long-term conservation of the building, the proposed development is considered to result in an overall neutral impact to the significance of its physical fabric. Comprehensive specifications for dismantling (including the labelling of important stone elements) and reconstruction have been submitted. It is proposed to clean elements of existing fabric (no details). Fabric will be stored for re-use (site storage containers shown on site plan). On 14/1/21 the case officer requested further detail/confirmation of: “A number of window frames are incorrectly shown in the elevation plans (e.g. the first floor right-hand side gable window is a 3/3-pane sash); Can any of the existing historic windows with significance be repaired rather than replaced? Are there opportunities for enhancement of significance e.g. replacing modern inappropriate windows and inappropriate uPVC downpipe? Window surrounds are referred to as ‘art stone’”. The agent responded 20/1/21: “attached an amended set of drawings with the noted errors rectified. I have also spoken to the applicant and can confirm the following, in response to your queries: - ‘We propose to re-use all windows, doors and stone surrounds. - No window replacements will be procured unless pre-approved by the LPA, and only if absolutely necessary. - All stone work will be re-use or reclaimed from this site. - The rainwater goods to be removed/replaced will be salvaged or like replacement cast components”. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon the special architectural and historic interest of the listed building, the setting of listed buildings, the character and appearance of Slaidburn Conservation Area and the cultural heritage of the Forest of Bowland AONB: The proposed works are those necessary to ensure the proper preservation of the listed building. Conditions are suggested in respect to the anticipated need to replace (rather than repair) elements of fabric and design and to ensure the prompt completion of works following their commencement. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bats: The condition suggested by RVBC Countryside is necessary. Highways: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: Therefore, in giving considerable importance and weight to the duties at section 16, 66 and 72 (where relevant) of the Planning (Listed Buildings and Conservation Areas) Act 1990 and in consideration to NPPF (2018) and Key Statement EN5 and Policies DME4 and DMG1 of the Ribble Valley Core Strategy it is recommended that listed building consent be approved. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  | That listed building consent be granted subject to conditions. |  |  |  |  |  |  |  |  |  |