| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Application Ref: |  | 3/2020/1077 |  |  |  |
| Date Inspected: |  | 15.03.21 |  |  |  |
| Officer: |  | RB |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  | REFUSAL |
| Development Description: |  |  |  | Proposed first floor rear extension |  |
| Site Address/Location: |  |  |  | 11 Mellor Brook, Mellor Brook, BB2 7PH |  |
| CONSULTATIONS: |  |  |  | Parish/Town Council |  |
| No comments received within consultation period |  |  |  |  |  |
| CONSULTATIONS: |  |  |  | Highways/Water Authority/Other Bodies |  |
| N/A |  |  |  |  |  |
| CONSULTATIONS: |  |  |  | Additional Representations. |  |
| Two letters of representation have been received from one address with the following objections: - Development is out of character with the area - Loss of light to property - Overlooking from balcony |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |
| Ribble Valley Core Strategy: Policy DMG1 – General Considerations Policy DMG2 – Strategic Considerations Policy DMH5 – Residential & Curtilage Extensions National Planning Policy Framework (NPPF) |  |  |  |  |  |
| Relevant Planning History: No relevant planning history. |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |
| Site Description and Surrounding Area: The application site relates to a mid-terrace dwelling located in Mellor. The application property benefits from a private garden area to the rear. |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for the erection of a first-floor extension to the rear of the property. The first-floor extension will extend off the eaves of the existing property to form a south facing gable. The extension will project off the roof at first floor by 3m measuring 2.9m in width. The eaves of the new gable will measure at approximately 4.2m from ground level and approximately 5.3m at the ridge when measured from ground level. To the rear elevation a balcony is proposed. The balcony will project beyond the rear wall by 0.4m measuring 1.8m in width. |  |  |  |  |  |
| Impact Upon Residential Amenity: Ribble Valley Core Strategy Policy DMG1 states that “development must not adversely affect the amenities of the surrounding area and provide adequate day lighting and privacy distances”. The development site is a mid-terrace dwelling therefore the attached neighbours have the potential to be affected by the development. Due to the solar orientation of the properties the neighbour to the west of the development would not be significant affected by the first-floor extension in terms of loss of light. The existing first floor window on the side of this neighbour does not serve a habitable room therefore any loss of light to this window would not warrant refusal of the application. The neighbour to the east may be affected by the development as it would create some loss of light when compared to the existing situation however due to the main bulk of the extension not projecting any further rearwards than the rear of the main dwelling the loss of light would mainly relate to the roof of the adjacent neighbour at no 9. To the rear elevation of the first-floor extension a set of doors are proposed, and a balcony is proposed to project by 400mm. The siting of the balcony on this elevation would mean that the rear garden areas of no 13 and no 9 would be overlooked by the patio doors and the balcony area. As such it is considered that the proposed development would result in a detrimental loss of privacy to the neighbouring occupiers at no 13 and no 9 Mellor Brook. The neighbour dwellings to the south of the property are bungalow properties on Woodfold Close. The rear elevations of these neighbours are over 25m from the rear elevation of the first-floor extension. Therefore, there is a sufficient separation distance between the development site and the rear wall of these neighbours. However, the rear boundaries of these neighbours are within 8-9m of the proposed first floor extension and proposed balcony. As such the balcony would result in the loss of privacy to the rear garden areas of no 4 and 5 Woodfold Close. |  |  |  |  |  |
| Visual Amenity/External Appearance: Ribble Valley Core Strategy Policy DMG1 states that “development should be sympathetic to existing and proposed land uses in terms of its size, intensity and nature as well as scale, massing, style, features and building materials”. The application site is a terrace dwelling located within Mellor. The property benefits from a private garden area to the rear. The rear elevation is not visible from the public highway. The adjacent dwelling at no 9 benefits from a first-floor extension that forms a south facing gable that is of a similar design to the proposed development. Therefore, the extension would not appear of character in the area and would partially screen an existing first floor extension that has a Skillon roof. The partial screening of the Skillon roof would be considered a benefit to the visual amenity of the area. The proposed first floor extension would remain subservient to the main roof at the ridge of the gable is set significantly lower than the ridge of the terrace row and the eaves match the eaves of the two-storey element of the property (front). Although the balcony would be the first of its kind on the rear of the terrace row it is not considered that the balcony would be detrimental to the character of the area. |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: In conclusion, I believe that the proposed extension by virtue of the proposed doors and balcony will have a detrimental impact on the residential amenity of the neighbouring properties and such the proposal does not comply with Policy DMG1 and DMH5 of the Ribble Valley Core Strategy. |  |  |  |  |  |
| RECOMMENDATION: |  |  | That planning consent be refused for the following reason(s) |  |  |
| 01 | The proposed first-floor rear extension, by virtue of its design and proximity to neighbouring properties, would result in the overlooking and loss of privacy to the rear garden areas of 9 and no 13 Mellor Brook and 4 and 5 Woodfold Close. The proposal is therefore contrary to Ribble Valley Core Strategy Policies DMG1 and DMH5, and paragraph 127 of the NPPF which requires a high standard of amenity for existing land users. |  |  |  |  |