| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: |  | Officer: | BT |  |  |  | Date: | 22/7/2021 | Manager: |  |  |  | Date: |  |
| Site Notice displayed |  | N/A | Photos uploaded |  |  |  | Y |  |  |  |  |  |  |  |
| Application Ref: |  |  |  | 3/2021/0530 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 23/6/2021 |  |  |  |  |  |  |  |  |  |  |
| Officer: |  |  |  | BT |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  | Decision |  | Refusal |  |  |
| Development Description: |  |  |  |  |  | Proposed loft conversion and single storey extension to rear. |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  |  | 102 Branch Road, Mellor. BB2 7NY |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |
| Mellor Parish Council consulted on 15/6 – no response. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |
| BT Openreach consulted on 15/6/21 – no objections. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |
| Five objections have been received in respect to the application. These objections are summarised as: - Impact of the proposal upon residential amenity - Impact of the proposal upon visual amenity The above objections are addressed in the corresponding sections of the report below. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1 – Development Strategy Key Statement DS2 – Presumption in Favour of Sustainable Development Policy DMG1 – General Considerations Policy DMG2 – Strategic Considerations Policy DMG3 – Transport and Mobility Policy DMH5 – Residential and Curtilage Extensions NPPF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: No recent planning history relevant to the determination of the application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a detached property in Mellor. The property is constructed from brick and render, concrete roof tiles and white UPVC windows. The surrounding area is residential with large areas of open countryside to the periphery. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for the construction of a front dormer window, rear box dormer, single storey rear extension and extension of the property’s front driveway to include a dropped kerb vehicle crossing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of development: The proposal is a domestic extension to a dwelling and is acceptable in principle subject to an assessment of the material planning considerations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Residential Amenity: The proposal would incorporate several new windows including three windows on the proposed rear box dormer, a window on the proposed front dormer and three windows on the proposed single storey rear extension. The three windows on the proposed single storey rear extension would primarily allow views into the property’s rear garden while the window on the proposed front dormer would face towards the front curtilages of properties on the Western side of Branch Road which are already viewable within the public realm. As such, it is not considered that the proposed rear extension or front dormer would provide any new opportunities for overlooking into private areas. The Southernmost window on the proposed rear box dormer would be sited approximately 4 metres above ground floor level and would look towards a conservatory within the rear garden of No. 12 Woodfold Close however given that the dormer would be located approximately 30 metres from the conservatory it is not considered there would be any significant loss of privacy to the residents of No. 12 Woodfold Close. The Northernmost dormer bedroom window would be sited approximately 4 metres above ground floor level and would look towards the rear gardens of No. 13, 14 and 15 Woodfold Close. This window would be sited approximately 25 metres from the rear gardens of No. 13 and 14 Woodfold Close and as such would not significantly affect the privacy of these residents however the Northernmost dormer bedroom window would be sited considerably closer to the rear garden of No. 15 Woodfold Close at a distance of approximately 12 metres away. Consequently, the window’s elevated position would allow clear views into the rear garden of No. 15 Woodfold close which contains a conservatory which in turn would significantly compromise the privacy of these residents. Accordingly, it is considered that the rear box dormer would lead to an unacceptable loss of privacy due to its proximity to and elevated position above the rear garden of No. 15 Woodfold Close. The North facing gable end of No. 102 Branch Road is situated approximately 2.5 metres from the rear curtilage boundary of No. 16 Woodfold which lies directly to the North. The proposed works include a roof lift which would increase the roof height of No. 102 by an additional 1.6 metres (5.2 feet). Site visit photographs and desktop analysis show that No. 16 Woodfold Close already experiences a considerable amount of overshadowing within its rear curtilage due to its close proximity to the North of No. 102 Branch Road which has a noticeably higher roof pitch than No. 16 Woodfold Close. Accordingly, the additional increase in roof pitch height to No. 102 Branch Road would almost certainly exacerbate the existing level of overshadowing within the rear curtilage of No. 16 Woodfold Close which includes a kitchen window and a conservatory. As such, it is considered that the proposed roof lift to No. 102 Branch Road would lead to an unacceptable loss of natural light for the residents of No. 16 Woodfold Close due to its proximity to and elevated position above the rear curtilage of No. 16 Woodfold Close. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity: The proposal involves a number of alterations to the existing property which include a front dormer window, rear box dormer, single storey rear extension and roof lift which would increase the height of the property’s existing roof pitch by 1.6 metres (5.2 feet). The front dormer would have a width and height of 1.9 metres and 2 metres and as such would be relatively modest in terms of size however the pitch of the dormer would adjoin the roof pitch of the main property which in turn would not read as subservient to the dimensions of the main property from a visual perspective. The proposed roof lift would increase the height of the property’s existing roof pitch by 1.6 metres (5.2 feet). The existing property is clearly viewable from Branch Road and most of the other properties which surround it therefore the visual impact of the proposed roof lift would be significant. Moreover, the existing roof pitch of No. 102 Branch Road is noticeably higher than No. 16 Woodfold Close therefore the proposed roof lift would result in significant visual imbalance between the two properties. Furthermore, roof lifts to bungalow properties are an uncommon feature within the surrounding area therefore it is considered that the proposed roof lift would be an anomalous addition to the existing street scene which in turn would be harmful to the visual amenities of the surrounding area. The single storey rear extension would have a height of 2.8 metres with an outwards projection and width of 4 metres and 8.2 metres respectively. The single storey extension would also be accompanied by a rear box dormer directly above which would have a height of 2.4 metres and a width 12.7 metres spanning almost the entire width of the property’s rear roof plane. These additions would not be viewable within the public realm however they would be clearly viewable to several residents on Woodfold Close. As such, it is considered that the cumulative impact of the rear single storey extension and rear box dormer, in conjunction with the proposed roof lift, would have an overbearing effect upon the residents residing at No. 12, 13, 14, 15 and 16 Woodfold Close. Policy DMG1 of the Ribble Valley Core Strategy states that all development must ‘be sympathetic to existing and proposed land uses in terms of its size, intensity and nature as well as scale, massing and style’ and ‘not adversely affect the amenities of the surrounding area’ however given the public visibility, scale and massing of the proposed works it is considered that the proposal as a whole would be an over dominant and anomalous addition to the existing property and surrounding pattern of development that would ultimately be harmful to the visual amenity of the surrounding area and contrary to the aims of the above policy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: No ecological constraints were identified in relation to the proposal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways: Lancashire County Council Highways have not been consulted on the proposal however given that the proposed works to the property’s front driveway would increase the property’s off-street parking provision it is not considered that the proposal would have any adverse impact upon highway safety. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposal raises concerns with regards to residential amenity in as much that the proposed works would result in a significant loss of natural light and privacy for the residents of No. 16 and No. 15 Woodfold Close respectively. Furthermore, the proposal raises some significant concerns in relation to visual amenity in as much that the cumulative visual impact of the proposed works would have an overbearing and disruptive presence upon the neighbouring residents and visual amenities of the surrounding area. It is for the above reasons and having regard to all material considerations and matters raised that planning consent be refused. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  |  | That planning permission be refused for the following reason(s): |  |  |  |  |  |  |  |  |  |
| 01 | The proposal is considered to be in conflict with policies DMG1 and DMH5 of the Ribble Valley Core Strategy in as much that the proposed roof lift and rear box dormer would lead to an unacceptable loss of natural light and privacy for the residents of No. 16 and No. 15 Woodfold Close respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 02 | The proposal is considered to be in conflict with policies DMG1 and DMH5 of the Ribble Valley Core Strategy in as much that the proposed works would be a disproportionate, over dominant, and unsympathetic addition to the original property and existing pattern of housing by virtue of their public visibility, scale and massing, all of which would be harmful to the visual amenity of the surrounding area. |  |  |  |  |  |  |  |  |  |  |  |  |  |