| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | BT |  |  |  | Date: | 26/8/2021 | Manager: |  |  |  | Date: |  |
| Site Notice displayed | N/A | Photos uploaded |  |  |  | Y |  |  |  |  |  |  |  |
| Application Ref: |  |  | 3/2021/0702 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  | 15/7/2021 |  |  |  |  |  |  |  |  |  |  |
| Officer: |  |  | BT |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  | Decision |  | Approval |  |  |
| Development Description: |  |  |  |  | Proposed extension of the existing garage which will include a change in roof profile from flat to pitched. |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 75 Mitton Road, Whalley. BB7 9JN |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |
| Whalley Parish Council consulted by email on 13/7/21 – no response. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |
| Lancashire County Council Highways consulted by email on 13/7/21 – no response. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |
| None. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1 – Development Strategy Key Statement DS2 – Presumption in Favour of Sustainable Development Policy DMG1 – General Considerations Policy DMG2 – Strategic Considerations Policy DMG3 – Transport and mobility Policy DMH5 – Residential and Curtilage Extensions NPPF |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: No recent planning history relevant to the determination of the application. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a semi-detached property in Whalley. The property’s existing garage to be extended consists of render and red brick, felt roofing and timber windows and metal / timber doors. The proposal site is situated within a residential area to the side of a busy main road in the North-western area of Whalley. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for the extension of an existing garage. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of development: The proposal is a domestic extension to a dwelling and is acceptable in principle subject to an assessment of the material planning considerations. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Residential Amenity: The extended section of the garage would contain three windows on its North-eastern side elevation which would be located in an identical position to the three windows on the existing North-eastern elevation of the garage. Furthermore, the garage would not be used for residential or recreational purposes therefore it is not considered that the proposed works would lead to any loss of privacy for any neighbouring residents. The proposed works comprise an extension to the North-eastern side of the property’s garage and the addition of a gabled roof which would replace the garage’s current flat roof design. The property’s garage is located approximately 3 metres away from the side elevation of No. 2 Whittam Road however given that this side elevation is located to South-west of the garage it is not considered that the modified roof pitch or additional width of the extended garage would lead to any loss of natural light or outlook for the adjacent neighbouring residents. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity: The proposed works would increase the width and height of the existing garage by 3.6 and 1.5 metres respectively. These alterations to the existing garage would be clearly visible from Whittam Road and partially visible from Mitton Road therefore the proposal would have a noticeable visual impact however it is not considered that the proposal would be an over dominant feature in as much that the modified garage would still be wholly subservient to the main property with regards to its eaves and roof pitch height with no more than a standard width incorporated to accommodate two vehicles in a double garage. Furthermore, the front South-eastern elevation of the garage would not project beyond the side elevation of the main property or any other properties on Whittam Road which in turn would keep the visual impact of the proposed works to a minimum. Moreover, the garage would be set back approximately 30 metres from the main road to the rear of the property within a quiet side street which would reduce the overall visual impact of the proposed works. Furthermore, the proposal would largely retain the external features of the existing garage in as much that the extended garage would consist of red brick with render, timber windows and metal / timber doors while the proposed gabled roof would match the gabled roof design of the property’s rear kitchen extension. As such, it is not considered that the proposed works would have any undue impact upon visual amenities of the immediate or wider area. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: No ecological constraints were identified in relation to the proposal. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways: At the time of writing no response had been received from Lancashire County Council Highways however given that the proposed works would create an additional off street parking space at the property it is not anticipated that the proposal would have any undue impact upon highway safety. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposal does not raise any concerns in relation to residential amenity in as much that the extension would not lead to any loss of privacy, natural light or outlook for any neighbouring residents. The extension would sufficiently integrate with the existing garage and main property without being an over dominant feature or having any harmful impact upon the visual amenities of the immediate or wider area. It is for the above reasons and having regard to all material considerations and matters raised that the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  | That planning permission be granted. |  |  |  |  |  |  |  |  |  |