| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: |  |  |  |  | Date: |  | Manager: |  |  |  | Date: |  |
| Site Notice displayed |  | Photos uploaded |  |  |  |  |  |  |  |  |  |  |  |
| Application Ref: |  |  | 3/2021/0985 (PA) |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  | 15/11/2021 |  |  |  |  |  |  |  |  |  |  |
| Officer: |  |  | AD |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  | Decision |  | Refusal |  |  |
| Development Description: |  |  |  |  | Proposed single storey extension to the rear, removal of ceiling to second floor rooms and insertion of 3 conservation rooflights in rear roof pitch. New stud walls to form en-suite on the second floor. Removal of existing garage prior to construction of replacement. |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 84 Whalley Road Clitheroe BB7 1EE |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |
| No objections |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |
| LCC Archaeology: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Content to leave decisions to the RVBC conservation specialist. Historic amenity societies: Consulted, no representations received. LCC Highways: No objection subject to conditions (implementation of parking & turning). RVBC Countryside: Note that trees within influencing distance haven’t been recorded on plan. Condition re: the event that bats are found. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |
| None received. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement EN5 – Heritage Assets Policy DMG1 – General Considerations Policy DME4 – Protecting Heritage Assets Policy DMH5 – Residential and Curtilage Extensions Planning (Listed Buildings and Conservation Areas) Act 1990. ‘Preservation’ in the duties at sections 16 and 66 of the Act means “doing no harm to” (South Lakeland DC v. Secretary of State for the Environment [1992]). NPPF NPPG |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2021/0985 - Proposed single storey extension to the rear, removal of ceiling to second floor rooms and insertion of 3 conservation rooflights in rear roof pitch. New stud walls to form en-suite on the second floor. Removal of existing garage prior to construction of replacement. PP under consideration. 3/1998/0225 & 0226 – Non-illuminated shop sign. LBC and AC granted. 3/1990/0413 – Demolition of external staircase. LBC granted. 3/1977/0850 – New window frames and cement rendering to top half of building. LBC granted 16/9/1977. 6/2/1331 – Change of use of ground floor front room to office. Refused. Appeal. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: ’78-88 Whalley Road’ is a Grade II listed (30/9/1976) terraced row of the early C19 prominently sited on the approach to Clitheroe town centre from the south-west. Primrose Mill pond area (environmental improvements being undertaken) is to the west. The list description identifies: “Early C19. 3 storeys, rendered, with slate and stone slate roofs and modillion eaves cornice. 1 window each, except for No 80 which has 2, stone surrounds, no glazing bars. Doorways have stone surrounds with cornices. Small mid C19 shop front to No 84. Modern windows and modern door to the end property which is occupied as a club. Nos 60 to 164 (even) form a group, all except Nos 76 to 110 being buildings of local interest only”. Upvc windows and a door have recently been installed to the rear elevation of No. 84. LBC (3/1977/0850) suggests that 2/2- sash windows were replaced at the front elevation. The row adjoins and is within the setting of ‘Commercial Hotel (including Stable Block to rear)’ and ’90-110 Whalley Road’ (both Grade II and of similar construction period). The row rear garden layout and access is as shown on the 1840s OS map. No. 84 retains the outbuilding close to the house shown on this map. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Planning permission is sought for alterations (removal of lower ground floor rear walling including historic door and window openings; second floor removal of ceilings, installation of 3 rear rooflights and installation of rear room en-suite) and a rear elevation rear garden room extension. It is also proposed to demolish an existing modern garage and construct a new garage closer to the rear access. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon the special architectural and historic interest of the listed building and the setting of listed buildings: The proposals are harmful to the special architectural and historic interest and setting of the listed building and the setting of the adjoining listed buildings. Rear elevations are visible from the access shown on the 1840s OS map. Removal of window and door and extension The lower ground floor window and door (and adjoining walling) are important elements of historic fabric (see stone surrounds) and also contribute to building aesthetics and an understanding of historic access/movement and planform. ‘Making changes to heritage assets’ (Historic England, 2016) is particularly relevant at paragraph 15, 42, 43, 48 and 50. ‘Alterations to listed buildings’ (IHBC, 2021) is particularly relevant at paragraph 4.1.2, 4.1.3 and 4.1.5. The proposed extension is across the whole width of the property and obscures views of the lower ground floor (window and door). The flat roof form, lantern and triple-light glazed doors are incongruous and conspicuous elements which draw the eye from the historic building. ‘Making changes to heritage assets’ is particularly relevant at paragraph 41, 48 and 50. Rooflights The rear roofslope of the row is prominent and has few disruptions (its unbroken line is part of its significance). The proposed rooflights are prominent. Incongruous and conspicuous. Two of the rooflights are to a bedroom which already has a window. No details submitted of impact to existing principal members of the roof structure. ‘Making changes to heritage assets’ is particularly relevant at paragraph 48. ‘Alterations to listed buildings’ is particularly relevant at paragraph 5.11.6. Removal of ceilings No evidence has been submitted to suggest that the ceilings are not an original/historic or important feature of the second floor of the late Georgian listed building (presumably inserted to hide the roof structure from view and form a proportioned space). ‘Making changes to heritage assets’ is particularly relevant at paragraph 42, 44 and 47. However, it is suggested that works harming the interior only do not form part of reasons for refusal on this planning application. En-suite No details of plumbing (interior and exterior) or ventilation (impact to roof structure when ceiling removed?) have been submitted. Original planform appears to have been retained (similar over all floors?). However, the proposed partition along the whole of one side of the room changes room proportions. ‘Making changes to heritage assets’ is particularly relevant at paragraph 45. However, it is suggested that works harming the interior only do not form part of reasons for refusal on this planning application. Proposed garage The 1890 OS map shows that outbuildings adjoining the access road were a historic feature of the row (however, existing structures are modern and of no merit). The existing and proposed garages are prominently sited and incongruous and conspicuous in their historic setting because of their form and materials (rendered walls; overtly modern single metal door). Head of Service comments: It is noted that there are numeous outbuildings situated at the rear of these properties and given the presence of an existing garage in this location the replacement garage , although modern design ad materials would not have a harmful impact to warrant refusal on this element of the scheme. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Residential Amenity: The proposals have an acceptable impact upon the amenity of adjoining and nearby residents. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ecology: The submitted bat survey concludes that removal of the garage will not impact any local bat population. Removal of ceiling and insertion of roof windows will not disturb roosting bats or remove high value roosting potential. The condition suggested by RVBC Countryside is a reasonable and necessary precaution. There are trees within falling distance of the development (see Q7 of 3/2021/0985). However, the potential loss/impact to these trees is not considered to be a reason for refusal. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways: The development is acceptable subject to the reasonable and necessary condition suggested by LCC Highways. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: Therefore, in giving considerable importance and weight to the duties at section 16 and 66 of the Planning (Listed Buildings and Conservation Areas) Act 1990 and in consideration to NPPF, NPPG and Key Statement EN5 and Policies DME4 and DMG1 of the Ribble Valley Core Strategy it is recommended that planning permission be refused. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  | The proposal is harmful to the special architectural and historic interest of the listed building and the setting of listed buildings because of the loss and alteration of important historic fabric, the installation of incongruous and conspicuous and rooflights, the incongruous and conspicuous design of the extension. This is contrary to Key Statement EN5 and Policies DME4 and DMG1 of the Ribble Valley Core Strategy. |  |  |  |  |  |  |  |  |  |