| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | SH |  |  |  | Date: | 04/01/2022 | Manager: |  |  | Date: |  |
| Application Ref: |  |  |  | 3/2021/1230 |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | N/A |  |  |  |  |  |  |  |  |
| Officer: |  |  |  | SH |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Certificate of Lawfulness for a proposed use of construction of a single storey rear extension. |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | Kelvedon, Beechthorpe Avenue, Waddington, BB7 3HT |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |
| The proposal will be assessed against the provisions of Schedule 2, Part 1, Class A of the Town and Country Planning (General Permitted Development) Order 2015 (as amended). |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2021/0801 - Proposed two storey and single storey side extension with alterations to the property's driveway. (Approved) |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a semi-detached property within the settlement boundary of Waddington. The property consists of grey cement render, slate roof tiles and white UPVC doors and windows. The surrounding area is primarily residential, comprising of a mixture of semi-detached two storey properties and bungalow dwellings. The site is situated within the Forest of Bowland AONB, and within the Waddington Conservation Area. |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: The proposed development is for the construction of a lean-to roof rear single storey extension. The dimensions will come to approximately 3m on the outward projection from the rear elevation, measuring around 4m in width, with an eaves height of approximately 2.4m, and the maximum roof height measuring around 3.4m from ground level. The application site benefits from its householder permitted development rights. |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: This application seeks a determination as to whether the proposed development requires planning consent. In this instance, the relevant part of the Town and Country Planning (General Permitted Development) Order 2015 (as amended) to consider is Schedule 2, Part 1, Class A. |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: In order for the proposal to be permitted development, the works need to satisfy a number of criteria as comprised in Part 1 Class A of the Town and Country Planning (General Permitted Development) Order 1995 (as amended) for the enlargement, improvement or other alteration of a dwellinghouse. A.1 Development is not permitted by Class A if – (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); Permission for use as a dwellinghouse was not granted by virtue of class M, N, P or Q of Part 3. (b) as a result of the works the total area of ground covered by buildings within the curtilage of the dwelling house (other than the original dwelling house) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwelling house); The single storey extension would not exceed 50% of the total area of curtilage of the property. (c) the height of the part of the dwelling house enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwelling house; The height of the proposed single storey extension will not exceed the height of the existing dwelling. (d) the height of the eaves of the part of the dwelling house enlarged or improved or altered would exceed the height of the existing dwelling house; The height of the eaves of the proposed extension will not exceed the height of the eaves of the existing dwellinghouse. (e) the enlarged part of the dwelling house would extend beyond a wall which - (i) fronts a highway; and - (ii) forms either the principal elevation or a side elevation of the original dwelling house; The proposed single storey extension would not extend beyond a wall which is considered to front a highway. (f) subject to paragraph (g), the enlarged part of the dwelling house would have a single storey and – (i) extend beyond the rear wall of the original dwelling house by more than 4 metres in the case of a detached dwelling house or 3 metres in the case of any other dwelling house, or (ii) exceed 4 metres in height; The single storey extension will protrude beyond the rear wall by 3m exactly and would measure a total height of approximately 3.4m to the highest point. (g) until 30th May 2019, for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and — (i) extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse, or(ii) exceed 4 metres in height;” The proposed development would be sited on article 2(3) land. h) the enlarged part of the dwelling house would have more than one storey and – (i) extend beyond the rear wall of the original dwelling house by more than 3 metres, or be within 7 metres of any boundary of the curtilage of the dwelling house opposite the rear wall of the dwelling house; The proposed rear extension would be single storey. (i) the enlarged part of the dwelling house would be within 2 metres of the boundary of the curtilage of the dwelling house, and the height of the eaves of the enlarged part would exceed 3 metres; The proposed single storey rear extension would be within 2 metres of the boundary of the curtilage of the dwellinghouse however it would measure approximately 2.4 metres in height to the eaves. (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would – (i) exceed 4 metres in height, (ii) have more than one storey, or (iii) have a width greater than half the width of the original dwellinghouse; or (k) it would consist of or include— (i) the construction or provision of a veranda, balcony or raised platform, (ii) the installation, alteration or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse. The proposed single storey extension would not extend beyond a wall forming the side elevation of the dwellinghouse or consist of or includes any of the above criteria. A.2 In the case of a dwellinghouse on article 2(3) land, development is not permitted by Class A if— (a) it would consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles; (b) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse; or (c) the enlarged part of the dwellinghouse would have more than a single storey and extend beyond the rear wall of the original dwellinghouse. The application site is sited on article 2(3) land, and would be clad with render on the exterior walls. Part A.3 states that ‘the materials used in any exterior work must be of similar appearance to those used in the construction of the exterior of the original dwellinghouse’, and as such would be considered acceptable. Previous plans were submitted which extended beyond the side elevation of the original dwelling, and equalled to 6.3m in width which is over half the original width of the original dwelling. This was therefore an unlawful development. Amended plans were received on 10/01/2022 which reduced the length of the development to 4m from 6.3m, and altered the roof from a flat roof with rooflight to a lean-to roof. These amendments made the development lawful. Adjoining neighbours have been informed of the proposal and no objections have been received from the occupier of the adjacent properties. On the basis of the above, Certificate of Lawfulness is granted for the proposed development. |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That consent for a Certificate of Lawfulness be granted. |  |  |  |  |  |  |  |  |  |