| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | BT |  |  |  | Date: | 24/2/2022 | Manager: |  |  |  | Date: |  |
| Site Notice displayed | N/A | Photos uploaded |  |  |  | Y |  |  |  |  |  |  |  |
| Application Ref: |  |  | 3/2022/0034 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  | 25/11/2021 |  |  |  |  |  |  |  |  |  |  |
| Officer: |  |  | BT |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  | Decision |  | Approval |  |  |
| Development Description: |  |  |  |  | Side extension to form garage with utility at ground floor and two bedrooms and ensuite facilities at first floor. Resubmission of 3/2021/1056. |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 2 Balmoral Avenue, Clitheroe. BB7 2QH |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |
| Clitheroe Town Council consulted on 24/1/22 – no response. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |
| LCC Highways: No objections. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |
| None. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1 – Development Strategy Key Statement DS2 – Presumption in Favour of Sustainable Development Policy DMG1 – General Considerations Policy DMG2 – Strategic Considerations Policy DMG3 – Transport And Mobility Policy DMH5 – Residential and Curtilage Extensions NPPF |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2021/1056: Proposed two storey extension to form garage with utility at ground floor and two bedrooms and en-suite facilities at first floor. (Refused) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a semi-detached property in Clitheroe which occupies a corner plot on the junction between Balmoral Avenue and Conway Avenue. The property consists of brick, stone, render, concrete roof tiles and UPVC doors and windows. The surrounding area is residential comprising numerous semi-detached two storey dwellings. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for the construction of a two storey side extension and an extension to an existing rear single storey extension. The application is a resubmission of a previously refused planning application which was refused planning consent on the grounds of adverse impacts upon visual amenity. The design of the current proposal has been revised to lower the roof pitch of the proposed two storey side extension in addition to setting its front elevation back from the front elevation of the main property. A split level design has also been incorporated to the Southern side elevation of the two storey extension. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of development: The proposal is a domestic extension to a dwelling and is acceptable in principle subject to an assessment of the material planning considerations. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Residential Amenity: The extension would incorporate a first floor window on its front Eastern elevation in addition to ground and first floor windows on its rear Western elevation. The proposed windows would be sited in a similar position to the property’s existing front and rear ground and first floor windows which respectively look towards the road junction between Balmoral and Conway Avenue and the rear of properties on Henthorn Road and Balmoral Avenue. Accordingly, it is not considered that the proposed works would compromise the privacy of any neighbouring residents. The Southern side elevation of the extension would be sited close to the common boundary shared with No. 4 Balmoral Avenue however the extension would be sited to the North-east of No. 4 with its side elevation off-set to the side elevation of No. 4 which does not contain windows forming part of any habitable rooms. As such, it is not considered that the proposal would lead to any loss of natural light or outlook for any neighbouring residents. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity: The front elevation and roof line of the two storey side extension would be set back and below the main property respectively with a split level incorporated to its Southern side elevation. The proposed rear extension would be adjoined to the eaves, roof pitch and rear elevation of the property’s existing single storey rear extension. Accordingly, the proposed extensions would read as subservient additions to the existing property. The two storey element of the proposal would comprise a sizeable footprint and would be publicly visible due to the main property occupying a corner plot within the surrounding pattern of development. As such, the proposal would have a noticeable visual impact however given the prevalence of similar looking two storey side extensions within the immediate pattern of housing it is not considered that the proposal would be an incongruous or over dominant addition to the existing property and street scene. Furthermore, the two storey extension would incorporate render and brick on its side and front elevations respectively which would be in keeping with the external materials of the existing property and neighbouring properties in the immediate area. Accordingly, it is not considered that the proposal would have any adverse impact upon the visual amenities of the immediate or surrounding area. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: A bat survey carried out at the property on 2/11/21 found no evidence of any bat related activity. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways: LCC Highways have reviewed the proposal and have no issues with the proposed works therefore it is not considered that the proposal would have any undue impact highway safety. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: It is not considered that the proposal would have any undue impact upon residential amenity for the neighbouring residents, nor is it considered that the proposal would be harmful to the visual amenities of the immediate or surrounding area. It is for the above reasons and having regard to all material considerations and matters raised that the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  | That planning permission be granted. |  |  |  |  |  |  |  |  |  |