| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | SK |  |  |  | Date: | 24.1.23 | Manager: |  | LH | Date: | 26.1.23 |
| Application Ref: |  |  |  | 2022/0869 |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 22.11.22 |  |  |  |  |  |  |  |  |
| Officer: |  |  |  | SK |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Proposed demolition of an existing single storey rear outrigger and construction of a new two storey side and rear extension, with single storey front extension. |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 23 Clitheroe Road Whalley Lancashire BB7 9AA |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |
| No representations received in respect of the application. |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |
| No representations received in respect of the application. |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Policy DMG1 – General Considerations Policy DMH5 – Residential and Curtilage Extensions National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: No recent planning history directly relevant to the determination of the application. |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to an existing semi-detached dwelling fronting Clitheroe Road, Whalley. The property occupies a corner plot location with the side (north) elevation of the property also fronting Hayhurst Drive. The dwelling is faced primarily in render with parking provision provide to the rear via a ‘side-driveway’ located off Hayhurst Road. The application site is located within the defined settlement limits of Whalley being located in a predominantly residential area characterised by dwellings of varying appearance and typologies including terraced properties, bungalows and two-storey semi-detached dwellings. |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: The submitted details seek consent for the erection of a two-storey side and rear extension including the demolition of an existing single-storey rear extension located on the west facing elevation of the dwelling. It is proposed that the side extension will project sideward by approximately 3.6m, benefitting from an eaves hight that matches that of the existing dwelling with an apex height set below that of the existing to ensure visual subservience. The two-storey rear extension is located on the northern extents of the west-facing rear-elevation with a rearward projection of approximately 4.1m. The extension benefits from a gabled appearance, being faced in render to match that of the existing dwelling. The side extension will also benefit from a slight froward project, at ground floor, forward of the principal elevation of the dwelling, it is proposed that this will accommodate a forward projecting mono-pitch roof that will extend across the principal elevation of the dwelling, interfacing with and forming a new roof over an existing bay window to form an arrangement to match that on the adjoining property. |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact Upon Residential Amenity: Given the rearward projection and scale of the proposed two-storey rear extension consideration must be given in respect of the potential for the extension to result in detrimental impacts upon the residential amenities of ‘Garden Cottage’ to the west. In this respect the western extents of extension is located sited a sufficient distance (approximately 12.6m) from the side elevation of the aforementioned neighbouring dwelling to ensure no overbearing impact. It is further noted that the west-facing elevation of the extension proposes a window at first floor, however it is proposed that this window will be obscure glazed, as such the proposal will not result in any elevated direct-overlooking into the private amenity space of ‘Garden Cottage’. Taking account of the relationship of the proposed extension(s) with that of the neighbouring adjoining property (number 21), the rear extension is sited a sufficient distance from the neighbouring shared boundary to ensure no adverse impacts. As such and taking account of the above it is not considered that the proposed development will result in any adverse measurable or quantifiable impacts upon adjacent or nearby residential amenities. |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity/External Appearance: The proposed alterations save that for the two-storey rear extension match that of the arrangement of the neighbouring adjoining property and as such will not be read as being incongruous or anomalous. However, given that the existing property occupies a corner-plot location consideration must be given in respect of the visual compatibility of the side extension with the existing streetscene. The sideward projection of the extension both respects and is of a lesser projection of that of the building line established by ‘Garden Cottage’ fronting Hayhurst Road. In this respect the extension will not result in establishing an anomalous pattern of development. It is further considered that the side elevation of the side extension, fronting Hayhurst Road, benefits from sufficient elevational animation to successfully and positively address the streetscene. Taking the above into account it is not considered that the proposed extension(s) or alterations will result in any measurable harm to the character or visual amenities of the area or immediate streetscene. |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: The application has been accompanied by a Preliminary Bat Roost Assessment (30.09.2022), the report concludes that there was no evidence of the building having been used for the purposes of roosting by bats and that the building offers negligible potential for roosting bats. As such there is no requirement to provide mitigation to offset the impacts of the development upon protected species or species of conservation concern. |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: As such and for the above reasons, having regard to all material considerations and matters raised, that the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That planning consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |