| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | SK |  |  |  | Date: |  | 08.03.23 |  | Manager: |  | LH | Date: | 14/3/23 |
| Application Ref: |  |  |  | 2022/0982 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 10/11/22 |  |  | Site Notice: |  | 10/11/22 |  |  |  |  |  |
| Officer: |  |  |  | SK |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Proposed demolition of existing derelict toilet block and erection of a new single storey dwelling. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | Former Public Toilets Market Place Longridge PR3 3RQ |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| No representations received in respect of the proposal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The Local Highways Authority (LHA) have offered the following observations: The submitted documents and plan 'Proposed site plan' JM-0254-003-Rev A have been reviewed and the following comments are made. Proposal The application seeks to demolish the former public toilet block and erect a one-bedroom dwelling with a new vehicle crossing on Market Place to serve a single car parking space. Access and parking The site plan proposes a block paved car parking space measuring 6.4m long by 2.6m wide. The size prevents the vehicle from being able to turn and exit onto Market Place in forward gear. The paving is bound and porous and is considered acceptable. This will require a new vehicle crossing on Market Place B5269 which is subject to a 30mph speed limit. There is a surface water road gully along the frontage of the site. The vehicle crossing causes conflict with the adjacent road junction and bus stop, where the kerb tapers across the development site. The single parking space will require a reversing manoeuvre on the adopted highway which is a classified road carrying a significant volume of traffic of approximately 8000 vehicles a day, with a high percentage of heavy vehicles of approximately 8%. The Highway Authority would object to the new vehicle crossing required for the car parking space at this location due to conflict with highway users on a busy section of classified road. Should the application be considered for approval we would require the off-street parking bay, accessed from Market Place, removed from the scheme and a secure, cycle store being provided. The site is located within walking distance of mainline bus services and local facilities and could support residents without a car. However, we would seek to secure a car parking place and electric vehicle charging point at the dwelling with a potential alternative point of access on King Street, which is adopted highway. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cadent Gas |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cadent Gas have identified that the proposal is in close proximity to medium and low pressure assets. As such they have recommended the inclusion of the following informative: Cadent Gas Ltd own and operate the gas infrastructure within the area of your development. There may be a legal interest (easements and other rights) in the land that restrict activity in proximity to Cadent assets in private land. The applicant must ensure that the proposed works do not infringe on legal rights of access and or restrictive covenants that exist. If buildings or structures are proposed directly above the apparatus the development may only take place following diversion of the apparatus. The applicant should apply online to have apparatus diverted in advance of any works, by visiting cadentgas.com/diversions Prior to carrying out works, including the construction of access points, please register on www.linesearchbeforeudig.co.uk to submit details of the planned works for review, ensuring requirements are adhered to. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| United Utilities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No objections in respect of the proposal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| One letter of representation has been received raising the following concerns: - The site should be a car-park - Inadequate parking in the area |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1 – Development Strategy Key Statement DS2 – Sustainable Development Key Statement DMI2 – Transport Considerations Key Statement EN5 – Heritage Assets Policy DMG1 – General Considerations Policy DMG2 – Strategic Considerations Policy DMG3 – Transport & Mobility Policy DME1 – Protecting Trees and Woodland Policy DME2 – Landscape and Townscape Protection Policy DME3 – Site and Species Protection and Conservation Policy DME4 – Protecting Heritage Assets Longridge Neighbourhood Plan National Planning Policy Framework (NPPF) Section 66 and 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990 Longridge Conservation Area Appraisal Longridge Conservation Area Management Guidance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 2022/0983: Application for the demolition of an unlisted building (Former Public Toilets) within a conservation area. (Live application) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to the former Public Toilets located off King Street, Longridge. The existing building is a single storey stone-faced pitched roof building of a simple utilitarian appearance with an open frontage to the public realm. The immediate area consists of both a mixture of residential and commercial properties, with the immediate adjacent neighbouring properties to the north and south being residential. The site is located within the designated Longridge Conservation Area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: The application seeks consent for the demolition of the existing building and the erection of a one-bedroomed bungalow style dwelling. It is proposed that the dwelling will benefit from an ‘L’ shaped plan-form, being faced in a mixture of natural coursed stone and white render with quoin detailing. It is further proposed that the roof will be faced in natural slate with the submitted drawings also detailing the inclusion of ‘recessed photovoltaic panels’ on the south facing roof-plane. The amended (following highways comments) proposed site plan proposes that the dwelling will maintain its open frontage to King Street to the south, with the southern boundary wall being rebuilt at a height of 900mm and the northern boundary will be retained and replaced to match the existing configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: The application seeks consent for the creation of a new residential planning unit, as such consideration must be given in respect of the proposal compliance with the adopted development strategy for the borough. The application site lies within the defined settlement limits of Longridge, in this respect the principle of the development of the site for residential purposes, notwithstanding other development management considerations, would be in broad compliance with Policy DMG2 which states that ‘development proposals in the principal settlements of Clitheroe, Longridge and Whalley and the tier 1 villages should consolidate, expand or round-off development so that it is closely related to the main built-up areas, ensuring this is appropriate to the scale of, and in keeping with, the existing settlement’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon Character/appearance of Conservations Area: The application site is located within the defined Longridge Conservation Area, as such Key Statement EN5 and Policy DME4 are primarily, but not solely, engaged for the purposes of assessing the proposal. In this respect Key Statement EN5 states that: There will be a presumption in favour of the conservation and enhancement of the significance of heritage assets and their settings. The Historic Environment and its Heritage Assets and their settings will be conserved and enhanced in a manner appropriate to their significance for their heritage value; their important contribution to local character, distinctiveness and sense of place; and to wider social, cultural and environmental benefits. This will be achieved through: - Recognising that the best way of ensuring the long term protection of heritage assets is to ensure a viable use that optimises opportunities for sustaining and enhancing its significance. - Keeping Conservation Area Appraisals under review to ensure that any development proposals respect and safeguard the character, appearance and significance of the area. - Considering any development proposals which may impact on a heritage asset or their setting through seeking benefits that conserve and enhance their significance and avoids any substantial harm to the heritage asset. - Requiring all development proposals to make a positive contribution to local distinctiveness/sense of place. - The consideration of Article 4 Directions to restrict permitted development rights where the exercise of such rights would harm the historic environment. With Policy DME4 stating, in respect of development within conservation areas or those affecting the listed buildings or their setting, that development will be assessed on the following basis: 1: CONSERVATION AREAS Proposals within, or affecting views into and out of, or affecting the setting of a conservation area will be required to conserve and where appropriate enhance its character and appearance and those elements which contribute towards its significance. This should include considerations as to whether it conserves and enhances the special architectural and historic character of the area as set out in the relevant conservation area appraisal. development which makes a positive contribution and conserves and enhances the character, appearance and significance of the area in terms of its location, scale, size, design and materials and existing buildings, structures, trees and open spaces will be supported. In the conservation areas there will be a presumption in favour of the conservation and enhancement of elements that make a positive contribution to the character or appearance of the conservation area. Policy DMG1 is also engaged in concert with Key Statement EN5 and Policy DME4 insofar that the policy sets out general Development Management considerations, with the policy having a number of inherent criterion that are relevant to the assessment of the current proposal, which state: In determining planning applications, all development must: DESIGN - Be of a high standard of building design which considers the 8 building in context principles (from the CABE/English Heritage building on context toolkit. - Be sympathetic to existing and proposed land uses in terms of its size, intensity and nature as well as scale, massing, style, features and building materials. - Consider the density, layout and relationship between buildings, which is of major importance. particular emphasis will be placed on visual appearance and the relationship to surroundings, including impact on landscape character, as well as the effects of development on existing amenities. AMENITY - Not adversely affect the amenities of the surrounding area. ENVIRONMENT - All development must protect and enhance heritage assets and their settings. In respect of the above, the proposed building is of a typical domestic appearance benefitting from a hipped roof arrangement, being faced in natural stone in render. The overall architectural language and site configuration of the dwelling is relatively simple in terms of overall articulation and detailing, in this respect it is not considered that the proposed building will be read as being anomalous or discordant when taking account of the immediate streetscene or the inherent character of the Longridge Conservation Area. The proposed building footprint represents an enlargement compared to that of the existing building footprint with a further projection forward at the buildings southern extents by virtue of a projecting gabled element. Whilst it is accepted that this will afford this element of the building a higher level of visual prominence than that of the existing structure, it is not considered that this enhanced prominence will be of detriment to the character of the area, particularly given the immediate irregular building line on the northern side of Kings Street. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact Upon Residential Amenity: It is noted that the rear of the proposed dwelling will have a direct interface with the side garden area associated with number 27 Dixon Road, as such consideration must be given in respect of the potential for the proposal to have undue impacts upon neighbouring residential amenities. Taking account of the single-storey nature of the dwelling, the orientation and location of the north-west facing habitable room windows and the presence of a high-level existing boundary wall between the proposed building and the garden area of number 72, it is not considered that there will be significant detrimental impacts upon the residential amenities of the occupiers of number 72, nor is it considered that the sense of privacy currently afforded to the existing garden area will be significantly diminished. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways and Parking: In respect of the proposal, it is noted that the LHA have stated that they ‘would object to the new vehicle crossing required for the car parking space at this location due to conflict with highway users on a busy section of classified road’ and that ‘should the application be considered for approval we would require the off-street parking bay, accessed from Market Place, removed from the scheme and a secure, cycle store being provided’ further stating that ‘the site is located within walking distance of mainline bus services and local facilities and could support residents without a car’. In light of the above observations the applicant has removed the on-site parking provision from the proposal. It is however noted that the LHA have stated further to the above that ‘however, we would seek to secure a car parking place and electric vehicle charging point at the dwelling with a potential alternative point of access on King Street, which is adopted highway’. On balance, and taking account that the LHA have acknowledged that the site is located within walking distance of services and local facilities - therefore being a sustainable location that could support residents without a car, it is not considered that the lack of dedicated parking provision would warrant the refusal to grant planning permission on these grounds alone nor is it considered that the proposal will result in any detrimental impacts upon the safe operation of the immediate highways network. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: The application has been accompanied by both a Preliminary Bat Roost Assessment and Arboricultural Impact Assessment (with associated method statement). The Preliminary Bat Roost Assessment concludes that no evidence was recorded to suggest bats were roosting within the building, that no bats were observed or recorded using the building for roosting and that the building is considered to be of negligible potential for roosting bats. As such no mitigation is required to offset the impact of the development upon protected species or species of conservation concern. The submitted Arboricultural Impact Assessment identifies three trees on site (2 x Birch and 1 x Willow) and a grouping (G4) of Rowan x 2. The Impact Assessment proposes the removal of T3 (Willow) and G4 which cannot be retained in the context of the development. The submitted Method Statement proposes a methodology for the undertaking of works to ensure the protection and retention of T1 and T2 during the construction phase of the development including long-term retention which are considered acceptable. As such, should consent be granted, a condition will be imposed requiring that the works be undertaken in strict accordance with the submitted methodology. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Matters: Given the proposal is located within the plan area of the Longridge Neighbourhood Development Plan consideration must be given in respect of the proposals compliance with the policies contained within the plan. In this respect, Policies LNDP3 (Longridge Design Principles) and LNDP4 (Conserving and Enhancing Our Local Heritage) are of particular relevance. LNDP3 requires that: All new development proposals will only be supported when they are of good design that responds positively to the local character and distinctiveness of the surroundings. Proposals should: - Conserve and enhance the locally distinctive built, historic and natural environment; - Are designed to take account of site characteristics and surroundings, including: - Layout – the predominantly green appearance of the area to be maintained with appropriate green space and planting of trees and shrubs; - Siting; - Scale; - Height; - Proportions and massing; - Fuel efficiency; - Architectural detailing; - Landscaping; - Materials; and - For domestic extensions these should be designed to appear an integral part of the original design of the house. - Have no significant adverse impact on residential amenity for existing and future residents; - Do not contribute to, or suffer from, adverse impacts arising from noise, light or air contamination, land instability or cause ground water pollution; - Utilise sustainable construction methods,minimising the use of non-renewable resources and maximising the use of recycled and sustainably sourced materials; - Minimise resource use towards a zero carbon dioxide emissions; - Provide easy access for all members of the community; - Create safe environments that minimise opportunities for crime; - Incorporate adaptable designs that can accommodate changing lifestyles/life stages and technologies; and - they positively address any other design issues that have been identified as having a significant adverse impact on local character and distinctiveness With LNDP4 stating that ‘all of the town’s heritage assets, including the three Conservation Areas, will be conserved in a manner appropriate to their significance’. Taking account of the direct synergy of the LNDP policies and those inherent to the Ribble Valley Core Strategy and taking account that the proposal is considered to be in broad compliance with those adopted development plan polices, it is not considered that the proposal raises or results in any significant direct measurable conflict with the Longridge Neighbourhood Development Plan. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: As such and for the above reasons, having regard to all material considerations and matters raised, the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| That planning consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |