| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | KH |  |  |  | Date: |  | 27/02/23 |  | Manager: |  | LH | Date: | 28/2/23 |
| Application Ref: |  |  |  | 2022/1037 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 05/01/23 |  |  | Site Notice: |  | 05/01/23 |  |  |  |  |  |
| Officer: |  |  |  | KH |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Listed Building Consent to refurbish existing single glazed metal and timber windows located within the existing main building to the front and rear elevations with new double glazed timber and PVCu windows, excluding the 1960’s extension located to the rear. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | Brennands Endowed Primary School Church Street Slaidburn BB7 3ER |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| No response. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1 – Development Strategy Key Statement DS2 – Sustainable Development Key Statement EN5 – Heritage Assets Policy DMG1 – General Considerations Policy DMG2 – Strategic Considerations Policy EN2 – Landscape & Townscape Protection Policy DME4 – Protecting Heritage Assets Planning (Listed Buildings and Conservation Areas) Act National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 2017/0709 – LBC removal of existing hardwood and replacement with stained timber double doors. Existing concrete slabs to be used on the external access – Approved. 3/2014/0246 - Formation of new main office adjacent to the main entrance at the location of previously refurbished staff toilets and creation of new Head Teacher’s Office and Chaplaincy. LBC application withdrawn 12 May 2014. 3/2013/0409 – Extension to rear of school to form a Sheltered Play Area. Observations. 3/2012/0624 – Proposed works comprise the erection of a permanent canopy to the rear of the main school. Observations. 3/2012/0352 – Proposed works comprise the erection of a permanent canopy to the rear of the main school. Observations. 3/2005/0672 – Proposed single storey extension to allow internal re-modelling to provide disabled toilet facilities and medical room and resource area. LBC granted 12 October 2005. 3/2005/0670 – Single storey extension to existing classroom together with the creation of a new Medical Room, Resource Area and WC Facilities including Disabled Toilet. Observations. 3/2003/0633 - Removal of roof coverings to provide additional timber roof structure. LBC granted 27 August 2003. 3/2001/0571 - Engineering works to create a new play area. PP granted 12 September 2001. 3/2000/0810 - New ventilation louvres to roof space (2 no). LBC granted 18 January 2001. 3/20000/0578 - Provision of concrete shed. PP granted 10 October 2000. 3/1998/0788 - Construct mezzanine floor, erect flat roof on side elevation and other adaptations. LBC granted 30 March 1999. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: Brennand’s Endowed School is a Grade II* listed school of the early 18th Century prominently sited within Slaidburn Conservation Area and the Forest of Bowland Area of Outstanding Natural Beauty. The school is within the setting of other listed buildings including the adjoining Grade I listed St Andrew’s Church. The list description of the school refers to the historic build only. The Slaidburn Conservation Area Appraisal (The Conservation Studio consultants, 2005) identifies: “Two particularly fine ensembles of historic buildings: St Andrew’s Church, Brennand’s School and Church Stile Farm in Church Street”. “St Andrew’s Church stands in a large, square well-stocked graveyard that has recently been extended to the south. The churchyard contains a sundial and base dated 1796, set into a base of four circular steps. Brennand’s Endowed School, adjacent to the church, is set well back from the road with a grassy playground behind. The space around these two buildings is vital to their setting, enabling uninterrupted views of both”. “Brennand’s Endowed School (grade II*) was endowed in 1717 by John Brennand of Pain Hill Farm. Over the hall door (the former main entrance) is the following inscription: "This Grammar School was erected and endowed by JOHN BRENNAND late of Panehill in this Parish. Gentleman who died the 15th day of May in the year of our Lord 1717". The building is a seven-bay two-storeyed building with a central doorway with a broken pediment on fluted pilasters. It was described by the architectural historian, Nikolaus Pevsner, as “a very pleasant building, orderly and not without substance”. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: The replacement and refurbishment of existing windows. The existing metal windows to the front elevation will be refurbished and repaired with excess paint removed, new putty pointing to the glazing, the obscure glazing will be removed within the frames and replaced with conservation glazing and frames resealed to prevent future water damage to the frames. The timber windows to the rear of the main building will be replaced with new double glazed timber casements in a similar style within the stone mullions. The three narrow windows in the modern toilet block to the side will be replaced using uPVC casement style frames with a built-in extractor fan located to the top to provide ventilation to the toilets. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: The replacement and refurbishment of existing windows and replacement of some of the modern windows in the modern block are acceptable in principle subject to appropriate details/materials and accordance with the Planning (Listed Building and Conservation Areas) Act 1990, local and national planning policies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon Listed Building(s) and Setting: The principle statutory duty under the Planning (Listed Building and Conservation Areas) Act 1990 is to preserve the special character of heritage assets, including their setting. Local Planning Authority’s should when coming to decisions consider the principal Act, which states the following: Listed Buildings - Section 66(1) In considering whether to grant planning permission for development which affects a listed building or its setting, the local planning authority shall have special regard to the desirability of preserving the building or its setting or any features of special architectural or historic interest which it possesses. Brennands Endowed Primary School is Grade II* Listed and was listed in 1954 prior to the erection of a large single storey extension to the rear/side. In determining planning applications LPA’s should take account of; - The desirability of sustaining and enhancing the significance of heritage assets and putting them to viable uses consistent with their conservation; - The positive contribution that conservation of heritage assets can make to sustainable communities including their economic vitality; and - The desirability of new development making a positive contribution to local character and distinctiveness. Para 199 of the National Planning Policy Framework states that when considering the impact of proposals on the significance of a designated heritage asset, great weight should be given to the asset's conservation. The more important the asset, the greater the weight should be applied. This is irrespective of whether any harm is identified as being substantial, total loss or less than substantial harm to its significance. The key heritage issue is whether any of the proposed works would harm the character of the significance of the listed building. The significance of the school building lies in its early C18, 2 storey, 7 bay, 2 storey range which includes a small rear wing. The windows within the school, especially those on the front elevation are of particular note and contribute to the character and importance of the Grade 2* listed building. It was noted that the windows to the rear of the original block have all been altered and are relatively modern single glazed casements of differing styles. The attached toilet block to the east side is part of the 60’s extensions and is of no particular interest. The proposed work, outlined in the submitted Schedule of Works and described in the Heritage Statement is acceptable. It is noted that the work to the main windows on the front elevation is largely repair works to enable the windows to be properly used. This repair work is to be welcomed as it will help sustain the existing windows. The only issue arising being the need to replace some of the existing glass. The schedule indicating that ‘restoration’ glass would be used. It would be prudent to ask for a sample of any new period style glass to be agreed which can be achieved with a suitable Condition. There are no objections to the treatment of the WC windows in the modern addition on the left hand side of the principle Listed Building. Whilst these windows are viewed in the same context as the front elevation of the C18 school building, they are of a functional and different design and of no value/significance. The proposed change to a one piece uPVC design incorporating the fan unit will not harm the significance of the Listed Building. Similarly, no objection to the proposed new windows on the rear/side elevation. The timber casement design replicates the existing window pattern and will consolidate the somewhat miss-matched designs already present. The existing timber windows have already got quite thick framing/casements and the use of double glazed units in this context will have little or no impact on the aesthetics of the school building and not result in any loss of significance. The only issue on the submitted detail is that no confirmation is given regarding the treatment of the glazing bars, which divide the lower panes. A more traditional approach should be taken to the construction of the casements and the windows should be through glazed with the glazing bars forming an integral part of the windows. It is not considered that the surface applied glazing bars are appropriate. If this is what is intended then it would be preferable for no glazing bar to be applied. Whilst it is not expanded upon in the submission documents the changes are also likely help with the thermal performance/comfort levels of the school. Under the Act, the test to be applied is whether the works preserve the listed building and would cause no harm. Subject to details on the glass to be used for the re-glazing and the treatment of the glazing bars this requirement will have been met. The school building is set back from the road and forms part of an attractive group made up of St Andrews Church, Church Stile Farmhouse and No 33 Church Street. The proposed refurbishment/repair of the metal windows on the front elevation and replacement windows elsewhere will result in no change to the setting and would have neutral impact. The impact of the proposed works would result in less than substantial harm and therefore must be assessed against the public benefits. There would be public benefits in terms of improvements to the fenestration and allowing the windows to function as well as enhancing and preserving the existing windows and appropriate replacement glazing. This is acceptable and accords with local and national policy as well as primary legislation in this regard subject to appropriate conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon Character/appearance of Conservations Area: Slaidburn Conservation Area covers the historic core of the settlement. Similar, to the above the works would not harm how the conservation area is viewed and whilst it is noted that the use of uPVC is very limited in the conservation area because of the set-back location, the tall slender uPVC toilet block windows will not be unduly noticeable or harmful change. The use of uPVC, for these three windows, does not necessarily compromise the approach within the CA to retain multi-pane timber sashes as these windows relate to a building of modern construction and do not contribute to the areas character. The integrated design, to incorporate the fans, is a visually acceptable solution. On this basis the proposed works would not harm either the character or appearance of the Slaidburn Conservation Area. The impact upon the conservation area would be limited as the works is, in the main, refurbishment and replacement of existing modern alterations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact Upon Residential Amenity: The nearest properties are St. Andrew’s Church to the immediate south which is Grade I Listed, 33 and 31 Church Street to the north which are Grade II Listed and Church Stile Farmhouse to the west which is also Grade II Listed. The proposed works would not result in any impact on the nearest residential properties which are designated assets. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity/External Appearance: There are public views into the site from Church Street to the west and Public Footpath no. 14 runs along the sit boundary to the south. There would be no undue impact on the visual amenity or external appearance of the building. The works proposed will enhance the building in terms of the refurbishment and replacement of modern additions. These small-scale changes would not result in any undue harm to visual amenity and would enhance and preserve the fenestration of the school for an extended period of time. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways and Parking: The proposal would not result in any impact on highways or raise any parking issues. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: The proposal would not result in any impact on landscaping or ecology. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Matters: As I am required to do so, I have given the duty imposed by s.66(1) and s.72(1) of the P(LBCA) Act 1990 considerable weight in my comments. Subject to confirmation of suitable glass for the refurbishment of the existing windows and the treatment of the glazing bars on the windows to the rear/side I consider that the proposal would meet the statutory test ‘to preserve’ and would not cause any discernible harm to the significance of the Grade II* listed building. The proposal would also cause no harm to the contribution made by the building to the significance of the adjacent Listed buildings or to the character or appearance of the Slaidburn Conservation Area. As such, the proposal would meet the requirements of Chapter 16 of the NPPF and would comply with Policy EN5 and Policy DME4 of the Local Plan. Therefore, in giving considerable importance and weight to the duty at section 16 of the Planning (Listed Buildings and Conservation Areas) Act 1990, in giving ‘great weight’ to the conservation of the designated heritage asset (NPPF paragraph 132), in consideration to NPPF paragraph 17 (conserve heritage assets in a manner appropriate to their significance), paragraph 60 (reinforce local distinctiveness) and paragraph 131 (development sustaining and enhancing the significance of heritage assets and positively contributing to local character and distinctiveness) and in consideration to Key Statement EN5 and Policies DME4 and DMG1 of the Ribble Valley Core Strategy I recommend that listed building consent be granted subject to appropriate conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: As such and for the above reasons, having regard to all material considerations and matters raised, that listed building consent be granted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| That planning consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |