| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | LW |  |  |  | Date: |  | 12/06/2023 |  | Manager: |  | KH | Date: | 12/06/23 |
| Application Ref: |  |  |  | 3/2023/0179 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 04/05/2023 |  |  | Site Notice: |  | 04/05/2023 |  |  |  |  |  |
| Officer: |  |  |  | LW |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Proposed two storey extension to (north) side elevation, single storey extension to (south) side elevation and conservatory to (east) rear elevation. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 4 Branch Road, Mellor Brook, Lancashire, BB2 7NU |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| No comments received. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | No objections. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| One letter of representation has been received in regard to the proposal. The concerns addressed within the letter relate to the potential loss of daylight as a result of the proposed two storey side extension. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable Development Key Statement DMI2: Transport Considerations Policy DMG1: General Considerations Policy DMG2: Strategic Considerations Policy DMG3: Transport & Mobility National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2012/0260: Raising of rear eaves level and erection of first floor extension over part of existing single storey flat roof extension – resubmission (Approved) 3/2011/0725: Proposed first floor bedroom over the existing ground floor extension (Refused and appeal dismissed) 3/2003/0453: Change of use of former newsagent/ domestic property back to solely domestic use only (Approved) 3/1992/0405: Internally illuminated double sided projecting sign (Approved) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a detached two-storey property at no.4 Branch Road, situated close to the junction with Preston New Road. The property features an existing single storey and first floor rear extension, along with an integral garage to the northern side elevation of the dwelling which has been converted into an office/utility area. The site is located within the defined settlement boundary of Mellor Brook, with the rear garden area of the property backing on to land designated as Green Belt where a Public Right of Way (FP80) allows public views of the site. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: The proposal seeks consent for a proposed two storey side extension, single storey side extension and conservatory to the rear. The proposed two storey side extension would project 4.65m from the north facing side elevation of the main dwellinghouse, replacing the existing office/utility. The proposed development would extend a depth of approximately 7.2m to align with the rear elevation of the original dwellinghouse and would feature a gable ended roof design which would measure 5.1m to the eaves and 7.1m to the ridge. At ground floor, a set of double doors would be featured to the front elevation to serve the proposed office, whilst at first floor 2no. windows would be featured to the front elevation and 1no. window to the rear elevation to serve the proposed bedroom. 1no. small window would also be installed to the first-floor gable elevation to serve the proposed en-suite. As part of the proposal, the existing flat roof single storey rear extension would also be extended by approximately 0.8m in order to align with the northern elevation of the proposed two storey addition. The proposed single storey side extension to the south facing elevation would serve as a utility, projecting 2.23m from the side elevation of the main dwellinghouse and extending a depth of approximately 7m. The proposed extension would feature a flat roof design measuring 2.9m in height and would include the installation of a single door to the front and rear elevation. The proposed conservatory would project 2.7m from the rear elevation of the existing rear extension. The proposal would extend a width of 6.9m and feature a lean-to roof measuring 2.3m to the eaves and 3m to the ridge. To the south facing side elevation of the development, a set of double doors would be installed, along with glazing to all elevations. In regard to materials, the proposed development would be constructed to match the external appearance of the existing dwellinghouse, including matching red facing brickwork, natural blue slate roof tiles and dark grey uPVC windows and doors. To the front elevation, red clay roof tiles would also be featured to the ground floor bay, along with render to the ground floor rear elevation, to match the existing. The original proposal also included the addition of a flat roof first floor extension to either side of the existing hipped roof first floor extension, resulting in a first-floor addition which extended across the full width of the main dwellinghouse and the proposed two storey side extension. However, it was considered that the design and scale of the proposal would not reflect the character of the original dwelling or respect its proportions and form. As such, this element of the proposal has been removed from the scheme. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: The proposal relates to domestic alterations to a dwelling and is therefore acceptable in principle subject to an assessment of the material planning considerations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact Upon Residential Amenity: The proposed two storey extension to the property’s north facing side elevation would feature 2no. additional windows to the front elevation and 1no. additional window to the rear. Despite this, the views from the proposed front and rear windows would not directly face any neighbouring residential properties and would be similar to those afforded by the existing first floor windows. As such, no new opportunities for overlooking or loss of privacy would be created. The proposed two storey addition would also project 4.65m from the side elevation of the main dwellinghouse, bringing the application dwelling closer the southern side elevation of no.6 Branch Road. Whilst the proposal would remain over 4m from this neighbouring property, it is acknowledged that no.6 Branch Road features several existing windows within the side elevation directly adjacent the proposal. Nevertheless, these windows either serve a non-habitable space or are secondary windows and therefore are not the primary source of daylight. The proposed extension would also feature 1no. window within the first-floor gable elevation directly adjacent no.6 Branch Road. However, this would serve the proposed en-suite and a condition has been attached to ensure this window is obscurely glazed and non-opening in order to protect the amenity of the occupiers of no.6 Branch Road. In view of this, the proposed two storey side extension would not result in any significantly detrimental impact upon the amenity of no.6 Branch Road by way of overshadowing, loss of outlook, daylight or privacy that would warrant the refusal of the application. Furthermore, the relatively modest outward projection (2.25m) of the flat roof single storey side extension would not result in any significant degree of overshadowing or loss of outlook in regard to the neighbouring property of no.2 Branch Road. The proposal would be distanced over 5m from the north facing side elevation of no.2 Branch Road and would feature no windows directly facing this adjacent residential property. The proposed flat roof, single storey extension would also be adequately screened by the existing boundary treatment. As such, the proposed development would not result in any undue impact upon no.2 Branch Road that would warrant a refusal. With regards to the proposed conservatory to the rear of the property, the proposed development would not unduly impact on the two adjacent dwellings and would be set approximately 6m from the main dwellinghouse at no.6 Branch Road and over 8m from the common boundary with no.2 Branch Road. Whilst glazing is proposed to all elevations of the conservatory, the proposed development would be adequately screened by the existing boundary treatments and therefore would not result in any unacceptable impact upon the amenity of no.2 Branch Road or no.6 Branch Road. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity/External Appearance: The front of the proposed development would be publicly visible from the highway of Branch Road, whilst the rear would be visible from Preston New Road and the Public Right Of Way (FP80) which runs through the fields to the rear of the site. As such, the proposed development would have a visual impact upon the surrounding area. Despite this, the wider area is characterised by a variety of house types/designs which reflect the different times/periods that dwellings have been constructed in Mellor and more specifically along Branch Road and Preston New Road. It is therefore not considered that the proposed alterations would significantly detract from the surrounding area and the existing dwelling is also not one of any substantial visual merit. The proposed two storey and single storey side extensions would be set approximately 0.45m back from the principal elevation of the property and would be of a size and scale appropriate to that of the existing dwellinghouse. The roof pitch of the proposed two storey addition would also be set approximately 0.4m below that of the main dwellinghouse. As such, the proposed development would appear subservient to the main property and not an overbearing or incongruous addition to the host property or the wider locality. Furthermore, although the proposed conservatory would project 2.7m from the rear wall of the existing single storey rear extension, the proposal would feature a lean to roof with a maximum ridge height of 3m and the eaves falling to approximately 2.3m. The proposed conservatory would therefore appear clearly subordinate to the main dwellinghouse and appropriate in size and scale. The proposed development would also be constructed in materials to match the external appearance of the existing dwellinghouse, including matching red brickwork, slate roof tiles and dark grey uPVC windows and doors. White painted render would also be featured to the ground floor rear elevation of the proposal to match the existing single storey rear extension. The proposed materials are therefore acceptable, further reducing the visual impact of the proposal and ensuring visual integration with the existing property. The proposal, while prominent, is therefore not considered to result in a significant negative impact on visual amenity or character of the area, to warrant the refusal of the application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways and Parking: Lancashire County Council Highways have been consulted in regard to the proposal and have no objections. As such, the proposed development is acceptable in regard to highway safety and parking. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: A daytime bat survey was undertaken at the site, dated 4th April 2023. The survey concluded that no bats or evidence of bats were found during the daytime inspection and the property has negligible potential for night roosting and a low potential for day and hibernation roosting. Despite this, a cautionary approach is advised. As such, any removal or disturbance of potential roost features such as soffits and roof coverings should be done carefully by hand and in the event that any bats are discovered, disturbed or harmed during the development, all work must cease immediately, and further advice sought from a licenced ecologist. In regard to nesting birds, no birds’ nests or bird nesting material were found during the survey. However, it was concluded that there is a high potential for nesting birds to utilise the boundary vegetation including Leylandii trees, ivy and privet hedges during the bird nesting season. This vegetation is close enough to the proposed development site to be subject to disturbance during the proposed works. The proposal would also include the removal of 2no. trees within the rear garden area of the site, located towards the north-eastern corner of the dwellinghouse. Whilst these trees are not considered to be of significant ecological importance and there are no Tree Preservation Orders associated with the site, the submitted bat survey did identify these trees as a potential bird nesting area. As such, no construction work, including the removal of vegetation, should be carried out within of the bird nesting season (1st March – 31st August inclusive) unless a nesting bird survey carried out immediately prior to any works, by a licenced ecologist, confirms the absence of nesting birds. A letter from the ecologist confirming the absence of nesting birds should be submitted to the Council within one month of the survey being undertaken. It is also advised that if birds are found to be nesting within or in close proximity to the work area, all work must cease immediately, and an exclusion area established to prevent disturbance. The extent of the exclusion area should be determined by a licenced ecologist and works should only proceed upon confirmation from a licenced ecologist. Monitoring for nesting birds should continue for the duration of the works thereafter. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: As such, for the above reasons and having regard to all material considerations and matters raised, the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That planning consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |