| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | LW |  |  |  | Date: |  | 28/06/23 |  | Manager: |  | KH | Date: | 28/06/23 |
| Application Ref: |  |  |  | 3/2023/0275 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 25/05/23 |  |  | Site Notice: |  | N/A |  |  |  |  |  |
| Officer: |  |  |  | LW |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Demolition of existing outbuilding and construction of single storey WC/ utility rear extension. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 32 Gardeners Row, Sabden, BB7 9EB |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| No comments received within consultation period. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| No comments received within consultation period. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable Development Key Statement EN2: Landscape Key Statement EN5: Heritage Assets Policy DMG1: General Considerations Policy DMG2: Strategic Considerations Policy DME4: Protecting Heritage Assets Planning (Listed Buildings and Conservation Areas) Act National Planning Policy Framework (NPPF) Sabden Conservation Area Appraisal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: No planning history. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a mid-terrace two storey property at no.32 Gardeners Row, which lies within the defined settlement boundary of Sabden. The property is situated in a quiet residential area just off Padiham Road and Whalley Road, with the front elevation looking onto Sabden Park. The dwelling is comprised of stonework to the front and rear elevation, with slate roof tiles and white uPVC windows and doors. The property has undergone a number of alterations in its history, including the addition of a large single storey flat roof rear extension which projects approximately 3.6m from the rear elevation of the original dwellinghouse and is finished in render. A small, detached outbuilding is also featured towards the rear of the yard which measures approximately 2.3m by 1m and is proposed for demolition as part of the proposal. The application site is situated within the Forest of Bowland Area of Outstanding Natural Beauty and the Sabden Conservation Area. The properties along Gardeners Row are also identified as Buildings of Townscape Merit within the Sabden Conservation Area Appraisal, and the application property is therefore considered to be within a designated heritage asset. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for a single storey rear extension following demolition of the existing outbuilding. The proposed extension would project 2.56m from the rear elevation of the existing single storey rear extension, extending the remaining length of the rear yard, and have a width of 2.3m. A flat roof design would be featured which would measure 2.49m in height and to the north-western facing side elevation of the proposal, 1no. single door and 1no. window would be installed. The proposed development would be finished in render to match the existing rear extension. As part of the overall proposal, the window within the rear elevation of the existing extension, which serves the kitchen, would also be reduced in size in order to accommodate the proposed development. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: The proposal relates to a domestic extension to a dwelling and is therefore acceptable in principle subject to an assessment of the material planning considerations. The application site is situated within the Sabden Conservation Area and is identified as a Building of Townscape Merit; therefore, consideration will be given towards the effect of the proposal on the historic character of the host dwelling and the surrounding area. The site is also situated within the Forest of Bowland Area of Outstanding Natural Beauty and therefore additional consideration will also be given towards the effect of the proposal on the visual character of the surrounding landscape. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon Character/appearance of Conservations Area: With reference to making decisions on applications for development in Conservation Areas, Section 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990 states that: “… special attention shall be paid to the desirability of preserving or enhancing the character or appearance of that area.” The Sabden Conservation Area Appraisal (2006) states that its primary purpose is to record, analyse, preserve, and enhance the various features which give the Sabden Conservation Area its special architectural and historic interest. It identified the ‘continuing loss of original architectural details and use of inappropriate modern materials or details’ as being the primary threat to the conservation Area. With specific regard to external alterations, the Sabden Conservation Area Management Guidance (2006) stipulates that ‘alterations or repairs to external elevations should respect historic fabric and match it in materials, texture, quality and colour’. The proposed rear extension would feature a flat roof design and be finished in render which would not match the form and materials of the original dwellinghouse. This would usually be considered to have a negative impact upon a Conservation Area; however, the application property features an existing single storey, flat roof, rendered rear extension and this style of development is also well established within the surrounding area, with many other properties along both Gardeners Row and Padiham Road benefiting from similar additions. As such, when examined in relation to the existing dwellinghouse and wider street scene, the proposal would not appear an uncharacteristic or out of keeping addition. Furthermore, the original features to the front elevation, which affords the property its Townscape Merit, would not be impacted by the proposal with the development being directed to the rear of the property which has already undergone considerable change. The proposed demolition of the existing outbuilding would also have a negligible impact upon the character of the host property and the surrounding area, with the existing structure not being highly visible from the public realm or of any substantial visual merit. As such, it is not considered that the proposed development would detract from the existing form of the building or result in any significant detrimental impact upon the aesthetic character of the host property or the wider Sabden Conservation Area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact Upon Residential Amenity: The proposed extension would include the installation of 1no. small window and 1no. single door to the north-western facing side elevation. These additions would primarily provide views towards the rear yard of the application property and the existing single storey extension featured to the rear of no.30 Gardeners Row which extends the full length of the common boundary with the application site. This neighbouring extension features no windows within the side elevation facing towards the application property and therefore the proposed development would not result in any new opportunities for overlooking or loss of privacy. Furthermore, whilst the proposal would project 2.56m from the existing rear extension and 6.2m from the rear elevation of the original property, no.34 Gardeners Row also benefits from an existing single storey rear extension which projects approximately 3.4m from the rear elevation of the original built form. The existing extension at this neighbouring property is situated close the common boundary with the application site and features a set of double doors to the rear elevation. Despite this, it is not considered that the proposal would result in an unacceptable overshadowing effect or loss of outlook in relation to the above-mentioned double doors. The proposed development would be a relatively modest addition to the existing rear extension, projecting approximately 2.8m beyond the rear elevation of no.34 Gardeners Row and featuring a flat roof design measuring 2.49m in height at its maximum. The proposal would also replace the existing outbuilding and high boundary treatment which measures approximately 2m in height and would be situated to the north of no.34 Gardeners Row. As such, sufficient levels of daylight would still be received to the rear of this neighbouring property. In view of the above, the proposed extension would not result in any significant detrimental impact upon the amenity of no.34 Gardeners Row that would substantially exceed the existing and warrant the refusal of the application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity/External Appearance: Ribble Valley Core Strategy Policy DMG1 states that “development should be sympathetic to existing and proposed land uses in terms of its size, scale, intensity and nature”. Furthermore, emphasis is placed on visual appearance and the relationship to surroundings. As the application site lies within the Forest of Bowland Area of Outstanding Natural Beauty, consideration must be given to the effect of the proposal on the surrounding natural landscape. Key Statement EN2 of the Ribble Valley Core Strategy states that “the Council will expect development to be in keeping with the character of the landscape, reflecting local distinctiveness, vernacular style, scale, style, features and building materials.” The proposed rear extension would have an outward projection of 2.56m from the rear elevation of the existing single storey rear extension and a maximum height of 2.49m. As such, the height of the proposed development would be set below that of the existing rear extension, which measures approximately 2.7m, and would appear clearly subordinate to the primary dwelling and appropriate in size for a single storey rear extension. Despite this, the proposal would result in a rear addition which extends the full length of the rear yard and would incorporate a flat roof design which is usually considered to have a negative impact upon the AONB. However, the existing rear extension features a flat roof design, and this type of development is well established within the surrounding area with the majority of dwellings along Gardeners Row and Padiham Road already benefiting from similar additions. As such, when examined in relation to the wider street scene, the proposed extension would not appear an uncharacteristic, incongruous, or out of keeping addition to the host property. The proposal would also be finished in render to match the existing rear extension and would therefore be in accordance with Key Statement EN2 in as much that the proposal would not detract from or have any undue impact upon the character of the surrounding AONB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways and Parking: The proposed rear extension would not result in an increase in the number of bedrooms at the application site or include any alterations to the existing parking arrangements. As such, the proposed development would be acceptable in regard to highway safety and parking. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: No ecological constraints were identified in relation to the proposal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposed single storey rear extension would not result in any undue impact upon the aesthetic character of the Sabden Conservation Area or the surrounding AONB landscape and would not cause any significantly detrimental impact upon residential amenity. It is for the above reasons and having regard to all material considerations and matters raised that the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That planning consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |