| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | LW |  |  |  | Date: |  | 31/05/23 |  | Manager: |  | KH | Date: |  |
| Application Ref: |  |  |  | 3/2023/0329 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | N/A |  |  | Site Notice: |  | N/A |  |  |  |  |  |
| Officer: |  |  |  | LW |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | PERMISSION NOT REQUIRED |  |  |  |
| Development Description: |  |  |  |  | Prior approval for a single storey rear extension 5.1m long, 3.5m high (max), 2.5m high to eaves. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 5 The Dales, Langho, BB6 8BW |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| No comments received. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The proposal is assessed against the provisions of Schedule 2 Part 1 Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2011/1010: Fell ash tree (Approved) 3/2006/0924: Extending above the garage to create two stories and a one storey extension to the rear of the property – resubmission (Refused) 3/1992/0304: Substitution of house type plot nos 30-33 and 35-36 and erection of 42 detached houses and garages plot nos 34, 48-88 – reserved matters (Approved) 3/1991/0781: Outline application for proposed housing development providing detached properties (Approved) 3/1990/0121: Outline application for the erection of 93 dwellings, sports pavilion and formation of car park and new vehicular access (Approved) 3/1989/0159: Demolition and redevelopment of site. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a two-storey detached property at no.5 The Dales, situated approximately 0.7km west of the defined settlement boundary of Langho. The property features an existing conservatory to the rear of the dwelling, which is proposed for demolition, along with an existing area of decking. The surrounding area is predominantly residential, comprising of numerous detached dwellings. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: The proposal seeks consent for a single storey rear extension which would replace the existing conservatory. The proposed single storey rear extension would project approximately 5.1m from the rear elevation of the main dwellinghouse and extend a width of 5.15m. A pitched roof design would be featured which would measure 2.6m to the eaves and 3.5m to the ridge and include the installation of 6no. roof lights. To the rear elevation, a set of glazed double doors would be featured, whilst to the north facing side elevation, a single door and 2no. windows would be installed. The proposed rear extension would be constructed in materials to match the existing dwellinghouse, including matching facing brickwork and brick cills, Marley Modern roof tiles and brown uPVC windows and doors. As part of the proposal, a small section of the existing decking would also be extended by approximately 0.5m. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Matters: Assessment of proposal in relation to the provisions of Schedule 2 Part 1 Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended): A.1 Development is not permitted by Class A if – a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, MA, N, P, PA or Q of Part 3 of this Schedule (changes of use); Permission to use the dwellinghouse was not granted by virtue of Class M, MA, N, P, PA or Q of Part 3. b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); The extension would not exceed 50% of the total area of curtilage of the property. c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse; The height of the proposed extension would not exceed the height of the existing dwelling. d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the existing dwellinghouse; The eaves of the proposed extension would not exceed the height of the eaves of the existing dwelling. e) the enlarged part of the dwellinghouse would extend beyond a wall which – (i) forms the principal elevation of the original dwellinghouse, or (ii) fronts a highway and forms a side elevation of the original dwellinghouse; The proposed extension would not extend beyond a wall which forms the principal elevation or fronts a highway and forms a side elevation. f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 4m in the case of a detached dwellinghouse or 3m in the case of any other dwellinghouse, or (ii) exceed 4m in height; The proposed extension would extend beyond the rear wall of the original dwellinghouse by more than 4m. g) for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 8m in the case of a detached dwellinghouse or 6m in the case of any other dwellinghouse, or (ii) exceed 4m in height; The proposed single storey rear extension would not extend beyond the rear wall of the original dwellinghouse by more than 8m and would not exceed 4m in height when measured from ground level. h) the enlarged part of the dwellinghouse would have more than a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 3m, or (ii) be within 7m of any boundary of the curtilage of the dwellinghouse being enlarged which is opposite the rear wall of that dwellinghouse; The proposed extension would be single storey. i) the enlarged part of the dwellinghouse would be within 2m of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3m; The proposed extension would be within 2m of the boundary of the curtilage; however, the height of the eaves would not exceed 3m. j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse and would – (i) exceed 4m in height, (ii) have more than a single storey, or (iii) have a width greater than half the width of the original dwellinghouse; The proposed extension would not extend beyond a wall forming a side elevation. ja) any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs (e) to (j); N/A k) it would consist of or include – (i) the construction or provision of a verandah, balcony or raised platform, (ii) the installation, alteration or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse; The proposed development would include an extension to the existing decking area. Despite this, the Technical Guidance states that a raised platform is any platform with a height greater than 0.3m. The submitted plans indicate that the decking would measure 0.15m in height. This has also been confirmed by the agent. As such, the proposed decking is therefore not considered to be a raised platform and the development would not include any of the above criteria. l) the dwellinghouse is built under Part 20 of this Schedule (construction of new dwellinghouses). The dwellinghouse is not built under Part 20. A.3 Development is permitted by Class A subject to the following conditions – a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; The proposed rear extension would be constructed in materials to match the existing dwellinghouse including matching facing brickwork and brick cills, Marley Modern roof tiles and brown uPVC windows and doors. b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse must be – (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7m above the floor of the room in which the window is installed; The proposal does not include any first-floor windows. c) where the enlarged part of the dwellinghouse has more than a single storey or forms an upper storey on an existing enlargement of the original dwellinghouse, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse. The proposed extension would not be more than single storey. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: On the basis of the above, the proposed single storey rear extension would meet the requirements of Schedule 2 Part 1 Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). Adjoining neighbours have been informed of the proposal and no objections have been received from the occupiers of the adjacent properties. As such, prior approval is not required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | Prior approval not required. |  |  |  |  |  |  |  |  |  |  |  |