| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | LW |  |  |  | Date: |  | 14/07/23 |  | Manager: |  | KH | Date: | 17/07/23 |
| Application Ref: |  |  |  | 3/2023/0411 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | N/A |  |  | Site Notice: |  | N/A |  |  |  |  |  |
| Officer: |  |  |  | LW |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Certificate of Lawfulness for a proposed single storey rear extension. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 33 Fuchsia Way, Longridge, PR3 2NG |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Schedule 2 Part 1 Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2021/0010: Variation of condition of planning application 3/2018/0975. Condition 1 – Proposed plot substitutions and house types (Approved) 3/2021/0009: Discharge of conditions of planning application 3/2018/0975. Condition 5 – Emergency link road (Approved) 3/2019/0870: Discharge of condition 8 (drainage strategy) and 11 (construction management plan) or planning permission 3/2017/0232 (Approved) 3/2019/0601: Discharge of conditions 2 (materials) and 4 (public open space) from planning permission 3/2018/0975 (Approved) 3/2018/0975: Approval of reserved matters (layout, scale, appearance, and landscaping) for Phases 2 and 3 for the erection of 193 dwellings, pursuant to outline planning permission 3/2017/0232 (Approved) 3/2017/0232: Variation of condition 8 (drainage strategy) of planning permission 3/2014/0764 (Approved) 3/2014/0764: Development of up to 363 homes including affordable housing and housing for the elderly, relocation of Longridge Cricket Club to provide a new cricket ground, pavilion, car park and associated facilities, new primary school, vehicular and pedestrian access landscaping, and public open space, with all matters reserved except for access (Approved) 3/2014/0728: Screening opinion for proposed planning application at Higgins Brook, Land east of Chipping Lane, Longridge (Decided) 3/2014/0223: Screening opinion Regulation 5 of the Town and Country Planning (Environmental Impact Assessment) Regulations 2011 Proposed Residential Development (Decided) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a detached bungalow property at no.33 Fuchsia Way, located within the defined settlement boundary of Longridge. The property has recently been constructed during Phase 2 of a new residential development situated on land to the east of Chipping Lane, and still benefits from permitted development rights. The surrounding area is predominantly residential and comprises a mix of detached, semi-detached, and terraced properties. The application site also benefits from no other designations or constraints. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for a Certificate of Lawfulness for a proposed single storey rear extension. The proposed rear extension would project 4m from the rear elevation of the dwellinghouse and extend a width of approximately 4.6m. A pitched roof design would be featured which would measure 2.64m to the eaves and 3.98m to the ridge and include the installation of 2no. velux roof lights. To the rear elevation of the proposal, a set of double doors would be installed, whilst 1no. single door would be installed to the north-western facing side elevation. In regard to materiality, the proposed development would be constructed to match the external appearance of the existing dwellinghouse, including matching red facing brickwork, concrete roof tiles and white uPVC windows and doors. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Matters: Assessment of the proposal in relation to the provisions of Schedule 2 Part 1 Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended): In order to be permitted development, the proposed development needs to satisfy a number of criteria as comprised in Schedule 2 Part 1 Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended) for the enlargement, improvement or other alteration of a dwellinghouse. A.1 Development is not permitted by Class A if – a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class G, M, MA, N, P, PA or Q of Part 3 of this Schedule (changes of use); Permission to use the dwellinghouse as a dwellinghouse as not been granted only by virtue of Class G, M, MA, N, P, PA or Q of Part 3. b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of curtilage (excluding the ground area of the original dwellinghouse); The total area of ground covered by buildings would not exceed 50% of the total area of the curtilage of the dwellinghouse. c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse; The height of the proposed extension would not exceed the height of the existing dwellinghouse. d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse; The eaves height of the proposed extension would not exceed the eaves height of the existing dwellinghouse. e) the enlarged part of the dwellinghouse would extend beyond a wall which – (i) forms the principal elevation of the original dwellinghouse, or (ii) fronts a highway and forms a side elevation of the original dwellinghouse; The proposed extension would not extend beyond a wall which forms the principal elevation or fronts a highway and forms a side elevation of the dwellinghouse. f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 4m in the case of a detached dwellinghouse, or 3m in the case of any other dwellinghouse, or (ii) exceed 4m in height; The proposed extension would not project more than 4m from the rear wall of the dwellinghouse and would not exceed 4m in height. g) for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 8m in the case of a detached dwellinghouse, or 6m in the case of the any other dwellinghouse, or (ii) exceed 4m in height; The dwellinghouse is not on article 2(3) land nor on a site of special scientific interest and the proposal would not involve a larger home extension. h) the enlarged part of the dwellinghouse would have more than a single storey and – (i) extend beyond the rear wall of the original property by more than 3m, or (ii) be within 7m of any boundary of the curtilage of the dwellinghouse being enlarged which is opposite the rear wall of that dwellinghouse; The proposed extension would not have more than a single storey. i) the enlarged part of the dwellinghouse would be within 2m of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3m; The proposed extension would be within 2m of the boundary of the curtilage, but the eaves height would not exceed 3m. j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse and would – (i) exceed 4m in height, (ii) have more than a single storey, or (iii) have a width greater than half the width of the original dwellinghouse; The proposed extension would not extend beyond a wall forming a side elevation. ja) any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraph e) to j); The dwellinghouse does not feature any existing enlargements. k) it would consist of or include – (i) the construction or provision of a verandah, balcony or raised platform, (ii) the installation, alteration, or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse; or The proposed development would include none of the above. l) the dwellinghouse is built under Part 20 of this Schedule (construction of new dwellinghouses) The dwellinghouse is not built under Part 20 of this Schedule. A.3 Development is permitted by Class A subject to the following conditions – a) the materials used in any exterior work (other than materials used in the construction of a conservatory) must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; The proposed extension would be finished in red facing brickwork, concrete roof tiles and white uPVC windows and doors to match the existing dwellinghouse. b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse must be – (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7m above the floor of the room in which the window is installed; and No first-floor windows are proposed. c) where the enlarged part of the dwellinghouse has more than a single storey or forms an upper storey on an existing enlargement of the original dwellinghouse, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse. The proposed extension would not have more than a single storey. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposed development constitutes permitted development under Schedule 2 Part 1 Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), subject to the conditions outlined in A.3 of this Part. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That the Certificate of Lawfulness be granted. |  |  |  |  |  |  |  |  |  |  |  |