| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: |  | Officer: | BT |  |  |  | Date: |  | 1/2/24 |  | Manager: |  | LH | Date: | 1.2.24 |
| Application Ref: |  |  |  |  | 3/2023/0946 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  |  | 5/1/24 |  |  | Site Notice: |  | 5/1/24 |  |  |  |  |  |
| Officer: |  |  |  |  | BT |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  |  | REFUSAL |  |  |  |
| Development Description: |  |  |  |  |  | Retention of re-positioned access gate and stone track off Slaidburn Road. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  |  | Moorcock House, Slaidburn Road, Waddington, BB7 3AA. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| Waddington Parish Council: |  |  |  |  |  | Consulted 29/11/23 – no response. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  |  | No objections subject to conditions. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| One objection has been received in relation to the proposal which is summarised as follows: - Impact of the proposal upon visual amenity - Impact of the proposal upon highway safety - Impact of the proposal upon flood risk Additional comments have also been made with respect to the development being implemented to support a future commercial use of the applicant’s land. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable Development Key Statement EN2: Landscape Key Statement DMI2: Transport Considerations Policy DMG1: General Considerations Policy DMG2: Strategic Considerations Policy DMG3: Transport & Mobility Policy DME2: Landscape & Townscape Protection National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: None relevant to the determination of the application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a private vehicle access and access track situated on the Western side of Slaidburn Road on the Northern outskirts of Waddington. The vehicle access in question is sited directly to the North of the residential property of No. 1 The Graces and comprises a trapezium shaped area of permeable bound tarmac. Metal gates providing access to an existing access track are positioned at the Western end of the access approximately 15 metres away from the Western edge of Slaidburn Road. Post and rail fencing and newly planted shrubbery line the Northern and Southern extents of the access. The wider area comprises a mixture of agricultural land, woodland and open countryside. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Retrospective planning consent is sought for the retention of the aforementioned vehicle access. The works carried out relate to the reconfiguration of a pre-existing vehicle access which formerly comprised a grassed area with its access gates set closer to the Western edge of Slaidburn Road. The application’s supporting information states that works for the vehicle access were commenced and completed on 1/8/23 and 15/8/23 respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: The proposal site lies outside of the defined settlement area of Waddington. Policy DMG2 of the Ribble Valley Core Strategy states that proposals for development outside the Borough’s defined settlement areas can be considered as justifiable if ‘the development is needed for the purposes of forestry or agriculture’. The application’s supporting information states that the land in the applicant’s ownership adjacent to the vehicle access extends to 40 acres and is a registered agricultural holding. The application’s supporting information further states that the applicant currently manages the land as an organic unit with their livestock ownership comprising a mixture of rare breed pigs, cows, sheep and poultry and that work is also being undertaken with the AONB Farming in the Protected Landscape (FIPL) officer to restore hedgerow and walls. It is stated that the unauthorised vehicle access currently provides access for agricultural vehicles including tractors and pasture topping machinery to the grazing land for general farm maintenance, fencing and bracken cutting with the access also being used to transport sheep to the grazing land using a 4 wheel drive and livestock trailer. It is stated that the access gate within the pre-existing access was originally located adjacent to the highway which in turn required vehicles to stop on the highway to allow the gate to be opened and closed. The applicant has stated that moving the access gate back 15 metres from the highway now allows vehicles (tractors, 4 wheel drives and trailers) to leave the highway before opening the gate. Notwithstanding the above, the Council has concerns regarding the extent and scale of the unauthorised works undertaken on site and it being reasonably necessary for the purposes of agriculture. Historic Google Street imagery shows that the former agricultural access was largely understated in terms of both its size and external appearance by virtue of its modestly sized width, natural grassed track surface and rustic timber gates. In contrast, the reconfigured vehicle access comprises a sizeable area of permeable bound tarmac which extends from the adjacent highway for a distance of approximately 15 metres with the width of the former access having now been enlarged to 8 metres. The applicant’s comments are noted with respect to the pre-existing access gates posing a potential highway safety concern due to their proximity to the roadside, as such relocating the access gate further into the site is considered to be justified. However the works to widen the access / access track and the extent of crushed tarmac are considered to be excessive for the agricultural use stated and insufficient evidence has been provided on an agricultural need for the works undertaken. Accordingly, the unauthorised works undertaken are not considered to be reasonably necessary for the purposes of agriculture. In addition, with respect to development within National Landscapes (formerly known as Areas Of Outstanding Natural Beauty) Policy DMG2 states: ‘The most important consideration in the assessment of any development proposals will be the protection, conservation and enhancement of the landscape and character of the area…development will be required to be in keeping with the character of the landscape and acknowledge the special qualities of the AONB by virtue of its size, design, use of material, landscaping and siting.’ As previously conveyed, significant operational development has been undertaken in the construction of the unauthorised vehicle access which for the most part reads as an anomalous form of development against the surrounding open countryside backdrop. Furthermore, these works are fully visible within the public realm from Slaidburn Road and additional works including a reduction in height to an adjacent dry stone wall and roadside vegetation would also be required in order to achieve adequate visibility splays for the access. Consequently, it is considered that the unauthorised development, if retained, would read as a harmful addition to the site that would fail to conserve or enhance the character of the surrounding National Landscape. Further assessment with respect to the visual impact of the unauthorised development is covered in more detail in the visual amenity section of this report. Taking account of the above, the proposal fails to satisfy the requirements of Policy DMG2 and is therefore considered to be unacceptable in principle. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact Upon Residential Amenity: The vehicle access proposed for retention is sited in close proximity to the rear curtilage area of No. 1 The Graces however use of the access in question was pre-existing prior to the development of the grouping of residential properties immediately to the South and in this instance it is not considered that its continued use would result in noise and disturbances that would be unduly harmful to the amenity of any neighbouring residents. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity/External Appearance: Paragraph 135 (c) of the NPPF states: ‘Planning policies and decisions should ensure that developments are sympathetic to local character and history, including the surrounding built environment and landscape setting’. Policy DMG1 of the Ribble Valley Core Strategy provides additional general design guidance as follows: ‘All development must be sympathetic to existing and proposed land uses in terms of its size, intensity and nature as well as scale, massing and style…particular emphasis will be placed on visual appearance and the relationship to surroundings, including impact on landscape character.’ With respect to development within National Landscapes (previously known as Areas Of Outstanding Natural Beauty) Paragraph 182 of the NPPF states: ‘Great weight should be given to conserving and enhancing landscape and scenic beauty in National Parks, the Broads and Areas of Outstanding Natural Beauty.’ The above is reiterated within Key Statement EN2 of the Core Strategy: ‘The landscape and character of the Forest of Bowland Area of Outstanding Natural Beauty will be protected, conserved and enhanced. Any development will need to contribute to the conservation of the natural beauty of the area. As a principle the Council will expect development to be in keeping with the character of the landscape, reflecting local distinctiveness, vernacular style, scale, style, features and building materials.’ As previously conveyed, significant operational development has been undertaken in the construction of the unauthorised vehicle access. These works include the construction of a sizeable area of permeable bound tarmac which extends in from the adjacent highway for a distance of approximately 15 metres, the visual impact of which has been further exacerbated through the enlargement of the former vehicle access width to 8 metres. Additional works undertaken on site include linear tree planting and the installation of a metal gate. All of the above works comprise a somewhat domestic appearance and read as a significant departure from the rustic external appearance of the former vehicle access. Further obtrusive works in the form of a reduction in height to an adjacent dry stone wall and roadside vegetation would also be required in order to achieve adequate visibility splays for the access. In addition, the vehicle access proposed for retention is sited directly adjacent to the Western edge of Slaidburn Road which serves as a busy thoroughfare between Waddington and the Borough’s Northern settlements. As such, the unauthorised vehicle access, if retained, would be fully viewable within the public realm. Taking account of the above, it is considered that retention of the unauthorised vehicle access would be visually harmful to the character of the surrounding National Landscape. The proposal therefore fails to satisfy the requirements of Paragraph 135 (c) and 182 of the NPPF and Key Statement EN2 and Policy DMG1 of the Core Strategy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways and Parking: Lancashire County Council Highways initially made a request for further information to be provided with respect to a visibility splay and access track drawing, information regarding drainage and an operation Statement detailing the use of the access. This information has since been provided and reviewed by the LHA who have deemed the supplementary information provided to be satisfactory, however the Council maintains its concerns with respect to the need to retain the extensive works undertaken solely for the purposes of agriculture. Notwithstanding this, it is not considered that retention of the unauthorised development would have any undue impact upon the surrounding highway network. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: The unauthorised vehicle access abuts the Eastern edge of land in the applicant’s ownership which is also a Biological Heritage Site however a case officer inspection of the application site and adjoining land to the West found no evidence to suggest that any operational development had been undertaken within the protected area. Analysis of historic Google Street imagery suggests that some minor tree removal may have been undertaken within the area currently comprising the access’s hardstanding area however the removed trees do not appear to have been mature and were not within the confines of the adjacent Biological Heritage Site. No other ecological constraints were identified in relation to the proposal. As such, it is not considered that retention of the unauthorised works would be harmful to the ecology of the area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Matters: Drainage In responding to the initial request from the LHA with respect to the provision of additional supporting information, the applicant had originally proposed to manage drainage from the access through connecting to an adjacent highway gully however the LHA have informed the applicant that new connections to a highway drainage system are not accepted. As such, the LHA have stipulated for surface water to be collected within the application site and drained to a suitable internal outfall in the event of the unauthorised works being retained. Third party representations received in relation to the proposal make reference to the unauthorised works having compromised land drains which in turn has allegedly created significant flooding issues for a neighbouring property. A drainage scheme will be required in the event of any positive determination in order to demonstrate that the proposal will not result in unacceptable surface water flooding to the site and surrounding land including the highway. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: It is not considered that sufficient evidence has been provided to demonstrate that retention of these works would be reasonably necessary for the purposes of agriculture. In addition, it is considered that retention of the unauthorised vehicle access would be visually harmful to the character of the surrounding National Landscape. As such, the proposal fails to satisfy the requirements of Paragraph 135 (c) and 182 of the NPPF and Key Statement EN2 and Policies DMG1 and DMG2 of the Core Strategy. As such, for the above reasons and having regard to all material considerations and matters raised that the application is recommended for refusal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  | That planning consent be refused for the following reasons: |  |  |  |  |  |  |  |  |  |  |  |
| 01: | The proposed works, including widening of the access and construction of a sizeable area of permeable bound tarmac, are considered to be excessive for the use intended, would fail to conserve or enhance the character of the surrounding Area of Outstanding Natural Beauty (National Landscape) and insufficient evidence has been provided to demonstrate that the works are reasonably necessary for the purposes of agriculture. The proposal would therefore fail to satisfy the requirements of Policy DMG2 of the Ribble Valley Core Strategy and as such is considered to be unacceptable in principle. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 02: | The proposed works, by virtue of the creation of hardstanding, widening of the former access width, public visibility of the access and the subsequent loss of dry stone walling and vegetation that would be required to facilitate retention of the access, would be visually harmful to the character of the surrounding Area of Outstanding Natural Beauty (National Landscape). The proposal therefore fails to satisfy the requirements of Paragraph 135 (c) and 182 of the NPPF and Key Statement EN2 and Policy DMG1 of the Ribble Valley Core Strategy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |