| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | BT |  |  |  | Date: |  | 22/2/24 |  | Manager: |  | LH | Date: | 23/2/24 |
| Application Ref: |  |  |  | 3/2023/1020 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 26/7/23 |  |  | Site Notice: |  | 22/12/24 |  |  |  |  |  |
| Officer: |  |  |  | BT |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Listed Building Consent for the erection of a single-storey extension to rear with formation of one wall opening with installation of structural support above. Resubmission of 3/2023/0551. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | The Priory, Hellifield Road, Gisburn, BB7 4HQ. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| Gisburn Parish Council: |  |  |  |  | Consulted 19/12/23 – no response. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| None. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| None. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable Development Key Statement EN5: Heritage Assets Policy DMG1: General Considerations Policy DMG2: Strategic Considerations Policy DME3: Site And Species Protection And Conservation Policy DME4: Protecting Heritage Assets Policy DMH5: Residential and Curtilage Extensions Planning (Listed Buildings and Conservation Areas) Act Section 16 & 66 National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2023/1019: Planning application for the proposed erection of a single-storey extension to rear. Resubmission of 3/2023/0552 (Ongoing) 3/2022/1057: Listed Building Consent for the amelioration of unauthorised works. Proposed internal alterations. Proposed internal and external repairs/works (Approved) 3/2022/0557: Discharge of conditions 3 (Method Statement) and 4 (Window Specifications) from Listed Building Consent 3/2022/0410 (Refused) 3/2022/0410: Listed Building Consent for the removal of existing render wall finish to the west elevation. Conversion of existing boiler room to form new WC and utility room. Proposed replacement of existing floor boards to the ground floor with new oak floor boards. Removal of radiators to the ground floor and installation of under-floor heating. Replacement of existing window to the west elevation of the proposed entrance hallway. Renewal/upgrading of existing electrical, heating and plumbing installations. Internal re-plastering. Associated drainage works. Removal of foul drainage/waste pipes from the east elevation. Investigation into condition of existing timber joists to ground floor and removal of banked up earth below the suspended floors to the ground floor and installation of concrete bases below the level of the suspended floors to the ground floor (Approved) 3/2022/0099: Discharge of conditions 3 (materials) and 6 (rooflights) from planning permission 3/2021/1076 (Approved) 3/2021/1282: Proposed erection of single storey extension to north elevation. Alteration of existing lean-to and removal of existing render wall finish to the west elevation. Replacement of existing window to the west elevation of the proposed entrance hallway (Refused) 3/2021/1281: Listed Building Consent for the erection of single storey extension to north elevation. Alteration of existing lean-to and removal of existing render wall finish to the west elevation. Proposed internal alterations, underfloor heating, replacement of existing window to the west elevation of the proposed entrance hallway. Renewal of existing electrical, heating and plumbing installations. Internal re-plastering. Associated drainage works. Removal of foul drainage/waste pipes from the east elevation (Refused) 3/2021/1076: Listed Building Consent for rebuilding and repairs to existing chimney stacks. Re-roofing works and repairs to all roofs, with repairs to central valley/channel gutter. Replacement of existing rainwater goods with aluminium rainwater goods. Brick plinth to wall bases to be removed and external ground levels lowered below level of internal floors, with French drains installed to building perimeter. Repairs to external windows, doors and fascia boards. Replacement of existing roof window with new conservation roof window. Existing soil/waste pipes to be replaced with aluminium/cast iron pipes (Approved) 3/2016/0158: Installation of new central heating system, boiler and 125mm flue projecting approximately 600mm above the slates (Approved) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a detached two storey property in Gisburn. The property consists of a slated roof, timber doors and windows and stone elevations painted in white with a rendered finish to the Western elevation of the property. The property comprises an inverted ‘T’ shaped footprint topped with a split level overhanging hipped roof profile with the Northernmost element of the property accommodating the lower of the building’s two hipped roof elements. The property comprises a single storey mono pitched element (a former dairy) which straddles the South-western and North-western elevations of the property’s internal hallway and entrance respectively. Access to the property is from Hellifield Road via a single track vehicle access which also serves the adjacent neighbouring properties of Kirk Close and The Old Vicarage. The Grade II* Listed Building of St. Mary’s Church and its associated grounds lie just to the South-west with the defined settlement area of Gisburn lying slightly further away to the South and East. The historic park and garden of Gisburne Park (Grade II Listed) abuts the Northern perimeter of the property’s curtilage with additional open countryside sited further away to the North. The application property holds Grade II Listed Building status. The official Historic England listing description for The Priory reads as follows: ‘House, early C19th. Slobbered rubble with hipped slate roof overhanging at eaves. 2 storeys. Windows sashed with glazing bars which intersect at their heads. The plain stone surrounds have pointed heads on the ground floor and hoods on the 1st floor. The central chimney has 3 diagonal caps. The east wall is of one bay and has a 1st floor window similar to those to the south. The door below on the ground floor has a plain stone surround with pointed head. On the north side, against part of the rear wall, is an outshut. In its east wall it has a door with plain stone surround having a pointed head above a flat lintel. The space between the lintel and the arch is glazed with intersecting glazing bars. Adjoining the outshut at the north is a wing with separate hipped roof. It is of one bay towards the east and has mullioned windows with elliptical heads to the lights, and hoods. The ground floor one is of 4 lights, the 1st floor one of 2 lights. On the north side is a chimney with 2 diagonal caps.’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Listed Building Consent is sought for the construction of a single storey rear extension. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: When assessing works to Listed Buildings and their settings, the LPA must accord with their duties at sections 16 and 66 of the Planning (Listed Buildings and Conservation Areas) Act 1990 which state: 16. In considering whether to grant listed building consent for any works the local planning authority or the Secretary of State shall have special regard to the desirability of preserving the building or its setting or any features of special architectural or historic interest which it possesses. 66. In considering whether to grant planning permission [or permission in principle] for development which affects a listed building or its setting, the local planning authority or, as the case may be, the Secretary of State 2 shall have special regard to the desirability of preserving the building or its setting or any features of special architectural or historic interest which it possesses. Chapter 16 of the National Planning Policy Framework sets out expectations with regards to conserving and enhancing the historic environment. Applicants are required to describe the significance of any heritage assets affected, including any contribution made by their setting. Local Planning Authorities should consider any loss of historic fabric to constitute harm, but to make an assessment as to the significance of the asset and apply weight to its conservation accordingly. Accordingly, the proposed works to the Listed Building will be carefully assessed with respect to the duties above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon Listed Building: Paragraph 205 of the NPPF states: ‘When considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). This is irrespective of whether any potential harm amounts to substantial harm, total loss or less than substantial harm to its significance.’ In addition, Key Statement EN5 of the Ribble Valley Core Strategy states: ‘There will be a presumption in favour of the conservation and enhancement of the significance of heritage assets and their settings.’ Furthermore, Policy DME4 of the Core Strategy states: ‘Alterations or extensions to Listed Buildings or buildings of local heritage interest, or development proposals on sites within their setting which cause harm to the significance of the heritage asset will not be supported.’ Heritage impact is considered to be the potential level of harm upon the significance of a heritage asset caused by development proposals. The NPPF defines significance as ‘the value of a heritage asset to this and future generations because of its heritage interest’. Such interest can be archaeological, architectural, artistic or historic. A Heritage Statement has been provided in support of the application which details the significance of The Priory as follows: Archaeological Interest: ‘The archaeological interest of the building lies in its extant historic fabric, which is now largely restricted to its walls, roof and windows, with other isolated elements i.e., individual floor timbers and surviving doors.’ Architectural And Artistic Interest: ‘The building is modest and appears to have been designed as two wings…this has led to a somewhat unusual yet interesting architectural form…the diagonal chimney stacks are of interest as well as the existing windows and pattern of fenestration.’ Historic Interest: ‘In terms of illustrative historic interest, the building is illustrative of a 19th century dwelling that likely belonged to someone higher up on the social scale and had the benefit of servants which is suggested by the historic plan form of the house.’ Accordingly, the above observations indicate that the significance of The Priory is largely underpinned by its architectural interest (unusual building form, chimney stacks, split level hipped roof profile and fenestration) and historic interest (evidence of historic social class divisions) along with some limited archaeological interest (surviving historic fabric). Historic England guidance Making Changes to Heritage Assets (2016) states: ‘The main issues to consider in proposals for additions to heritage assets are proportion, height, massing, bulk, use of materials, relationship with adjacent assets [and] alignment…the plan form of a building is frequently one of its most important characteristics…historic fabric will always be an important part of the asset’s significance…it is not appropriate to sacrifice old work simply to accommodate the new…new features added to a building are less likely to have an impact on the significance if they follow the character of the building…it would not normally be good practice for new work to dominate the original asset or its setting in either scale, material or as a result of its siting.’ In a similar vein, the Institute Of Historic Building Conservation (2021) advises: ‘Modern extensions should be harmonious and not dominate the existing building in scale, materials, situation, or impact on setting. Successful extensions require a thorough understanding of the building type and sensitive handling. The design of new elements intended to stand alongside historic fabric needs to be very carefully considered and to be successful should respect the setting and the fundamental architectural principles of scale, height, massing, alignment, and use of appropriate materials…a building derives much of its character and appearance from its roof. Roofs often contain the oldest and least altered parts of a building. Alterations should respect what survives in its original form…The alteration of original roof forms and pitches should generally be discouraged.’ In this instance, the proposed extension would serve as an infill between the North-western and South-western elevations of the property’s utility room and kitchen components respectively. The extension proposed would add additional floorspace to the property through the creation of a dining room area and as such would result in a deviation from the original plan form of the property. Notwithstanding this, the footprint of the proposed extension would largely mirror the square / rectangular proportions of the property’s kitchen, living room, entrance hall and rear bedroom and would neatly round the symmetry of the building’s footprint off to a discernible ‘L’ shape. Furthermore, the existing property is of modest proportions with limited functional space therefore a relatively unobtrusive deviation from the property’s original plan form to accommodate modern living is considered to be acceptable in this instance. Accordingly, given the proportions and layout of the proposed extension, it is not considered that the proposal would be overtly disruptive to the original plan form of the building to the extent of harming the historic significance of the heritage asset. The proposed extension would incorporate an overhanging hipped roof design which would be largely in keeping with and respectful to the overhanging hipped roof profile of the property. Fenestration within the proposed extension would comprise a square window opening and bi-fold door opening, with the square window opening being of similar proportions to the property’s South-western facing entrance / ensuite windows and with the bi-fold door opening mirroring the vertical emphasis of the property’s entrance door. The extension proposed would therefore read as largely harmonious with the existing property in terms of its fenestration. The works proposed would involve the loss of some historic building fabric through the creation of a doorway between the proposed extension and the property’s kitchen however the doorway opening proposed would be partially formed through the use of an existing window opening therefore the loss of historic wall fabric from the works proposed would be minimal. Taking account of all of the above, the works proposed would be largely compliant with current heritage guidance and it is not considered that the proposed development would result in any harm to the significance of the heritage asset. The proposed development would therefore satisfy the requirements of Section 16 of the Listed Buildings and Conservation Areas) Act 1990, Paragraph 205 of the NPPF and Key Statement EN5 and Policy DME4 of the Ribble Valley Core Strategy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposed extension would predominantly mirror the existing symmetry, proportionality and fenestration of the application property and as such would read as a largely congruent addition to the heritage asset. The proposed development would result in some deviation from the original plan form of the building in addition to the loss of some historic building fabric however it is not considered that this would detract from or result in any harm to the significance of the heritage asset. As such, for the above reasons and having regard to all material considerations and matters raised that the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That Listed Building Consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |