| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | EP |  |  |  | Date: |  | 04/04/2024 |  | Manager: |  | SK | Date: | 9.4.24 |
| Application Ref: |  |  |  | 3/2024/0100 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | N/A |  |  | Site Notice: |  | N/A |  |  |  |  |  |
| Officer: |  |  |  | EP |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Certificate of lawfulness for proposed single-storey extension to rear. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 23 Woodlands Park, Whalley BB7 9UG. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| No comments received. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The proposal is assessed against the provisions of Schedule 2, Part 1, Class A and Class G of the Town and Country Planning (General Permitted Development) Order 2015 (as amended). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: No recent planning history. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a detached dwelling in Whalley. The surrounding area is predominantly residential and the application site itself is not on any designated land. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for the erection of a single storey extension sited at the rear of 23 Woodlands Park and the installation of a flue on said extension. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: The proposal seeks to determine whether the proposal falls under the realm of permitted development or if full planning consent is required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Matters: Assessment of proposal in relation to the provisions of Schedule 2, Part 1, Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015: In order to be permitted development, the proposal needs to satisfy a number of criteria as comprised in Schedule 2, Part 1, Class A and Class G of the Town and Country Planning (General Permitted Development) Order 2015 (as amended) for the enlargement, improvement or other alteration of a dwellinghouse. Single-storey extension. Development is not permitted by Class A if – (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); Permission for use as a dwellinghouse was not granted by virtue of class M, N, P or Q of Part 3. (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); The single storey extension would not exceed 50% of the total area of curtilage of the property. (c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse; The height of the proposed single storey extension would not exceed the height of the existing dwelling. (d) the height of the eaves of the part of the dwelling house enlarged or improved or altered would exceed the height of the eaves of the existing dwelling house; The height of the eaves of the proposed extension would not exceed the height of the eaves of the existing dwellinghouse. (e) the enlarged part of the dwellinghouse would extend beyond a wall which— (i) forms the principal elevation of the original dwellinghouse; or (ii) fronts a highway and forms a side elevation of the original dwellinghouse; The proposed single storey extension would not extend beyond a wall which forms the principal elevation of the original dwellinghouse or fronts a highway and forms a side elevation of the original dwellinghouse. (f) subject to paragraph (g), the enlarged part of the dwelling house would have a single storey and – (i) extend beyond the rear wall of the original dwelling house by more than 4 metres in the case of a detached dwelling house or 3 metres in the case of any other dwelling house, or (ii) exceed 4 metres in height; The dwelling is detached, and the proposed single storey extension would extend beyond the rear wall of the original dwellinghouse by 1.98 metres and will measure 3.2 metres in height. (g) For a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and — (i) extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse, or (ii) exceed 4 metres in height;” N/A h) the enlarged part of the dwelling house would have more than a single storey and – (i) extend beyond the rear wall of the original dwelling house by more than 3 metres, or (ii) be within 7 metres of any boundary of the curtilage of the dwelling house opposite the rear wall of the dwelling house; The proposed rear extension would be single storey. (i) the enlarged part of the dwelling house would be within 2 metres of the boundary of the curtilage of the dwelling house, and the height of the eaves of the enlarged part would exceed 3 metres; The proposed single storey extension will measure 2.58 metres in height to the eaves. (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would – (i) exceed 4 metres in height, (ii) have more than one storey, or (iii) have a width greater than half the width of the original dwellinghouse; or The proposed single storey extension would not extend beyond a wall forming a side elevation of the original dwellinghouse. (k) it would consist of or include— (i) the construction or provision of a veranda, balcony or raised platform, N/A (ii) the installation, alteration or replacement of a microwave antenna, N/A (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or The proposed development involves the introduction of a flue to the eastern extent of the extension; however this can be assessed under Class G of this part. (iv) an alteration to any part of the roof of the dwellinghouse. N/A The proposal meets the above criteria to be classed as permitted development subject to an assessment of the proposed flue under Class G. Conditions A.3 Development is permitted by class A subject to the following conditions (a) the materials used in any exterior work (other than materials used in the construction of a conservatory) must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; The proposed development will be constructed in off white render to match the existing dwelling. Flue. G.1 Development is not permitted by Class G if— (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class G, M, MA, N, P, PA or Q of Part 3 of this Schedule (changes of use); Permission for use as a dwellinghouse was not granted by virtue of class M, N, P or Q of Part 3. (b) the height of the chimney, flue or soil and vent pipe would exceed the highest part of the roof by 1 metre or more; or The proposed flue will not exceed the height of the highest part of the roof by more than 1 metre. (c) in the case of a dwellinghouse on article 2(3) land, the chimney, flue or soil and vent pipe would be installed on a wall or roof slope which— (i) fronts a highway, and (ii) forms either the principal elevation or a side elevation of the dwellinghouse.; or N/A (d) the dwellinghouse is built under Part 20 of this Schedule (construction of new dwellinghouses). N/A The proposal meets the above criteria to be classed as permitted development. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposed works constitute permitted development under The Town and Country Planning (General Permitted Development) (England) Order 2015 Schedule 2, Part 1, Class A and Class G subject to the external facing materials being of a similar appearance to those used in the construction of the exterior of the existing dwelling house. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | To approve the application for a Certificate of Lawfulness. |  |  |  |  |  |  |  |  |  |  |  |