| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | LW |  |  |  | Date: |  | 02/05/24 |  | Manager: |  | LH |  | Date: | 3/5/24 |
| Application Ref: |  |  | 3/2024/0199 |  |  |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  | N/A |  |  |  | Site Notice: |  | N/A |  |  |  |  |  |  |
| Officer: |  |  | LW |  |  |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | Decision |  | PERMISSION NOT REQUIRED |  |  |
| Development Description: |  |  |  |  | Prior notification of a roof over an existing agricultural block waste building. |  |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | Bolton Fold Farm, Alston Lane, Longridge, PR3 3BN |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |  |
| A consultation response from Longridge Town Council was received on 12th April, stating that the Council had no observations to make. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cadent Gas |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No comments received within consultation period. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Schedule 2, Part 6, Class A of the Town and Country Planning (General Permitted Development) Order 2015 and (Amendment) Order 2018 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: # 3/2024/0055: Prior approval of a roof over an existing slurry store (Permission Not Required) # 3/2023/0650: Erection of new agricultural building for cow cubicle (Approved). # 3/2023/0206: Proposed single storey side extension (Approved). # 3/2022/0208: Proposed new buildings Unit 1 and Unit 3 to provide storage of animal feed to include hay/straw and machinery (Permission Required). # 3/2022/0094: Proposed new buildings Unit 1 and Unit 3 to provide storage of animal feed to include hay/straw and machinery (Refused). # 3/2021/0090: Proposed new building to provide storage of animal feed to include hay/straw and also machinery storage (Permission not Required). # 3/2016/1084: Phase 2 additional cattle housing building (Resubmission of application 3/2015/0621) (Approved). # 3/2016/1083: Phase 1 extension to existing cattle housing and reconfiguration of orientation of existing building. Resubmission of planning application 3/2015/0616 (Approved). # 3/2016/0897: Phase 1 roof covering and walls to existing earth banked silo pit (Approved). # 3/2016/0896: Phase 2 extension of roof covering and walls to existing earth banked silo pit (Approved). # 3/2015/0621: Extension to existing cattle housing Phase 2 of 2 (Withdrawn). # 3/2015/0616: Extension to existing cattle housing Phase 1 of 2 (Refused). # 3/2013/0707: Outline application for proposed erection of a single dwelling (Approved). # 3/2012/0563: Proposed new agricultural building for the housing of dairy cattle (Approved). # 3/2010/0932: Proposed chill store extension to existing dairy (Approved). # 3/2009/0565: Application for the removal of condition no.5 of planning consent 3/2009/0141P (Approved). # 3/2009/0141: Resubmission of 3/2008/0227 to deal with elevation changes to construction of building to house farm shop, café, education room and bakery, layout of parking area and creation of small animal enclosures (Approved). # 3/2008/0227: Construction of building to house farm shop, café, education room and bakery, laying out of parking area and creation of small animal enclosures (Approved). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to an existing agricultural complex known as Bolton Fold Farm, accessed off Alston Lane and comprising numerous agricultural buildings. The site to which the application relates is located within the Open Countryside, over 500m south of the defined settlement area of Longridge. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: The applicant seeks a determination as to whether the Council’s prior approval is required for the construction of a proposed roof over an existing agricultural block waste building, along with a single bay extension. The proposed structure would have a maximum width and depth of 19.9m and 45.8m respectively and would incorporate a pitched roof form measuring 9.1m to the eaves and 11.83m to the ridge. The proposal would be open ended and constructed from pre-cast concrete panels to the elevations and Green Hill System sheet cladding to the roof. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Whether or not permitted development The scheme must satisfy a number of criteria as set out under Class A of Part 6 of Schedule 2 of the Town and Country Planning (General Permitted Development) Order 2015 (as amended 2018). The first of those requirements is that the development must be ‘carried out on an agricultural unit of 5 hectares or more’ and be ‘reasonably necessary for the purposes of agriculture within that unit’. The agricultural holding is 242 hectares in area and the proposal is for a roof over an existing agricultural block waste building in order to allow waste to be kept dry and to prevent effluent discharge and pollution to nearby watercourses in compliance with Environment Agency guidelines. Accordingly, the proposed development is considered to be reasonably necessary for the purposes of agriculture in this instance. Having regard to criteria a) – k), development is not permitted by Class A if – (a) the development would be carried out on a separate parcel of land forming part of the unit which is less than 1 hectare in area; The submitted application form states that the land parcel where the development would be located has an area of less than 1 hectare. However, the submitted extent of holding plan shows that the development would be located on a parcel of land which has an area greater than 1 hectare and therefore the proposal would comply with the above criterion. (b) it would consist of the erection or extension of any agricultural building on an established agricultural unit (as defined in paragraph X of Part 3 of this Schedule) where development under Class Q or S of Part 3 (changes of use) of this Schedule has been carried out within a period of 10 years ending with the date on which development under Class A(a) begins; No development on the site has been carried out under Class Q or S of Part 3 of this schedule within the last 10 years. (c) it would consist of, or include, the erection, extension or alteration of a dwelling; The development does not include any works in relation to a dwelling. (d) it would involve the provision of a building, structure or works not designed for agricultural purposes; The proposal comprises a roof over an existing agricultural block waste building. (e) the ground area which would be covered by— (i) any works or structure (other than a fence) for accommodating livestock or any plant or machinery arising from engineering operations; or (ii) any building erected or extended or altered by virtue of Class A, would exceed 1000 square metres, calculated as described in paragraph D.1(2)(a) of this Part; An area calculated as described in paragraph D.1(2)(a) comprises the ground area which would be covered by the proposed development, together with the ground area of any building (other than a dwelling), or any structure, works, plans, machinery, ponds or tanks within the same unit which are being provided or have been provided within the preceding two years and any part of which would be within 90 metres of the proposed development The ground area covered by the proposed structure would measure approximately 911 square metres. However, it is noted that prior notification 3/2024/0055 recently allowed for the construction of a proposed roof over an existing slurry store at Bolton Fold Farm with a footprint of approximately 222 square metres. As such, the two proposals in combination would exceed 1000 square metres. Despite this, Bolton Fold Farm comprises two separate agricultural holdings, with the boundary of the two units meeting in the centre of the main farmyard area, as confirmed by the agent and identified on a plan. In this respect, the proposal would comply with the above criterion insofar that the proposed building would be sited on a separate unit to that of the structure allowed under 3/2024/0055 and would comprise a ground area of less than 1000 square metres. (f) the height of any part of any building, structure or works within 3 kilometres of the perimeter of an aerodrome would exceed 3 metres; The proposed building is not within 3km of an aerodrome. (g) the height of any part of any building, structure or works not within 3 kilometres of the perimeter of an aerodrome would exceed 12 metres; The maximum height of the proposed structure would measure 11.83m. (h) any part of the development would be within 25 metres of a metalled part of a trunk road or classified road; The proposed development would not be within 25 metres of a metalled part of a trunk road or classified road. (i) it would consist of, or include, the erection or construction of, or the carrying out of any works to, a building, structure or an excavation used or to be used for the accommodation of livestock or for the storage of slurry or sewage sludge where the building, structure or excavation is, or would be, within 400 metres of the curtilage of a protected building; The development would not be within 400m of the curtilage of a protected building. (j) it would involve excavations or engineering operations on or over article 2(4) land which are connected with fish farming; or The proposal would not involve excavations or engineering operations on or over article 2(4) land which are connected with fish farming. (k) any building for storing fuel for or waste from a biomass boiler or an anaerobic digestion system— (i) would be used for storing waste not produced by that boiler or system or for storing fuel not produced on land within the unit; or (ii) is or would be within 400 metres of the curtilage of a protected building. The proposed building would not be used for storing fuel for or waste from a biomass boiler or an anaerobic digestion system. The proposal satisfies all criteria a) – k) and therefore is defined as permitted development. Whether or not prior approval is needed In accordance with condition A2 (2) (i) the Local Authority must determine whether prior approval is required as to the siting, design and external appearance of the proposal. Siting The proposed development would be sited immediately adjacent to a variety of other agricultural buildings and would comprise a single bay extension and roof over an existing walled structure. As such, it is not considered that the proposal would read as an incongruous or out of keeping addition to the application site or surrounding landscape. However, the proposal would be sited within the consultation zone of a Major Gas Pipeline and as such, prior approval is required in terms of siting given it is unknown whether the proposal would affect this infrastructure asset. Following consultation with the pipeline operator, Cadent Gas, no comments have been received within the statutory consultation period. On this basis, the siting of the development is considered to be acceptable in this particular instance. As such, Prior approval is required and approved in terms of siting. Design and Appearance The size, design, and materials of the proposed development would be typical of an agricultural structure and would be similar to the existing buildings in its immediate context. It is therefore not considered that the design and appearance of the proposed development would result in any measurable undue harm. As such, Prior Approval is not required in terms of design and appearance. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposal complies with all of the criteria set out within Schedule 2, Part 6, Class A of the Town and Country Planning (General Permitted Development) Order 2015 and is therefore defined as permitted development. The design and siting of the proposal is also considered to be acceptable for the reasons stated above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  | Prior Approval Required and Approved. |  |  |  |  |  |  |  |  |  |  |  |