| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | LW |  |  |  | Date: |  | 03/09/24 |  | Manager: |  | KH | Date: | 03/09/24 |
| Application Ref: |  |  |  | 3/2024/0524 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 30/07/24 |  |  | Site Notice: |  | N/A |  |  |  |  |  |
| Officer: |  |  |  | LW |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Proposed demolition of existing side and rear extension and replacement with new single storey, flat roof extension with lantern lights to side and rear. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 14 Copster Drive, Longridge, PR3 3SH |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| No comments received in respect to the proposed development. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| No representations received. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable Development Policy DMG1: General Considerations Policy DMG2: Strategic Considerations National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2002/0910: Erection of conservatory (Approved). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a semi-detached dormer bungalow at no.14 Copster Drive, situated on a corner plot at the junction of Copster Drive and Darwen Close. The property comprises brickwork and render to its external elevations, concrete roof tiles and uPVC windows and benefits from an existing conservatory to the side and rear which is proposed for demolition. The site to which the proposal relates is located within the defined settlement area of Longridge and benefits from no other designations or constraints. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for the demolition of the existing conservatory and construction of a single storey side and rear extension. The proposed extension would project 3.4m from both the northern rear and eastern side elevation of the application property, extending a total width and depth of 9.3m and 5.4m respectively. A flat roof form would be incorporated measuring 3.1m in height, whilst 2no. lantern lights would also be included. To the rear elevation, a set of bi-fold doors and 1no. glazed panel would be featured. An additional bi-fold door would also be featured to the front elevation, along with 2no. glazed panels to the eastern side elevation. In respect to materiality, the proposal would be constructed from brickwork and uPVC windows and doors to match that of the existing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Developemnt: The proposal relates to a domestic extension and alterations to an established residential property and is therefore acceptable in principle subject to an assessment of the material planning considerations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact Upon Residential Amenity: The openings featured to the proposed extension would provide views similar to those afforded by the existing conservatory in which it would replace and therefore no new opportunities for direct overlooking or loss of privacy are anticipated as a result of the works proposed. The proposed extension would have a rearward projection of 3.4m and be sited within close proximity to the common boundary with no.16 Copster Drive, as well as an existing window featured to the rear of this adjoining property. Despite this, the application property still benefits from permitted development rights which would allow for the construction of a 3m rear extension without the need for planning consent, and the proposed development would be set approximately 0.9m from the shared boundary with no.16 Copster Drive and 1.3m from the aforementioned window. In this respect, it is not considered that the proposed development would result in any significant detrimental impact upon the occupiers of this neighbouring dwelling that would warrant the refusal to grant planning permission. In addition to this, the proposal would remain in excess of 10m from the residential property at no.1 Darwen Close and 20m from no.12 Copster Drive. Taking account of the above, it is not considered that the proposed development would result in any significant detrimental harm upon the existing amenities of any nearby residents that would warrant the refusal of the application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity/External Appearance: The application property occupies a prominent position within the street scene being sited on the corner plot at the junction of Copster Drive and Darwen Close. The proposed side and rear extension would therefore be visible from the highway and adjacent public realm. Nevertheless, the proposed development would largely be sited on the same footprint as that of the existing conservatory and would appear appropriate in size and scale for a single storey extension in respect to the existing built form of the property. As such, it is not considered that the proposal would read as an incongruous or over dominant addition to the application site or wider street scene. In addition to this, the external facing materials of the proposed extension would match that of the existing dwellinghouse, ensuring visual integration and further reducing the impact of the proposal Accordingly, it is not considered that the proposed development would result in any measurable undue harm upon the existing visual amenities of the application property or surrounding area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways and Parking: Lancashire County Council Highways have not been consulted on the proposed development; however, the proposal would not result in an increase to the number of bedrooms at the site, nor are any alterations to the existing site access or parking arrangements proposed. As such, the proposal is considered acceptable in respect to highway safety and parking. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: No ecological constraints have been identified in relation to the works proposed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: As such, for the above reasons and having regard to all material considerations and matters raised, the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That planning consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |