| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | BT |  |  |  | Date: |  | 16/1/25 |  | Manager: |  | LH | Date: | 17/1/25 |
| Application Ref: |  |  |  | 3/2024/0623 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 10/10/24 |  |  | Site Notice: |  | 10/10/24 |  |  |  |  |  |
| Officer: |  |  |  | BT |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Listed Building Consent for proposed conversion of barn 1 to two-storey, five bedroom dwelling; demolition of outbuilding 1 and construction of new double garage; construction of new roof and conversion of outbuilding 2 to form single-storey incidental domestic hobby space; alterations to existing vehicular access, creation of domestic curtilage and landscaping; new sewage treatment plant and diversion of public footpath. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | Writtenstone Farm, Writtenstone Lane, Longridge, PR3 2ZN. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| Longridge Town Council: |  |  |  |  | Request that RVBC ensure that the footpath mentioned in the application is not the commonly used public right of way footpath as this needs to be protected. Additional comments provided with respect to the evolution of public footpaths within the locality stating preference for retention of the existing alignment of the footpaths surrounding the application site. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | No objections subject to conditions |  |  |  |  |  |  |  |  |  |
| LCC PROW: |  |  |  |  | Neither supports nor objects to the plans. Further information requested with respect to the alignment of the new route and the construction specification of the proposed diversion. |  |  |  |  |  |  |  |  |  |
| LCC Archaeology: |  |  |  |  | No objections subject to condition. |  |  |  |  |  |  |  |  |  |
| RVBC countryside: |  |  |  |  | No objections subject to conditions. |  |  |  |  |  |  |  |  |  |
| GMEU: |  |  |  |  | No objections subject to conditions in initial response. No further response received following subsequent reconsultation on 9/12/24. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| None. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable development Key Statement EN2: Landscape Key Statement EN4: Biodiversity and Geodiversity Key Statement EN5: Heritage Assets Key Statement DMI2: Transport Considerations Policy DMG1: General considerations Policy DMG2: Strategic considerations Policy DMG3: Transport and Mobility Policy DME2: Landscape and Townscape Protection Policy DME3: Site and Species Protection and Conservation Policy DME4: Protecting Heritage Assets Policy DMH3: Dwellings In The Open Countryside And AONB Policy DMH4: The Conversion Of Barns And Other Buildings To Dwellings Policy DMB5: Footpaths And Bridleways Planning (Listed Buildings and Conservation Areas) Act Section 16 National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2024/0746: Listed Building Consent for proposed re-slating roof including re-leading valleys and chimneys and re-pointing all external walls and washing existing render (Approved) 3/2024/0622: Planning permission for proposed conversion of barn 1 to two-storey, five bedroom dwelling; demolition of outbuilding 1 and construction of new double garage; construction of new roof and conversion of outbuilding 2 to form single-storey incidental domestic hobby space; alterations to existing vehicular access, creation of domestic curtilage and landscaping; new sewage treatment plant and diversion of public footpath (Ongoing) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a farmstead situated on the Eastern outskirts of Longridge. Access to the application site is via Writtenstone Lane from Lower Road with Writtenstone Lane also serving as Public Right Of Way BW0302019. Subsequent access to the site from Writtenstone Lane is via an access track which serves as Public Right Of Way FP0302021 which runs along the South-eastern perimeter of the application site. Writtenstone Farm comprises a detached two storey property set on an ‘L’ shaped footprint with a cross gabled slated roof, stone elevations and mullioned windows within its principal South-eastern elevation. A barn building and two outbuildings lie directly adjacent to the South / South-west of the farmhouse which form the basis of this application, with an additional barn building and agricultural buildings lying to the North-east and North of the farmhouse respectively. The surrounding area comprises a mixture of woodland, agricultural land and open countryside with the defined settlement area of Longridge lying approximately 1 kilometre away to the West of the application site. The application site lies within the setting of the Forest Of Bowland National Landscape by virtue of the National Landscape bordering the North-eastern extents of Writtenstone Lane. Writtenstone Farm holds Grade II Listed Building status with the official Historic England listing description reading as follows: ‘House, probably late C18th. Sandstone rubble with slate roof. L-plan. 2 storeys. Facade of 3 bays with chamfered quoins and windows of 3 lights with square stone mullions and plain stone surrounds, except for the central window on the 1st floor which has a plain stone surround with semi-circular head. The door has a plain stone surround with a semi-circular head.’ The above listed building description does not specifically mention the barn to be converted however the barn building lies within the curtilage of Writtenstone Farm and pre-dates July 1948 (estimated to have C18th origins) and as such falls within the realm of a curtilage listed structure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Listed Building Consent is sought for alterations to the barn building identified as ‘Barn 01’ within the submitted Historic Buildings Appraisal to facilitate its conversion to a dwellinghouse which are as follows: - Installation of doors and windows to historic openings - Installation of new window openings within North-western and South-eastern elevations of barn - Replacement of existing rainwater goods - Re-rendering of South-western elevation of barn with lime based render - Re-pointing of external walls with lime based mortar - Existing roof of barn and attached shippon to be re-slated / replaced - Installation of new roof lights within existing roof light openings of main barn and attached shippon - Installation of flues to North-western and South-eastern roof slopes of barn and attached shippon respectively - Reconfiguration of the barn’s existing internal layout |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: The principle statutory duty under the Planning (Listed Building and Conservation Areas) Act 1990 (as amended by s.58B (1) of Levelling-up and Regeneration Act 2023) is to preserve or enhance the special character of heritage assets, including their setting. LPAs should, in coming to decisions, consider the principle Act which states the following; Listed Buildings – Section 66(1) (as amended by s.58B of Levelling-up and Regeneration Act 2023) In considering whether to grant planning permission for development which affects a listed building or its setting, the local planning authority shall have special regard to the desirability of preserving or enhancing the building or its setting. Under s.58B (2) this includes preserving or enhancing any feature, quality or characteristic of the asset or setting that contributes to the significance of the asset. Listed buildings - Section 16 (2) (as amended by s.58B of Levelling-up and Regeneration Act 2023) In considering whether to grant listed building consent for any works to a listed building the local planning authority shall have special regard to the desirability of preserving or enhancing the building. Under s.58B (2) this includes preserving or enhancing any feature, quality or characteristic of the asset or setting that contributes to the significance of the asset. Chapter 16 of the National Planning Policy Framework sets out expectations with regards to conserving and enhancing the historic environment. Applicants are required to describe the significance of any heritage assets affected, including any contribution made by their setting. Local Planning Authorities should consider any loss of historic fabric to constitute harm, but to make an assessment as to the significance of the asset and apply weight to its conservation accordingly. Accordingly, the proposed works to the Listed Building will be carefully assessed with respect to the duties above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon Listed Building: Given the proposal relates to a Grade II Designated Heritage Asset, special regard must also be given to the statutory duties imposed on the authority, pursuant to national legislation, particularly in respect of the preservation and enhancement of such assets. The principle statutory duty under the Planning (Listed Building and Conservation Areas) Act 1990 (as amended by s.58B (1) of Levelling-up and Regeneration Act 2023) is to preserve or enhance the special character of heritage assets, including their setting. As such, in determining applications that affect designated heritage assets, the authority must consider the duties contained within the principle Act which states the following; Listed Buildings – Section 66(1) (as amended by s.58B of Levelling-up and Regeneration Act 2023): In considering whether to grant planning permission for development which affects a listed building or its setting, the local planning authority shall have special regard to the desirability of preserving or enhancing the building or its setting. Under s.58B (2) this includes preserving or enhancing any feature, quality or characteristic of the asset or setting that contributes to the significance of the asset. Listed buildings - Section 16 (2) (as amended by s.58B of Levelling-up and Regeneration Act 2023): In considering whether to grant listed building consent for any works to a listed building the local planning authority shall have special regard to the desirability of preserving or enhancing the building. Under s.58B(2) this includes preserving or enhancing any feature, quality or characteristic of the asset or setting that contributes to the significance of the asset. Paragraph 212 of the NPPF states: ‘When considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). This is irrespective of whether any potential harm amounts to substantial harm, total loss or less than substantial harm to its significance.’ In addition, Key Statement EN5 of the Ribble Valley Core Strategy states: ‘There will be a presumption in favour of the conservation and enhancement of the significance of heritage assets and their settings.’ Furthermore, Policy DME4 of the Core Strategy states: ‘Alterations or extensions to Listed Buildings or buildings of local heritage interest, or development proposals on sites within their setting which cause harm to the significance of the heritage asset will not be supported.’ Heritage impact is considered to be the potential level of harm upon the significance of a heritage asset caused by development proposals. The NPPF defines significance as ‘the value of a heritage asset to this and future generations because of its heritage interest’. Such interest can be archaeological, architectural, artistic or historic. Statements Of Heritage Significance, Historic England (2019) defines these as follows: ‘Archaeological Interest: There will be archaeological interest in a heritage asset if it holds, or potentially holds, evidence of past human activity worthy of expert investigation at some point.’ ‘Architectural And Artistic Interest: Interests in the design and general aesthetics of a place. They can arise from conscious design or fortuitously from the way the heritage asset has evolved. More specifically, architectural interest is an interest in the art or science of the design, construction, craftsmanship and decoration of buildings and structures of all types. Artistic interest is an interest in other human creative skills, like sculpture’. ‘Historic Interest: An interest in past lives and events (including pre-historic). Heritage assets can illustrate or be associated with them. Heritage assets with historic interest not only provide a material record of our nation’s history but can also provide meaning for communities derived from their collective experience of a place and can symbolise wider values such as faith and cultural identity.’ National Planning guidance requires applicants to describe the significance of any heritage assets affected, including any contribution made by their setting in order to allow the LPA to come to a judgment about the level of impact on that significance and therefore on the merits of the proposal. A Historic Buildings Appraisal has been provided in support of the application which summarises the significance of the application building as follows: ‘The barn as well as the other buildings at Written Stone Farm provide physical evidence of both settlement and agricultural practices at the site since at least the mid-17th century…some 17th century fabric does remain, in particular the two 17th century doorways located to the north east gable and to the north end of the south east elevation…the building is the product of vernacular building tradition, common amongst such buildings and would have been built using locally sourced materials and by skilled craftsman using local building techniques…the building possesses few features of interest and are largely limited to the two-17th century doorways and the existing roof structure. The 19th century cart entry sliding door arrangement is also of interest…in terms of associative historical value, the barn, being part of the farm, is associated with its past occupants including the Radcliffe and Bourn families… There is also associative historical value in relation to the written stone and its associated stories of supposed paranormal activity…in terms of illustrative historical value, the building is a constituent part of a 17th century farmstead that has been altered in the 19th century and likely to have belonged to a farmer of the Yeoman class and their family.’ Accordingly, the above observations indicate that the significance of the application building is underpinned by a combination of its archaeological interest (physical evidence of both settlement and agricultural practices at the site and remnant C17th building fabric), architectural interest (17th century doorways, existing roof structure and 19th century cart entry) and historic interest (association with past occupants). In terms of impacts to the significance of the heritage asset from the proposed works, it is stated that the proposal would result in less than substantial harm to the heritage asset which in turn would be comfortably outweighed by the resultant public benefits and overall enhancement that would be delivered to the character and appearance of the barn. Installation of doors and windows to historic openings & installation of new window openings within North-western and South-eastern elevations of barn The Institute Of Historic Building Conservation (2021) provides guidance with respect to the installation of doors and windows within heritage assets as follows: ‘The appearance and character of a building will greatly depend on the design and detailing of its walls and its windows and doors. Any alteration to the form of the latter is likely to have a considerable impact upon the overall appearance of the building as a whole.’ In addition, Historic England guidance on converting traditional farm buildings states: 'There should always be a presumption in favour of maximising the use of existing openings without changing their size and limiting the formation of new ones. Where new openings are added or new windows inserted within existing door openings, great care needs to be given to their placing and design'. In this instance, new window openings are proposed for the North-western and South-eastern elevations of the barn building. These would be modestly sized openings serving habitable rooms (bedrooms 1 & 4) and their proportions would be in keeping with the proportionality of the barn’s historic openings therefore the resultant loss of historic building fabric would not be significant. Notwithstanding the aforementioned new openings, the proposed scheme of residential conversion would otherwise be centred around the use of the barn’s historic openings. In addition, all windows and door frames would be finished in a timber detailing which would be wholly appropriate for the agricultural character of the building and rural setting of the site. Furthermore, it is stated that all new window frames would be set well back into the existing and new external openings, with the new glazed timber frame to be installed within the main cart opening also set back to expose the full depth of the barn’s original stone wall. As such, the cumulative treatment of the barn’s openings would be respectful to the heritage asset overall and is therefore considered to be acceptable, subject to the provision further details with respect to glazing detail, moulding finish and method of opening, fixing and finish. Replacement of existing rainwater goods New rainwater goods are to be installed on the barn to replace all missing and defective gutters and downpipes which were observed to be in a somewhat dilapidated condition during the case officer’s site visit. The replacement rainwater goods would consist of half-round gutters and round downpipes which would comprise a cast iron based design detailed in a quality black heritage paint finish which in turn would deliver a noticeable visual enhancement to the external profile of the heritage asset. In terms of the practicalities of installing the new rainwater goods, the application’s methodology statement indicates that the new gutters would be installed to the eaves of the barn using drive in rise and fall brackets installed within mortar joints, with the replacement downpipes mechanically fixed to the external walling in mortar joints where possible. This approach to installation would minimise potential disturbances to the existing building fabric and as such is considered to be appropriate. Accordingly, the replacement of the building’s existing rainwater goods is considered to be acceptable, subject to the provision of further details in the form of samples or detailed specifications of the new cast iron rainwater goods to be installed, method(s) of fixing and further details of the proposed paint finish. Re-rendering of South-western elevation of barn with lime based render & re-pointing of external walls with lime based mortar In this instance, the South-western gable end of the barn’s South-western component has historically been finished in a cement render, with the existing cement render being in a largely defective state as shown in photographic evidence within the application’s Historic Buildings Appraisal and also as observed on site by the application case officer. As such, the defective cement render is to be removed, with the South-western gable end of the barn’s South-western component to be subsequently re-rendered with a lime based render. In a similar vein, the existing stone walls of the barn would be cleaned down before being re-pointed a lime based mortar. The proposed use of a lime based render and mortar for the works of re-rendering and re-pointing respectively would allow for more breathability to the elevations of the barn building which in turn would prevent potential future occurrences of damp to the heritage asset. As such, the proposed works of re-rendering and re-pointing are considered to be acceptable, subject to the provision of further details in the form of a sample of the new mortar/pointing work prior to the commencement of the re-pointing works. Existing roof of barn and attached shippon to be re-slated / replaced Guidance on Alterations to Listed Buildings (IHBC, 2021) provides specific guidance with respect to works of repair to roofs as follows: ‘A building derives much of its character and appearance from its roof. Roofs often contain the oldest and least altered parts of a building…alterations should respect what survives in its original form… existing slates can usually be reused…the shortfall should, if possible, be made up from new material of the same or closely similar geological type.’ In this instance, it is understood that the existing stone roof flags in place on the South-western end of the barn have resulted in structural issues with this section of the barn’s roof due to their loading. As such, the existing stone flags are to be removed and replaced with natural slates, with an identical treatment also proposed for the existing corrugated fibre cement roof on the barn’s shippon component. The use of natural slate roof tiles would be in keeping with the materiality of the barn’s main roof profile (which currently comprises of blue slate tiles) and is therefore considered to be acceptable. In addition, the roof of the main barn would be stripped and re-slated through the use of the building’s existing roof slates which would preserve the existing building fabric of the heritage asset. Furthermore, the application’s methodology statement indicates that the existing blue slates within the main roof of the barn would be carefully stripped, stacked by size and securely stored ready for re-use which in turn would both afford sufficient protection to the existing roof slates and reduce the need to utilise any new roof slates. Consequently, the re-slating and replacement of the building’s existing roofing materials is considered to be acceptable, subject to the provision of further details in the form of samples or detailed specifications of any replacement roof slates, and on the basis of any replacement roof slates matching the existing roof slates in appearance, size and quality. Installation of new roof lights within existing roof light openings of main barn and attached shippon Making changes to heritage assets (Historic England, 2016) states: ‘The insertion of new elements such as doors and windows, (including dormers and roof lights to bring roof spaces into more intensive use) is quite likely to adversely affect the building’s significance…New elements may be more acceptable if account is taken of the character of the building, the roofline and significant fabric.’ In this instance, the converted barn building would accommodate several roof light openings however all of the new roof lights would be installed within the existing roof light openings of the main barn and attached shippon and as such would not read as incongruous features within the roofspace of the barn building. Furthermore, all roof lights would comprise a conservation style design with a recessed and flush fitting which in turn would ensure minimal visual impact and seamless integration with the roof profile of the historic farm building. Moreover, utilisation of the barn’s existing roof light openings would significantly minimise any disturbance to the historic roof fabric of the heritage asset (in the case of the roof of the main barn). Accordingly, the proposed installation of roof lights to the barn building is considered to be acceptable, subject to the provision of further details with respect to elevational and section details, and on the basis of the proposed roof lights comprising a conservation style design with a recessed and flush fitting. Installation of flues to North-western and South-eastern roof slopes of barn and attached shippon respectively Guidance on Alterations to Listed Buildings (IHBC, 2021) states: ‘External flues (including balanced flues) should not be sited on the front elevation of a building. It may be possible for this to be accommodated on a rear elevation but should have a non-reflective surface finish.’ The proposal involves the installation of flue features to the main barn and its attached shippon. The applicant was asked to give consideration towards reducing the height of the flue features proposed for the barn building in order to ensure a more visually sympathetic development however these changes could not be accommodated due to the constraints of building regulations requirements. Notwithstanding this, the proposed flue features are not considered to be overtly unsympathetic or incongruous features and these would be sited on the least visually prominent roof slopes of the main barn / attached shippon, with all flues detailed in a discreet matt black finish, as specified in the above guidance. The incorporation of these features in the proposed scheme of residential conversion is therefore not considered to be unduly harmful to the architectural interest of the heritage asset and as such is considered to be acceptable. Reconfiguration of the barn’s existing internal layout Given the nature of the proposal, the proposed residential conversion of the heritage asset would require deviation from the existing plan form of the building. Notwithstanding this, it is noted within the application’s Historic Building Appraisal that there is little evidence remaining within the interior of the main barn to allow for an accurate interpretation of the building’s historic interior layout or past uses of these spaces due to past changes and alterations which have likely been implemented due to the requirements of modern farming practices and livestock welfare standards. Furthermore, the proposed retention of full height voids to the proposed ground floor entrance hall and kitchen would retain some of the openness of the building’s interior and the overall quantity of subdivision proposed to the interior of the heritage asset is not considered to be unduly disproportionate in light of the residential use proposed for the building. The proposed works of subdivision and subsequent alteration to the building’s plan form are therefore not considered to be unduly harmful to the historic interest of the heritage asset and as such are considered to be acceptable. Assessment of harm to the heritage asset As previously conveyed, creation of the new window openings proposed for the North-western and South-eastern elevations of the barn building would result in the loss of historic building fabric however given the modest proportions of the window openings, the resultant loss of historic building fabric would not be significant therefore these works would carry a low level of harm to the archaeological interest of the heritage asset. In addition, the height of the proposed flue features is considered to be less than desirable in the context of a historic farm building however as previously conveyed, these would be sited on the least visually prominent roof slopes of the main barn / attached shippon, with all flues detailed in a discreet matt black finish to reduce their visual impact. As such, the addition of these features would carry a low level of harm to the architectural interest of the heritage asset. The proposed residential conversion of the heritage asset would require deviation from the existing plan form of the building however as outlined above, the building’s historic plan form retains limited legibility and the overall quantity of subdivision proposed to the interior of the heritage asset is not considered to be unduly disproportionate in this instance. The harm arising to the historic interest of the heritage asset from the proposed works of subdivision and subsequent alteration to the building’s plan form would therefore be minimal. Accordingly, the cumulative level of harm to the significance of the heritage asset from the aforementioned aspects of the proposal is considered to be less than substantial in this instance. Paragraph 215 of the NPPF states: ‘Where a development proposal will lead to less than substantial harm to the significance of a designated heritage asset, this harm should be weighed against the public benefits of the proposal including, where appropriate, securing its optimum viable use.’ In this instance, it is considered that the proposed works and re-use of the barn building as a dwellinghouse is in the public interest in as much that this would prevent further deterioration of a heritage asset which contributes to the visual amenities of the surrounding rural landscape and local history of the area. Further limited public benefits would be derived from the proposal through the use of short term contractor employment during the construction phase of the development. Taking account of the above, it is considered that the proposed development as a whole would deliver public benefits that would outweigh the less than substantial harm identified. Accordingly, the proposed development would therefore satisfy the requirements of Section 16 of the Listed Buildings and Conservation Areas) Act 1990, Paragraphs 212 and 215 of the NPPF and Key Statement EN5 and Policy DME4 of the Ribble Valley Core Strategy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposed scheme of residential conversion would deliver an enhancement to the character and appearance of the heritage asset and the public benefits arising from proposal are considered to outweigh the less than substantial harm identified. As such, for the above reasons and having regard to all material considerations and matters raised that the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That Listed Building Consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |