| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: |  | Officer: | BT |  |  |  | Date: |  | 18/11/24 |  | Manager: |  | LH | Date: | 20/11/24 |
| Application Ref: |  |  |  |  | 3/2024/0755 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  |  | 31/10/24 |  |  | Site Notice: |  | 31/10/24 |  |  |  |  |  |
| Officer: |  |  |  |  | BT |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  |  | REFUSAL |  |  |  |
| Development Description: |  |  |  |  |  | Planning Permission for proposed alterations to wall adjacent to driveway and installation of three air source heat pumps. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  |  | Waddington Hall, Clitheroe Road, Waddington, BB7 3HP. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| Waddington Parish Council: |  |  |  |  |  | Consulted 23/10/24 – no response received. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Archaeology: |  |  |  |  |  | No objections. |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  |  | No objections. |  |  |  |  |  |  |  |  |  |
| RVBC Environmental Health: |  |  |  |  |  | Recommended for refusal of the proposal due to concerns in relation to noise levels from the proposed air source heat pumps and the subsequent impact of this upon the neighbouring residents of Wadda Cottage. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| None. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable Development Key Statement EN5: Heritage Assets Policy DMG1: General Considerations Policy DMG2: Strategic Considerations Policy DMG3: Transport and Mobility Policy DME4: Protecting Heritage Assets Policy DME5: Renewable Energy Policy DME6: Water Management Policy DMH5: Residential And Curtilage Extensions Planning (Listed Buildings and Conservation Areas) Act Section 16, 66 & 72 National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2024/0756: Listed Building Consent for proposed alterations to wall adjacent to driveway and installation of three air source heat pumps (Ongoing) 3/2024/0789: Approval of details reserved by condition 7 (alterations to west facing elevation of garage/home gym) and 9 written Scheme of Investigation of Listed Building Consent 3/2022/1144 (Ongoing) 3/2024/0790: Approval of details reserved by condition 5 (alterations to west facing elevation of garage/home gym) and 6 (written Scheme of Investigation) of Planning Permission 3/2022/1143 (Ongoing) 3/2022/1144: Application for Listed Building Consent for proposed external and internal alterations including the replacement of a conservatory with a new extension. Resubmission of 3/2022/0889 (Approved) 3/2022/1143: Proposed external and internal alterations including the replacement of a conservatory with a new extension. Resubmission of 3/2022/0839 (Approved) 3/2022/0889: Application for Listed Building Consent for proposed external and internal alterations including the replacement of a conservatory with a new extension (Withdrawn) 3/2022/0839: Application for Planning Permission for proposed external and internal alterations including the replacement of a conservatory with a new extension (Withdrawn) 3/1997/0509: Conversion of old service courtyard into indoor swimming pool (modification to 3/96/0011)(listed building consent) (Approved) 3/1997/0508: Conversion of old service courtyard into indoor swimming pool (modification to approved plans 3/96/0010) (Approved) 3/1996/0564: New bay, gable porch and construction of 3 car open cloistered garage and forecourt listed building consent (Approved) 3/1996/0563: Addition of new bay gable and porch to north elevation construction of 3 car open cloistered garage and forecourt (Approved) 3/1996/0011: Change of use from domestic garages into swimming pool, changing rooms etc. (app. For listed building consent) (Approved) 3/1996/0010: Convert existing outbuilding into swimming pool with changing room, steam room etc (Approved) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a detached historic property in Waddington. Access to the property is from Clitheroe Road from both a pedestrian access which leads to the principal South-western frontage of the property and a vehicular access which leads into a cobbled driveway area which adjoins the property’s North-eastern elevation. The application property has been subject to some C20th alterations and extensions including the addition of a conservatory extension, three bay garage extension and the formation of a cobbled driveway and landscaped area bound by a modern stone retaining wall. The property sits within a sizeable curtilage area comprised of manicured lawns, hard landscaping, extensive planting and mature trees. The application property lies within the centre of Waddington within the Waddington Conservation Area with the surrounding character of the area being predominantly residential. The wider area comprises a mixture of woodland, agricultural land and open countryside. Waddington Hall holds Grade II Listed Building status with the official Historic England listing description reading as follows: ‘House, possibly early C17th, rebuilt c.1900. Sandstone rubble with stone slate roof. H-plan. At the west the cross-wings project forward to form a courtyard. Both have gables with copings and the 3 walls facing the courtyard are topped by a balustrade with square balusters. The right-hand cross-wing has buttresses. On the ground floor is a 2-light mullioned window with inner hollow chamfer and outer chamfer. On the 1st floor is an early C18th cross window with an inner chamfer and an outer cyma-moulded surround. Above is a 2-light mullioned attic window of c,1900. All the windows of the left-hand cross-wing appear to be of c.1900. On the ground floor is a 5-light mullioned window with Tudor-arched heads to the lights. On the flat floor is a 10-light mullioned and transomed window. The attic has a 2-light mullioned window. The central section has a 2-light mullioned window with inner ovolo and outer rebate. To its right is a one-light window with moulded surround. On the 1st floor are 2 2-light double-chamfered mullioned windows, re-tooled and with cinquefoiled ogee heads to the lights. These are probably re-used from elsewhere. Across the angle with the right-hand cross-wing is a moulded doorway with Tudor-arched head. Above is a carved coat of arms under a shaped coping. At the rear all the windows appear to be of c.1900, in keeping. The south cross-wing has a 2-storey canted bay window. The south wall of this cross-wing has a C17th mullioned window of 4 lights with inner ovolo and outer chamfer. Interior not fully inspected, but rear of hall has wooden doorway with ogee head, probably C17th.’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Planning consent is sought for the following works: - Partial reconfiguration of an existing retaining driveway wall to create extended driveway and parking area - Installation of three air source heat pumps and construction of new stone wall boundary (using materials from the partly demolished driveway wall) around proposed heat pumps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon Listed Building: Paragraph 205 of the NPPF states: ‘When considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). This is irrespective of whether any potential harm amounts to substantial harm, total loss or less than substantial harm to its significance.’ In addition, Key Statement EN5 of the Ribble Valley Core Strategy states: ‘There will be a presumption in favour of the conservation and enhancement of the significance of heritage assets and their settings.’ Furthermore, Policy DME4 of the Core Strategy states: ‘Alterations or extensions to Listed Buildings or buildings of local heritage interest, or development proposals on sites within their setting which cause harm to the significance of the heritage asset will not be supported.’ Heritage impact is considered to be the potential level of harm upon the significance of a heritage asset caused by development proposals. The NPPF defines significance as ‘the value of a heritage asset to this and future generations because of its heritage interest’. Such interest can be archaeological, architectural, artistic or historic. Statements Of Heritage Significance, Historic England (2019) defines these as follows: ‘Archaeological Interest: There will be archaeological interest in a heritage asset if it holds, or potentially holds, evidence of past human activity worthy of expert investigation at some point.’ ‘Architectural And Artistic Interest: Interests in the design and general aesthetics of a place. They can arise from conscious design or fortuitously from the way the heritage asset has evolved. More specifically, architectural interest is an interest in the art or science of the design, construction, craftsmanship and decoration of buildings and structures of all types. Artistic interest is an interest in other human creative skills, like sculpture’. ‘Historic Interest: An interest in past lives and events (including pre-historic). Heritage assets can illustrate or be associated with them. Heritage assets with historic interest not only provide a material record of our nation’s history but can also provide meaning for communities derived from their collective experience of a place and can symbolise wider values such as faith and cultural identity.’ National Planning guidance requires applicants to describe the significance of any heritage assets affected, including any contribution made by their setting in order to allow the LPA to come to a judgment about the level of impact on that significance and therefore on the merits of the proposal. A heritage statement has been provided in support of the application which summarises the significance of the application building as follows: ‘The archaeological interest of the building is derived from its extant historic fabric and its evidence of historical alteration…the architectural and artistic interest of the building is derived from both its external and internal appearance…the external appearance has been altered to accommodate architectural embellishments such as the eaves balustrading, verge copings, the two-storey rear canted bay window and new window and door openings, as well as additions including the front entrance porch and the addition to the north of the building. The building is an imposing and impressive structure within the site and the wider village streetscape and its stylistic appearance is evidence of its high status and its historical development since its origins…the building’s associations with its past occupants appear to be well documented, particularly, its association with Henry VI, who was briefly hosted at the building.’ Accordingly, the above observations indicate that the significance of the application building is largely underpinned by its architectural interest (architectural embellishments referenced above) along with some additional archaeological interest (extant historic fabric) and historic interest (past associations with royal figures). In terms of justification for the proposed works, the application’s Heritage Statement states that the proposed alterations to the property’s driveway wall would improve the existing parking arrangement on site. It is further stated that installation of the three proposed air source heat pumps would be integral to providing a sustainable future for the application property and improving its energy efficiency. Making changes to Heritage Assets, Historic England (2016) advises: ‘It would not normally be good practice for new work to dominate the original asset or its setting in either scale, material or as a result of its siting.’ Furthermore, with specific regard to the siting of external services and equipment on Listed Buildings, the Institute Of Historic Building Conservation (2021) states: ‘The negative impact of some commonplace external fixtures on the character of a building is often ignored. These include meter boxes, burglar alarms, security lighting, stair lighting sensors, fixed and manoeuvrable video cameras, air source heat pumps, external central heating and other flues (both standard and balanced) and gas supply pipework. Ill-considered proposals for the location of any one of these fixtures or several in combination, particularly on principal facades and on smaller buildings, can have a particularly deleterious impact. They are only appropriate in visually unobtrusive positions where they do not cause damage to existing surface finishes and avoid any significant element of external architectural detail or decoration.’ In this instance, the proposal would involve the partial demolition of the property’s retaining driveway wall (approximately two thirds of the wall) in order to create additional vehicle parking space and to facilitate construction of the new walled area to house the proposed air source heat pumps. The wall to be partly demolished is a modern addition to the property (understood to have been constructed in 2002 around the same time that the property’s swimming pool extension was constructed) of little architectural merit therefore the partial demolition and subsequent reconfiguration of this feature would not result in the loss of any historic building fabric, nor would these works have any undue impact upon the architectural interest of the application property. These works would therefore have a neutral impact upon the significance of the application property. Notwithstanding the above, the application’s supporting information shows that the proposed air source heat pumps would comprise an overtly modern design with each of the units detailed in a grey metallic finish and with large circular fan openings to their front profiles. As such, the overtly modern design and external profile of the proposed heat pumps would be noticeably at odds with the traditional stone based vernacular of the application property and its driveway area. In addition, the air source heat pump unit proposed for the property would comprise a width and height of 1.5 metres and 1.3 metres respectively and would therefore not be insignificant in terms of size and in this instance the proposal seeks to install a total of three such units. Furthermore, whilst the proposed heat pumps would be sited inside the confines of the reconfigured section of the existing driveway wall, each of the units would nonetheless be visible within the confines of this area on approach to the property’s garage extension and swimming pool area, with each of the units also projecting slightly above the reconfigured wall area. As such, and contrary to the guidance above, the proposed air source heat pumps would not read as small scale or discreetly located additions to the property. Whilst it is accepted that these structures would partially be read in concert with the more modern components of the heritage asset, the proposed air source heat pumps, by virtue of their modern design and detailing, size, siting and quantity (in addition to the absence of any other similar modern microregeneration equipment within the site) would nonetheless read as largely incongruous and unsympathetic additions to the site that would be largely at odds with the prevailing historic character of the Listed Building. Taking account of the above, the proposed development would fail to accord with the above guidance and policies in relation to the installation of modern services and protection of heritage assets, to the extent that the proposed air source heat pumps would detract from the historic and architectural character of the Listed Building which in turn would be harmful to the significance of the heritage asset. The level of harm to the heritage asset from the proposed development is considered to be less than substantial in this instance. Paragraph 208 of the NPPF states: ‘Where a development proposal will lead to less than substantial harm to the significance of a designated heritage asset, this harm should be weighed against the public benefits of the proposal including, where appropriate, securing its optimum viable use.’ In this instance, installation of the proposed air source heat pumps would offer limited small scale public benefits in the form of short term contractor employment. It is stated that the proposed works are integral to providing a sustainable future for the heritage asset and improving its energy efficiency however the application’s supporting information offers no further expansion on this assertion. Furthermore, any resultant reduction in carbon emissions from the proposal would likely be small scale. As such, the limited public benefits identified are not considered to outweigh the harm that would occur to the heritage asset from the proposed development in this instance. In addition, the heritage asset provides an existing residential use that would not cease in the event of the proposal failing to be implemented therefore the heritage asset is already considered to be in its optimum viable use. Accordingly, the proposed development fails to satisfy the requirements of Paragraphs 205 and 208 of the National Planning Policy Framework and Key Statement EN5 and Policy DME4 of the Ribble Valley Core Strategy and is therefore considered to be unacceptable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon Character/appearance of Conservation Area: The proposal site is situated within the Waddington Conservation Area. With reference to making decisions on applications for development in Conservation Areas, Section 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990 states that: “...special attention shall be paid to the desirability of preserving or enhancing the character or appearance of that area.” This guidance is reiterated in Key Statement EN5 of the Ribble Borough Valley Core Strategy which stipulates that all development proposals should respect and safeguard the character, appearance and significance of all Conservation Areas. The Waddington Conservation Area Appraisal (2005) identifies the following elements as contributing to the Conservation Area’s special interest: - Coronation Gardens running alongside the Waddington Brook - Meadows and working farms within the heart of the village - Attractive footpaths that follow the village’s numerous water courses - Numerous historic buildings Northwards views towards St. Helen’s Church and Waddington Methodist Church and Southward views from the junction of West Bradford Road and Branch Road are denoted as Key Views on the Waddington Conservation Area Map. Threats to the Conservation Area are listed as the continuing loss of original architectural details and use of inappropriate modern materials or details. In this instance, and as conveyed above, it considered that the proposed air source heat pumps would read as unsympathetic additions to the application site that would be largely at odds with the prevailing historic character of the Listed Building. Notwithstanding this, the proposed air source heat pumps would only be readily visible from within the confines of the property’s curtilage and would therefore have little to no impact upon the setting of the Conservation Area. In addition, the proposed microgeneration equipment would not be read in concert with any of the area’s elements of special interest listed above, nor would the air source heat pumps be read in concert with any of the area’s key views as referenced on the Waddington Conservation Area Townscape Appraisal Map. In light of the above, it is not considered that the proposed development would have any undue impact upon the character and appearance of the Waddington Conservation Area. As such, the proposed development would satisfy the requirements of Key Statement EN5 and Policy DME4 of the Ribble Valley Core Strategy and Section 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact Upon Residential Amenity: Paragraph 135 (f) of the National Planning Policy Framework states: ‘Planning policies and decisions should ensure that developments create places that are safe, inclusive and accessible and which promote health and well-being, with a high standard of amenity for existing and future users’. In addition, Paragraph 191 (a) of the National Planning Policy Framework states: ‘Planning policies and decisions should also ensure that new development is appropriate for its location taking into account the likely effects (including cumulative effects) of pollution on health, living conditions and the natural environment, as well as the potential sensitivity of the site or the wider area to impacts that could arise from the development…in doing so they should mitigate and reduce to a minimum potential adverse impacts resulting from noise from new development – and avoid noise giving rise to significant adverse impacts on health and the quality of life.’ Additional national planning guidance with respect to the management of potential noise impacts from new development advises: ‘Noise needs to be considered when development may create additional noise, or would be sensitive to the prevailing acoustic environment…good acoustic design needs to be considered early in the planning process to ensure that the most appropriate and cost-effective solutions are identified from the outset. Plan-making and decision making need to take account of the acoustic environment and in doing so consider: - whether or not a significant adverse effect is occurring or likely to occur; - whether or not an adverse effect is occurring or likely to occur; and - whether or not a good standard of amenity can be achieved. Furthermore, Policy DMG1 of the Core Strategy requires all proposals for development to consider the effects of development upon existing amenities. Moreover, Policy DME5 states: ‘The Borough Council will support the development of renewable energy schemes, providing it can be shown that such developments would not cause unacceptable harm to the local environment or local amenity.’ In this instance, the proposed air source heat pumps would be sited directly on the property’s Northern common boundary wall shared with the neighbouring property of Wadda Cottage, with the air source heat pumps carrying noise obvious implications when operational therefore consultation has been undertaken with the Council’s Environmental Health team. The subsequent response from RVBC Environmental Health states that sound power level calculations for the units show that the collective noise level from the three units proposed would be excessively high in this instance, to the extent that the units when operational would unduly impact upon the amenity of the neighbouring occupants of Wadda Cottage. Furthermore, the response from RVBC Environmental Health makes reference to the absence of any data within the application submission with regards to barrier attenuation calculations, in addition to the absence of any measures to control noise emissions (acoustic enclosures) or information to demonstrate that consideration has been given to the use of quieter air source heat pump units. Consequently, the Council’s Environmental Health team have recommended for refusal of the proposed development. In light of the above assessment, the proposed development is considered to be unacceptable on the basis that the noise generation from the proposed air source heat pumps would unduly impact upon the amenity of the neighbouring occupants of Wadda Cottage. The proposal would therefore be in conflict with the aims and objectives of Paragraphs 135 (f) and 191 (a) of the NPPF and Policies DMG1 and DME5 of the Core Strategy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity/External Appearance: Paragraph 135 (c) of the NPPF states: ‘Planning policies and decisions should ensure that developments are sympathetic to local character and history, including the surrounding built environment and landscape setting’. In addition, Policy DMG1 of the Ribble Valley Core Strategy states: ‘All development must be sympathetic to existing and proposed land uses in terms of its size, intensity and nature as well as scale, massing and style…particular emphasis will be placed on visual appearance and the relationship to surroundings, including impact on landscape character.’ In this instance, the proposed air source heat pumps would be sited within the Northern extent of the property’s driveway area. The property’s driveway area is enclosed by a stone boundary wall on its Northern side (which appears to be historic), with a more modern linear stone based extension comprised of three large distinctive archways forming the driveway’s Eastern boundary. Whilst this is a more recent addition to the property (approved under applications 3/1996/0563 & 3/1996/0564), this work has nonetheless been carried out to a high standard and as such largely reflects the historic character of the application property. The aforementioned linear extension subsequently adjoins to the more historic North-western profile of the application property which holds a greater level of architectural merit. As such, the historic theme of the application property and its surrounding grounds has been largely retained within the property’s driveway area. The proposed development seeks to introduce air source heat pumps to the Northern extents of the property’s driveway area and these would comprise an overtly modern design with each of the units detailed in a grey metallic finish and with large circular fan openings to their front profiles. As such, the overtly modern design and external profile of the proposed heat pumps would be noticeably at odds with the traditional stone based vernacular of the application property and its driveway area. Furthermore, the air source heat pumps would read as largely anomalous additions to the site by virtue of the absence of any other modern fixtures and fittings within this area of the application site. In addition, the air source heat pump unit proposed for the property would comprise a width and height of 1.5 metres and 1.3 metres respectively and would therefore not be insignificant in terms of size and in this instance the proposal seeks to install a total of three such units. Furthermore, whilst the proposed heat pumps would be sited inside the confines of the reconfigured section of the existing driveway wall, each of the units would nonetheless be visible within the confines of this area on approach to the property’s garage extension and swimming pool area, with each of the units also projecting slightly above the reconfigured wall area. As such, the proposed air source heat pumps would not read as small scale or discreetly located additions to the property. As such, the proposed air source heat pumps, by virtue of their modern design and detailing, size, siting and quantity would read as unsympathetic, incongruous and harmful additions to the host property. The proposal therefore fails to satisfy the requirements of Paragraph 135 (c) of the NPPF and Policy DMG1 of the Core Strategy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways and Parking: The proposed development has been subject to review from Lancashire County Council Highways who in their response acknowledge that the proposed alterations to the property’s driveway would not significantly change the level of off street parking provisions within the site or result in any changes to the site’s visibility splays. As such, the LHA have raised no issues with the proposed development. The proposal would therefore satisfy Policy DMG1 of the Core Strategy (highways). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: Trees A tree (species unknown) lies within close proximity to the Western end of the property’s retaining driveway wall however analysis shows that the tree in question would not be affected by the proposed alterations to the Eastern half of the driveway wall. As such, the proposal raises no concerns with respect to impacts upon trees. BNG The development is exempt from having to achieve the mandatory Biodiversity Net Gain requirement as it is a householder application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Matters: Flood Risk Analysis shows the area of proposed development within the application site as lying within Flood Zone 2. National planning legislation requires the provision of a Flood Risk Assessment for all development within Flood Zones 2 and 3, with sequential tests required for developments within Flood Zones 2 & 3 which do not fall within the following categories: - domestic extensions with a footprint of less than 250 square metres - non-domestic extensions with a footprint of less than 250 square metres - change of use (except changes of use to a caravan, camping or chalet site, or to a mobile home or park home site) The proposed development comprises microregeneration units with a collective footprint of less than 250 square metres in area therefore application of the sequential test is not required in this instance. National planning legislation requires the provision of a Flood Risk Assessment for all development within Flood Zones 2 and 3 however no flood risk assessment has been provided in support of the application in this instance. Notwithstanding this, it is not anticipated that the proposed development would exacerbate occurrences of flooding within the site or elsewhere and the submitted drawings and product brochure for the heat pumps indicates that the units would be set on platforms above the ground level. As such, the proposal raises no significant concerns with respect to flood risk however a flood risk assessment (including flood risk mitigation measures) would need to be provided in support of any future planning application submission. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposed development is considered to be unacceptable on the basis that the noise generation from the proposed air source heat pumps would unduly impact upon the amenity of the neighbouring occupants of Wadda Cottage. Furthermore, the proposed air source heat pumps, by virtue of their modern design and detailing, size, siting and quantity would read as unsympathetic, incongruous and harmful additions to the host property. Moreover, Paragraph 139 of the National Planning Policy Framework states: ‘Development that is not well designed should be refused, especially where it fails to reflect local design policies and government guidance on design’. As such, for the above reasons and having regard to all material considerations and matters raised that the application is recommended for refusal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  | That planning permission be refused for the following reason: |  |  |  |  |  |  |  |  |  |  |  |
| 01: | The proposed development is considered to be unacceptable on the basis that the noise generation from the proposed air source heat pumps would adversely impact upon the amenity of the neighbouring occupants of Wadda Cottage. The proposal would therefore be in conflict with the aims and objectives of Paragraphs 135 (f) and 191 (a) of the NPPF and Policies DMG1 and DME5 of the Ribble Valley Core Strategy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 02: | The proposed air source heat pumps, by virtue of their modern design and detailing, size, siting and quantity would have a harmful impact upon the character and appearance of the application property, a Grade II Listed Building. As such, the proposal would fail to meet the requirements of Key Statement EN5 and Policies DME4 and DMG1 of the Ribble Valley Core Strategy, Sections 16 and 66 of the Planning (Listed Buildings and Conservation Areas) Act 1990 and Paragraphs 135 (c), 205 and 208 of the National Planning Policy Framework. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |