| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | LW |  |  |  | Date: |  | 13/02/25 |  | Manager: |  | KH | Date: | 14/02/25 |
| Application Ref: |  |  |  | 3/2024/0960 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 20/01/25 |  |  | Site Notice: |  | 20/01/25 |  |  |  |  |  |
| Officer: |  |  |  | LW |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Proposed single storey extension to rear. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 61 Knowsley Road, Wilpshire, BB1 9PN. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| A consultation response from Wilpshire Parish Council was received on 23rd January 2025 stating that the Parish Council has no comments to make on the application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| No representations have been received with respect to the proposed development. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable Development Policy DMG1: General Considerations Policy DMG2: Strategic Considerations National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: No relevant planning history. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a two-storey, semi-detached dwelling at no.61 Knowsley Road. The property comprises pebble dash render and brickwork to the external elevations, slate roof tiles and uPVC windows and benefits from an existing flat roof, single storey rear extension. The site to which the proposal relates is located within the defined settlement area of Wilpshire and the property benefits from no other designations or constraints. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for the construction of a single storey extension to the rear. The proposed extension would project 3.8m from the rear elevation of the existing single storey rear extension and 6.5m from rear elevation of the main dwellinghouse, with a width of 8.8m. A flat roof form would be incorporated measuring 3m in height, while a set of bi-fold doors and full height glazing would be featured to the rear elevation, along with 1no. window to the southern facing side elevation. As part of the overall development, the existing personnel door featured to the northern facing side elevation of the existing rear extension would also be replaced by a single window opening. With respect to materiality, the proposed development would be finished in render and uPVC windows and doors. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: The proposal relates to a domestic extension to an established residential property and is therefore acceptable in principle, subject to an assessment of the material planning considerations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact Upon Residential Amenity: The proposal would result in an extension with a total rearward projection of 6.5m located adjacent the common boundary with no.59 Knowsley Road. Despite this, the proposal would project approximately 3.5m beyond the rear of the existing extension featured to no.59 Knowsley Road. Taking account of the above and given the extension would also be sited to the north of no.59 Knowsley Road, it is not anticipated that the proposed development would result in any significant detrimental harm upon these neighbouring residents by way of overshadowing, loss of outlook or daylight, that would warrant the refusal to grant planning permission. With regard to no.63 Knowsley Road, the proposed extension would not project beyond the existing structure featured to the rear of this neighbouring property. The proposal site is also situated on lower ground compared to no.63 Knowsley Road, and therefore it is also not anticipated that any significant loss of light would be resultant in this respect. Furthermore, the openings proposed to the rear of the extension would provide views solely towards the rear garden area of the application property and therefore no new opportunities for direct overlooking or loss of privacy would be created as a result of the works proposed. The window to the south facing side elevation of the extension would provide views towards no.59 Knowsley Road; however, this window would replace an existing obscurely glazed window featured to the side elevation of the existing single storey extension and would not have a direct interface with any neighbouring habitable room windows. Nevertheless, a condition has been attached to accompanying decision notice to ensure that the opening remains obscurely glazed in order to protect the amenity of the neighbouring residents. The new window proposed to the northern side elevation of the existing extension, facing towards no.63 Knowsley Road, would also be adequately screened by the existing boundary treatment Accordingly, it is not anticipated that the proposed development would result in any significant adverse harm upon the existing amenities of any nearby residents. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity/External Appearance: The proposed development would be sited to the rear of the application property and would therefore not be afforded a high level of visibility from the adjacent public realm. Nevertheless, the proposal would not read as an overtly incongruous or over dominant addition to the proposal site and would sufficiently integrate with the visual appearance of existing built form of the application property. As such, the proposed development would not result in any significant detrimental impact upon the existing visual amenities of the application property or surrounding area that would warrant the refusal of the application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways and Parking: Lancashire County Council Highways have not been consulted on the proposed development; however, the proposal would not result in an increase in the number of bedrooms at the site, nor are any alterations to the existing parking arrangements or site access proposed. As such, the proposal is considered acceptable with respect to highway safety and parking. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: No ecological constraints have been identified with respect to the proposal. The development is exempt from having to achieve the mandatory Biodiversity Net Gain requirements as it is a householder application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: As such, for the above reasons and having regard to all material considerations and matters raised, the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That planning consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |