| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | LW |  |  |  | Date: |  | 15/01/25 |  | Manager: |  | KH | Date: | 15/01/25 |
| Application Ref: |  |  |  | 3/2024/0965 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | N/A |  |  | Site Notice: |  | N/A |  |  |  |  |  |
| Officer: |  |  |  | LW |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Certificate of Lawfulness for proposed construction of a single storey extension to rear; formation of new/extension of existing first floor rear dormer; partial conversion of existing garage, to retain part of garage as a bike store. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 25 Hacking Drive, Longridge, PR3 3EP. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Schedule 2 Part 1 Class A and B of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: No relevant planning history. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a semi-detached dormer bungalow property at no.25 Hacking Drive, comprising brickwork to the external elevations and concrete roof tiles. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for a Certificate of Lawfulness for the construction of a single storey rear extension and new rear dormer, as well as the partial conversion of the existing garage. The proposed rear extension would project 3m from the rear elevation of the application property with a width of 5.6m. A reverse gable roof form would be incorporated measuring 2.4m to the eaves and 3.8m to the ridge. The proposed rear dormer would project 2.7m from the roof slope of the main dwellinghouse with a length of 7.3m. A lean-to roof form would be featured, with an eaves and ridge height of 1.4m and 2.5m respectively. With respect to materiality, the proposed rear extension would be finished in brickwork and concrete roof tiles, while the proposed dormer would incorporate grey uPVC cladding and concrete roof tiles. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Matters: Assessment of the proposal in relation to the provisions of Schedule 2 Part 1 Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). In order to be permitted development, the proposed development needs to satisfy a number of criteria as comprised in Schedule 2 Part 1 Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended) for the enlargement, improvement or other alteration of a dwellinghouse. A.1 Development is not permitted by Class A if – a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class G, M, MA, N, P, PA or Q of Part 3 (change of use); Permission to use the dwellinghouse as a dwellinghouse has not been granted only by virtue of Class G, M, MA, N, P, PA or Q of Part 3. b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of curtilage (excluding the ground area of the original dwellinghouse); The total area of ground covered by buildings would not exceed 50% of the total area of the curtilage of the dwellinghouse. c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse; The height of the proposed extension would not exceed the height of the existing dwellinghouse. d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse; The eaves height of the proposed extension would not exceed the eaves height of the existing dwellinghouse. e) the enlarged part of the dwellinghouse would extend beyond a wall which – (i) forms the principal elevation of the original dwellinghouse, or (ii) fronts a highway and forms a side elevation of the original dwellinghouse; The proposed extension would not extend beyond a wall which forms the principal elevation or fronts a highway and forms a side elevation of the dwellinghouse. f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 4m in the case of a detached dwellinghouse, or 3m in the case of any other dwellinghouse, or (ii) exceed 4m in height; The proposed extension would not extend more than 3m from the rear wall of the dwellinghouse. g) for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 8m in the case of a detached dwellinghouse, or 6m in the case of any other dwellinghouse, or (ii) exceed 4m in height; The dwellinghouse is not on article 2(3) land nor on a site of special scientific interest and the proposal would not involve a larger home extension. h) the enlarged part of the dwellinghouse would have more than a single storey and – (i) extend beyond the rear wall of the original property by more than 3m, or (ii) be within 7m of any boundary of the curtilage of the dwellinghouse being enlarged which is opposite the rear wall of that dwellinghouse; The proposed extension would not have more than a single storey. i) the enlarged part of the dwellinghouse would be within 2m of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3m; The proposed extension would be within 2m of the boundary of the curtilage; however, the eaves height would not exceed 3m. j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse and would – (i) exceed 4m in height, (ii) have more than a single storey, or (iii) have a width greater than half the width of the original dwellinghouse; The proposed development would not extend beyond a wall forming a side elevation of the original dwellinghouse. ja) any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraph e) to j); N/A k) it would consist of or include – (i) the construction or provision of a verandah, balcony or raised platform, (ii) the installation, alteration, or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse; or The proposed development would include none of the above. l) the dwellinghouse is built under Part 20 of this Schedule (construction of new dwellinghouses) The dwellinghouse is not built under Part 20 of this Schedule. A.2 In the case of a dwellinghouse on article 2(3) land, development is not permitted by Class A if – a) it would consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles; b) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse; c) the enlarged part of the dwellinghouse would have more than a single storey and extend beyond the rear wall of the original dwellinghouse; or ca) any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs b) and c). The dwelling is not sited on article 2(3) land. A.3 Development is permitted by Class A subject to the following conditions – a) the materials used in any exterior work (other than materials used in the construction of a conservatory) must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; The external elevations of the proposed extension would be finished in brickwork and concrete roof tiles to match the existing dwellinghouse. b) any upper-floor windows located in a wall or roof slope forming a side elevation of the dwellinghouse must be – (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7m above the floor of the room in which the window is installed; and No first-floor windows in a wall or roof slope forming a side elevation are proposed. c) where the enlarged part of the dwellinghouse has more than a single storey or forms an upper storey on an existing enlargement of the original dwellinghouse, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse. The proposed extension would not have more than a single storey. Assessment of the proposal in relation to the provisions of Schedule 2 Part 1 Class B of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). In order to be permitted development, the proposed development needs to satisfy a number of criteria as comprised in Schedule 2 Part 1 Class B of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended) for the enlargement of a dwellinghouse consisting of an addition or alteration to its roof. B.1 Development is not permitted by Class B if – a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class G, M, MA, N, P, PA or Q of Part 3 of this Schedule (changes of use); Permission to use the dwellinghouse has not been granted by virtue of Class G, M, MA, N, P, PA or Q of Part 3. b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; The proposed development would not exceed the height of the existing roof. c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; No part of the proposed development would extend beyond the plane of any existing roof slope which forms the principal elevation and fronts a highway. d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than – (i) 40 cubic metres in the case of a terrace house, or (ii) 50 cubic metres in any other case; The cubic content of the resulting roof space would exceed the cubic content of the original roof space by approximately 25 cubic metres. e) it would consist of or include – (i) the construction or provision of a verandah, balcony or raised platform, or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; The proposed development would not consist of or include any of the above. f) the dwellinghouse is on article 2(3) land; The dwellinghouse is not on article 2(3) land. g) the dwellinghouse is built under Part 20 of this Schedule (construction of new dwellinghouses), or The dwellinghouse is not built under Part 20. h) the existing dwellinghouse has been enlarged in reliance on the permission granted by Class AA (enlargement of a dwellinghouse by construction of additional storeys). The existing dwellinghouse has not been enlarged in reliance on the permission granted by Class AA. B.2 Development is permitted by Class B subject to the following conditions – a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; The Householder Technical Guidance ensures that “any addition or alteration to a roof for a loft conversion results in an appearance that minimises visual impact and is sympathetic to the existing house. This means that the materials used should be of similar visual appearance to those in the existing house but does not mean that they need to be same materials or match exactly”. As a result, the proposed grey uPVC cladding to the dormer walls would be considered acceptable insofar that the material would appear to be of similar colour to the materials used in the main roof of the house. b) the enlargement must be constructed so that – (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof a rear side extension – (aa) the eaves of the original roof are maintained or reinstated; and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2m from the eaves, measured along the roof slope from the outside edge of the eaves; and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; and The eaves of the original roof would be maintained or reinstated, and the edge of the proposed dormer would be more than 0.2m from the eaves. The proposed development would also not extend beyond the outside face of any external wall of the original dwellinghouse. c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be – (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7m above the floor of the room in which the window is installed. No windows are proposed to a wall or roof slope forming a side elevation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposed development constitutes permitted development under Schedule 2 Part 1 Class A and Class B of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), subject to the conditions outlined in A.3 and B.2 of this Part. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That the Certificate of Lawfulness be granted. |  |  |  |  |  |  |  |  |  |  |  |