| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | EP |  |  |  | Date: |  | 19/12/2024 |  | Manager: |  | LH |  | Date: | 19/12/24 |
| Application Ref: |  |  | 3/2024/0977 |  |  |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  | N/A |  |  |  | Site Notice: |  | N/A |  |  |  |  |  |  |
| Officer: |  |  | EP |  |  |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | Decision |  | APPROVAL |  |  |
| Development Description: |  |  |  |  | Prior notification of proposed multi-purpose agricultural building 36.6m long, 20m wide, 7m high to eaves, 9.78m high to ridge under Part 6 Class A of the GPDO. |  |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | Laneside Farm, Grindleton Road, West Bradford BB7 4QJ. |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |  |
| West Bradford Parish Council have sent a letter of representation raising the following points - The main business running from the farm is not of a traditional farming nature. - The livestock are not owned by the applicant. - Concerns over the scale of the business and buildings. - Large building not in keeping with the local environment. - Concerns over work carried out without consent. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |  |
| One letter of representation has been received raising the following concerns - Concerns over the use of the buildings in relation to the commercial enterprise at the site. - Impact of the development on outlook and the character of the neighbourhood. - Increase in noise and disturbance. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Schedule 2, Part 6, Class A of the Town and Country Planning (General Permitted Development) Order 2015 and (Amendment) Order 2018 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2020/0256: Construction of multi-purpose agricultural building. (permission not required). 3/2019/0534: One new building to provide farm office, workshop and store. (approved with conditions). # 3/2017/0614- Discharge of condition 3 (materials) from planning permission 3/2017/0397. (Approved with Conditions) # 3/2017/0397- Proposed extension to an existing agricultural machinery storage building. (Approved with Conditions) # 3/2007/1118- Erection of a portal framed agricultural building for general purpose and lambing shed. (Refused) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application site is located within the farm yard associated with Laneside Farm located on the southern side of Grindleton Road being outside the settlement boundary of West Bradford. The opposing northern side of Grindleton Road delineates the southern boundary of the National Landscape, the site to which the application relates is located within the defined open countryside. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: The application seeks to determine whether the proposed agricultural building falls under the real of permitted development, or if full planning consent is required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Whether or not permitted development The scheme must satisfy a number of criteria as set out under Class A of Part 6 of Schedule 2 of the Town and Country Planning (General Permitted Development) Order 2015 (as amended 2018). The first of those requirements is that the development must be ‘carried out on an agricultural unit of 5 hectares or more’ and be ‘reasonably necessary for the purposes of agriculture within that unit’. The agricultural holding is 113 hectares in area. The proposal is for the construction of an agricultural building for the purposes of housing extensive farm machinery in order to protect them from the elements. The applicant has provided supporting information which outlines the variety of machinery owned and used at the farm, including various tractors, diggers and trailers. It also outlines the significant number of livestock owned which includes 450 sheep, 700 lambs and a smaller number of cows. It is noted that representations have been received raising concerns over the use of the site and highlighting a commercial use which appears to be operating from the farm office on the site without consent. However, it is evident that there is a substantial and established agricultural enterprise at this site, in accordance with the requirements of the GPDO. Therefore, an additional unauthorised use, would not form basis for refusal. Accordingly, the proposed development is considered to be reasonably necessary for the purposes of agriculture in this instance. Having regard to criteria a) – k), development is not permitted by Class A if – (a) the development would be carried out on a separate parcel of land forming part of the unit which is less than 1 hectare in area; The development would be carried out on a parcel of land greater than 1 hectare in area. (b) it would consist of the erection or extension of any agricultural building on an established agricultural unit (as defined in paragraph X of Part 3 of this Schedule) where development under Class Q or S of Part 3 (changes of use) of this Schedule has been carried out within a period of 10 years ending with the date on which development under Class A(a) begins; No development under Class Q or Class S has been carried out in the last 10 years. (c) it would consist of, or include, the erection, extension or alteration of a dwelling; The proposal does not consist of the alteration of a dwelling. (d) it would involve the provision of a building, structure or works not designed for agricultural purposes; The proposed building is designed for the storage of agricultural machinery utilised in association with the farm enterprise. (e) the ground area which would be covered by— (i) any works or structure (other than a fence) for accommodating livestock or any plant or machinery arising from engineering operations; or (ii) any building erected or extended or altered by virtue of Class A, would exceed 1000 square metres, calculated as described in paragraph D.1(2)(a) of this Part; The proposed building would measure less than 1000 square metres in area. (f) the height of any part of any building, structure or works within 3 kilometres of the perimeter of an aerodrome would exceed 3 metres; The proposed building is not within 3km of an aerodrome. (g) the height of any part of any building, structure or works not within 3 kilometres of the perimeter of an aerodrome would exceed 12 metres; The building will measure 9.78 metres in height to the ridge. (h) any part of the development would be within 25 metres of a metalled part of a trunk road or classified road; The proposed building is not within 25 metres of a classified road. (i) it would consist of, or include, the erection or construction of, or the carrying out of any works to, a building, structure or an excavation used or to be used for the accommodation of livestock or for the storage of slurry or sewage sludge where the building, structure or excavation is, or would be, within 400 metres of the curtilage of a protected building; The proposed building is not for the purposes of housing livestock or slurry. (j) it would involve excavations or engineering operations on or over article 2(4) land which are connected with fish farming; or The development is not connected to fish farming. (k) any building for storing fuel for or waste from a biomass boiler or an anaerobic digestion system— (i) would be used for storing waste not produced by that boiler or system or for storing fuel not produced on land within the unit; or (ii) is or would be within 400 metres of the curtilage of a protected building. The building will not be used for storing fuel for or waste from a biomass boiler. The proposal satisfies criteria a) – k) therefore is defined as permitted development. Whether or not prior approval is needed In accordance with condition A2 (2) (i) the Local Authority must determine whether prior approval is required as to the siting, design and external appearance of the proposal. Siting – The proposed building is to be located immediately adjacent to other agricultural buildings within the unit on an existing area of hardstanding. The existing buildings will provide a partial level of screening of the development when viewed from Grindleton Road. It is therefore not considered that the proposed building would be of harm to the character of landscape of wider area as it is well related to existing built form. As such Prior approval is not required in terms of siting. Design / appearance- The proposed building is to be partially open sided and will be constructed using brown metal cladding, a concrete block plinth and metal box profile roof. This is typical for a modern agricultural building and is consistent with other buildings found within the farmstead. As such Prior approval is not required in terms of design and appearance. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposal does meet all of the criteria set out within Schedule 2, Part 6, Class A of the Town and Country Planning (General Permitted Development) Order 2015; therefore, prior approval is not required. The siting and design would be acceptable for the reasons stated above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  | Prior Approval Not Required. |  |  |  |  |  |  |  |  |  |  |  |