| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | BT |  |  |  | Date: |  | 11/2/25 |  | Manager: |  | LH | Date: | 13/2/25 |
| Application Ref: |  |  |  | 3/2024/1026 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 31/10/24 |  |  | Site Notice: |  | 16/1/25 |  |  |  |  |  |
| Officer: |  |  |  | BT |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Listed Building Consent for proposed alterations to wall adjacent to driveway and installation of three air source heat pumps. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | Waddington Hall, Clitheroe Road, Waddington, BB7 3HP. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| Waddington Parish Council: |  |  |  |  | Consulted 2/1/25 – no response received. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Archaeology: |  |  |  |  | No objections. |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | No objections. |  |  |  |  |  |  |  |  |  |
| RVBC Environmental Health: |  |  |  |  | No objections subject to condition. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| None. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable Development Key Statement EN5: Heritage Assets Policy DMG1: General Considerations Policy DMG2: Strategic Considerations Policy DMG3: Transport and Mobility Policy DME4: Protecting Heritage Assets Policy DME5: Renewable Energy Policy DME6: Water Management Policy DMH5: Residential And Curtilage Extensions Planning (Listed Buildings and Conservation Areas) Act Section 16 National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2024/1025: Planning Permission for proposed alterations to wall adjacent to driveway and installation of three air source heat pumps (Ongoing) 3/2024/0756: Listed Building Consent for proposed alterations to wall adjacent to driveway and installation of three air source heat pumps (Refused) 3/2024/0755: Planning Permission for proposed alterations to wall adjacent to driveway and installation of three air source heat pumps (Refused) 3/2024/0789: Approval of details reserved by condition 7 (alterations to west facing elevation of garage/home gym) and 9 written Scheme of Investigation of Listed Building Consent 3/2022/1144 (Ongoing) 3/2024/0790: Approval of details reserved by condition 5 (alterations to west facing elevation of garage/home gym) and 6 (written Scheme of Investigation) of Planning Permission 3/2022/1143 (Ongoing) 3/2022/1144: Application for Listed Building Consent for proposed external and internal alterations including the replacement of a conservatory with a new extension. Resubmission of 3/2022/0889 (Approved) 3/2022/1143: Proposed external and internal alterations including the replacement of a conservatory with a new extension. Resubmission of 3/2022/0839 (Approved) 3/2022/0889: Application for Listed Building Consent for proposed external and internal alterations including the replacement of a conservatory with a new extension (Withdrawn) 3/2022/0839: Application for Planning Permission for proposed external and internal alterations including the replacement of a conservatory with a new extension (Withdrawn) 3/1997/0509: Conversion of old service courtyard into indoor swimming pool (modification to 3/96/0011)(listed building consent) (Approved) 3/1997/0508: Conversion of old service courtyard into indoor swimming pool (modification to approved plans 3/96/0010) (Approved) 3/1996/0564: New bay, gable porch and construction of 3 car open cloistered garage and forecourt listed building consent (Approved) 3/1996/0563: Addition of new bay gable and porch to north elevation construction of 3 car open cloistered garage and forecourt (Approved) 3/1996/0011: Change of use from domestic garages into swimming pool, changing rooms etc. (app. For listed building consent) (Approved) 3/1996/0010: Convert existing outbuilding into swimming pool with changing room, steam room etc (Approved) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a detached historic property in Waddington. Access to the property is from Clitheroe Road from both a pedestrian access which leads to the principal South-western frontage of the property and a vehicular access which leads into a cobbled driveway area which adjoins the property’s North-eastern elevation. The application property has been subject to some C20th alterations and extensions including the addition of a conservatory extension, three bay garage extension and the formation of a cobbled driveway and landscaped area bound by a modern stone retaining wall. The property sits within a sizeable curtilage area comprised of manicured lawns, hard landscaping, extensive planting and mature trees. The application property lies within the centre of Waddington within the Waddington Conservation Area with the surrounding character of the area being predominantly residential. The wider area comprises a mixture of woodland, agricultural land and open countryside. Waddington Hall holds Grade II Listed Building status with the official Historic England listing description reading as follows: ‘House, possibly early C17th, rebuilt c.1900. Sandstone rubble with stone slate roof. H-plan. At the west the cross-wings project forward to form a courtyard. Both have gables with copings and the 3 walls facing the courtyard are topped by a balustrade with square balusters. The right-hand cross-wing has buttresses. On the ground floor is a 2-light mullioned window with inner hollow chamfer and outer chamfer. On the 1st floor is an early C18th cross window with an inner chamfer and an outer cyma-moulded surround. Above is a 2-light mullioned attic window of c,1900. All the windows of the left-hand cross-wing appear to be of c.1900. On the ground floor is a 5-light mullioned window with Tudor-arched heads to the lights. On the flat floor is a 10-light mullioned and transomed window. The attic has a 2-light mullioned window. The central section has a 2-light mullioned window with inner ovolo and outer rebate. To its right is a one-light window with moulded surround. On the 1st floor are 2 2-light double-chamfered mullioned windows, re-tooled and with cinquefoiled ogee heads to the lights. These are probably re-used from elsewhere. Across the angle with the right-hand cross-wing is a moulded doorway with Tudor-arched head. Above is a carved coat of arms under a shaped coping. At the rear all the windows appear to be of c.1900, in keeping. The south cross-wing has a 2-storey canted bay window. The south wall of this cross-wing has a C17th mullioned window of 4 lights with inner ovolo and outer chamfer. Interior not fully inspected, but rear of hall has wooden doorway with ogee head, probably C17th.’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Listed Building Consent is sought for the following works: - Partial reconfiguration of an existing retaining driveway wall to create extended driveway and parking area - Installation of three air source heat pumps and construction of new stone wall boundary (using materials from the partly demolished driveway wall) around proposed heat pumps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: The principle statutory duty under the Planning (Listed Building and Conservation Areas) Act 1990 (as amended by s.58B (1) of Levelling-up and Regeneration Act 2023) is to preserve or enhance the special character of heritage assets, including their setting. LPAs should, in coming to decisions, consider the principle Act which states the following; Listed Buildings – Section 66(1) (as amended by s.58B of Levelling-up and Regeneration Act 2023) In considering whether to grant planning permission for development which affects a listed building or its setting, the local planning authority shall have special regard to the desirability of preserving or enhancing the building or its setting. Under s.58B (2) this includes preserving or enhancing any feature, quality or characteristic of the asset or setting that contributes to the significance of the asset. Listed buildings - Section 16 (2) (as amended by s.58B of Levelling-up and Regeneration Act 2023) In considering whether to grant listed building consent for any works to a listed building the local planning authority shall have special regard to the desirability of preserving or enhancing the building. Under s.58B (2) this includes preserving or enhancing any feature, quality or characteristic of the asset or setting that contributes to the significance of the asset. Chapter 16 of the National Planning Policy Framework sets out expectations with regards to conserving and enhancing the historic environment. Applicants are required to describe the significance of any heritage assets affected, including any contribution made by their setting. Local Planning Authorities should consider any loss of historic fabric to constitute harm, but to make an assessment as to the significance of the asset and apply weight to its conservation accordingly. Accordingly, the proposed works to the Listed Building will be carefully assessed with respect to the duties above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon Listed Building: Paragraph 212 of the NPPF states: ‘When considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). This is irrespective of whether any potential harm amounts to substantial harm, total loss or less than substantial harm to its significance.’ In addition, Key Statement EN5 of the Ribble Valley Core Strategy states: ‘There will be a presumption in favour of the conservation and enhancement of the significance of heritage assets and their settings.’ Furthermore, Policy DME4 of the Core Strategy states: ‘Alterations or extensions to Listed Buildings or buildings of local heritage interest, or development proposals on sites within their setting which cause harm to the significance of the heritage asset will not be supported.’ Heritage impact is considered to be the potential level of harm upon the significance of a heritage asset caused by development proposals. The NPPF defines significance as ‘the value of a heritage asset to this and future generations because of its heritage interest’. Such interest can be archaeological, architectural, artistic or historic. Statements Of Heritage Significance, Historic England (2019) defines these as follows: ‘Archaeological Interest: There will be archaeological interest in a heritage asset if it holds, or potentially holds, evidence of past human activity worthy of expert investigation at some point.’ ‘Architectural And Artistic Interest: Interests in the design and general aesthetics of a place. They can arise from conscious design or fortuitously from the way the heritage asset has evolved. More specifically, architectural interest is an interest in the art or science of the design, construction, craftsmanship and decoration of buildings and structures of all types. Artistic interest is an interest in other human creative skills, like sculpture’. ‘Historic Interest: An interest in past lives and events (including pre-historic). Heritage assets can illustrate or be associated with them. Heritage assets with historic interest not only provide a material record of our nation’s history but can also provide meaning for communities derived from their collective experience of a place and can symbolise wider values such as faith and cultural identity.’ National Planning guidance requires applicants to describe the significance of any heritage assets affected, including any contribution made by their setting in order to allow the LPA to come to a judgment about the level of impact on that significance and therefore on the merits of the proposal. A heritage statement has been provided in support of the application which summarises the significance of the application building as follows: ‘The archaeological interest of the building is derived from its extant historic fabric and its evidence of historical alteration…the architectural and artistic interest of the building is derived from both its external and internal appearance…the external appearance has been altered to accommodate architectural embellishments such as the eaves balustrading, verge copings, the two-storey rear canted bay window and new window and door openings, as well as additions including the front entrance porch and the addition to the north of the building. The building is an imposing and impressive structure within the site and the wider village streetscape and its stylistic appearance is evidence of its high status and its historical development since its origins…the building’s associations with its past occupants appear to be well documented, particularly, its association with Henry VI, who was briefly hosted at the building.’ Accordingly, the above observations indicate that the significance of the application building is largely underpinned by its architectural interest (architectural embellishments referenced above) along with some additional archaeological interest (extant historic fabric) and historic interest (past associations with royal figures). In terms of justification for the proposed works, the application’s Heritage Statement states that the proposed alterations to the property’s driveway wall would improve the existing parking arrangement on site. It is further stated that installation of the three proposed air source heat pumps would be integral to providing a sustainable future for the application property and improving its energy efficiency. Making changes to Heritage Assets, Historic England (2016) advises: ‘It would not normally be good practice for new work to dominate the original asset or its setting in either scale, material or as a result of its siting.’ Furthermore, with specific regard to the siting of external services and equipment on Listed Buildings, the Institute Of Historic Building Conservation (2021) states: ‘The negative impact of some commonplace external fixtures on the character of a building is often ignored. These include meter boxes, burglar alarms, security lighting, stair lighting sensors, fixed and manoeuvrable video cameras, air source heat pumps, external central heating and other flues (both standard and balanced) and gas supply pipework. Ill-considered proposals for the location of any one of these fixtures or several in combination, particularly on principal facades and on smaller buildings, can have a particularly deleterious impact. They are only appropriate in visually unobtrusive positions where they do not cause damage to existing surface finishes and avoid any significant element of external architectural detail or decoration.’ In this instance, the proposal would involve the partial demolition of the property’s retaining driveway wall (approximately two thirds of the wall) in order to create additional vehicle parking space and to facilitate construction of the new walled area to house the proposed air source heat pumps. The wall to be partly demolished is a modern addition to the property (understood to have been constructed in 2002 around the same time that the property’s swimming pool extension was constructed) of little architectural merit therefore the partial demolition and subsequent reconfiguration of this feature would not result in the loss of any historic building fabric, nor would these works have any undue impact upon the architectural interest of the application property. These works would therefore have a neutral impact upon the significance of the application property. The proposed air source heat pumps would comprise a modern design with each of the units detailed in a grey metallic finish and with large circular fan openings to their front profiles. In addition, the air source heat pump units proposed for the property would comprise a width and height of 1.5 metres and 1.3 metres respectively and would therefore not be insignificant in terms of size and in this instance the proposal seeks to install a total of three such units. Notwithstanding their modern design, the proposed heat pumps would be sited within the confines of the reconfigured section of the existing driveway wall and as such would remain predominantly screened. It was noted in the assessment of previous application ref: 3/2024/0756 that the proposed heat pump units were to comprise a projection above the reconfigured wall area, thus rendering the units visible within the confines of the property’s driveway area which largely retains the historic theme of the application property and its surrounding grounds. In light of this, the current proposal seeks to install the proposed air source heat pumps below the ground level of the site’s driveway area through creating a lowered ground level within the confines of the configured wall area which in turn would allow the heat pumps to remain sited below the reconfigured wall level. These measures would afford an additional level of screening to the heat pumps compared with the previously refused proposal, resulting in the structures being sited in a visually unobtrusive position within the site. As such, the proposed microregeneration equipment would not dominate the heritage asset as a result of its siting, nor would the works proposed to facilitate the heat pumps result in any damage to historic surfaces. Taking account of all of the above, it is considered that the installation of the proposed air source heat pumps along with the proposed reconfiguration of the existing driveway wall would collectively have a neutral impact upon the significance of the heritage asset, with no harm arising to the heritage asset from the works proposed. The proposed development would therefore satisfy the requirements of Section 16 of the Listed Buildings and Conservation Areas) Act 1990, Paragraph 212 of the NPPF and Key Statement EN5 and Policy DME4 of the Ribble Valley Core Strategy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposed works would collectively have a neutral impact upon the significance of the heritage asset, with no harm arising to the heritage asset from the works proposed. The proposed development would therefore satisfy the requirements of Section 16 of the Listed Buildings and Conservation Areas) Act 1990, Paragraph 212 of the NPPF and Key Statement EN5 and Policy DME4 of the Ribble Valley Core Strategy. As such, for the above reasons and having regard to all material considerations and matters raised that the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That Listed Building Consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |