| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | EP |  |  |  | Date: |  | 28/02/2025 |  | Manager: |  | LH | Date: | 4/3/25 |
| Application Ref: |  |  |  | 2025/0015 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 30/01/2025 |  |  | Site Notice: |  | 30/01/2025 |  |  |  |  |  |
| Officer: |  |  |  | EP |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Proposed two-storey side extension with single-storey link to the garage, single-story extension to the rear and alterations. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | 1 Waters Edge, Whalley BB7 9UF. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| No comments received. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Highways: |  |  |  |  | No objection. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| No comments received. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable Development Policy DMG1: General Considerations Policy DMG2: Strategic Considerations Policy DMH5: Residential and Curtilage Extensions National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: No recent planning history. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a semi-detached property on Waters Edge in Whalley. The application dwelling benefits from a detached garage, which forms part of the block of garages utilised by other residents, which sits immediately adjacent to the application property. The application site falls within flood zones 2 and 3 but has no other designations. The surrounding are is predominantly residential in nature, being typified of varying styles of property. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Consent is sought for the construction of a two-storey side extension and single-story side extension to connect the main dwelling to the adjacent detached garage block. The extension will accommodate extended living space to the ground floor and an additional bedroom with ensuite to the first floor. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact Upon Residential Amenity: The proposed development will be located to the Northern side of the application dwelling where there is one immediately adjacent neighbouring receptor known as 28 Woodfield View. The single storey element, due to the orientation of the existing dwelling and garage, will not have any impact on the neighbouring dwelling and will be screened by existing boundary treatment for the receptors of Woodfield View. The proposed two-storey element has been carefully considered, in as much that whilst accommodating two floors, the eaves and ridge are low. As a result, the proposed two-storey extension will not result in any unacceptable sense of overbearing or substantial loss of light for any neighbouring receptors. There are two window openings proposed at first floor in the side elevation of the extension which faces a number of neighbouring properties along Woodfield View and Queen Stret. In order to prevent any potential for overlooking into residential amenity space, the windows at first floor will need to be obscurely glazed. This will be dealt with via relevant condition. It is therefore not considered that there would be any significant adverse impact on residential amenity subject to suitable conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visual Amenity/External Appearance: Policy DMG1 of the RVCS states that development must 1. Be of a high standard of building design which considers the 8 building in context principles (from the cabe/English heritage building on context toolkit. 2. Be sympathetic to existing and proposed land uses in terms of its size, intensity and nature as well as scale, massing, style, features and building materials. 3. Consider the density, layout and relationship between buildings, which is of major importance. Particular emphasis will be placed on visual appearance and the relationship to surroundings, including impact on landscape character, as well as the effects of development on existing amenities. The proposed extensions are to be located to the northern side elevation of the dwelling and as result will host a somewhat visually prominent position within the street scene. As such, careful consideration must be given into the impact of the development on the character of the area. The proposed two-story extension will benefit from a substantial set-down from the ridge of the existing dwelling, with a slight set-back so that the principal elevation of the extension falls behind the principal building line of the host dwelling. As a result, the extension will take an entirely subservient position to the application dwelling. The proposed single-storey link extension will be somewhat screened from view by the existing detached garage block. However, this part of the proposal is modest in terms of footprint and will integrate sufficiently into the dwelling. In respect to materials, the proposed development will be constructed using natural stone to the elevations, natural blue slate roof tiles and white uPVC windows and doors. This is consistent with the application dwelling and properties in the vicinity. Furthermore, it was observed on a site visit that there is a similar type of development at No.10 Waters Edge. As such it is not considered the proposal would read as out of character within the street scene. As such, the proposal is acceptable from a visual amenity perspective. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Highways and Parking: LCC Highways were consulted in relation to the proposal and raised no objection on highway safety or amenity grounds. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Landscape/Ecology: Biodiversity. The development is exempt from having to achieve the mandatory Biodiversity Net Gain requirement as it is a householder application. Bats. A preliminary bat roost assessment has been conducted at the application site on 17.10.2024. They survey concluded that no evidence of bats was recorded, and the building offers negligible roosting potential. Flood Risk. Constraint analysis shows that the property lies within Flood Zones 2 and 3. As such, the applicant is advised to adhere to the design and risk management measures recommended within the submitted flood risk assessment in addition to the Environment Agency’s ‘Householder and other minor extensions in Flood Zones 2 and 3’ guidance found online. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: As such, for the above reasons and having regard to all material considerations and matters raised that the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| That planning consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |