| Report to be read in conjunction with the Decision Notice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signed: | Officer: | BT |  |  |  | Date: |  | 7/4/25 |  | Manager: |  | LH | Date: | 9/4/25 |
| Application Ref: |  |  |  | 3/2025/0057 |  |  |  |  |  |  |  |  |  |  |
| Date Inspected: |  |  |  | 26/2/25 |  |  | Site Notice: |  | 26/2/25 |  |  |  |  |  |
| Officer: |  |  |  | BT |  |  |  |  |  |  |  |  |  |  |
| DELEGATED ITEM FILE REPORT: |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |
| Development Description: |  |  |  |  | Listed Building Consent for installation of underfloor heating at ground floor. |  |  |  |  |  |  |  |  |  |
| Site Address/Location: |  |  |  |  | Waddington Hall, Clitheroe Road, Waddington, BB7 3HP. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Parish/Town Council |  |  |  |  |  |  |  |  |  |
| Waddington Parish Council: |  |  |  |  | No objections; numerous queries raised. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Highways/Water Authority/Other Bodies |  |  |  |  |  |  |  |  |  |
| LCC Archaeology: |  |  |  |  | No objections subject to condition. |  |  |  |  |  |  |  |  |  |
| RVBC Environmental Health: |  |  |  |  | No objections. |  |  |  |  |  |  |  |  |  |
| CONSULTATIONS: |  |  |  |  | Additional Representations. |  |  |  |  |  |  |  |  |  |
| None. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RELEVANT POLICIES AND SITE PLANNING HISTORY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ribble Valley Core Strategy: Key Statement DS1: Development Strategy Key Statement DS2: Sustainable Development Key Statement EN5: Heritage Assets Policy DMG1: General Considerations Policy DMG2: Strategic Considerations Policy DME4: Protecting Heritage Assets Policy DME5: Renewable Energy Planning (Listed Buildings and Conservation Areas) Act Section 16 National Planning Policy Framework (NPPF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Relevant Planning History: 3/2024/1026: Listed Building Consent for proposed alterations to wall adjacent to driveway and installation of three air source heat pumps (Approved) 3/2024/1025: Planning Permission for proposed alterations to wall adjacent to driveway and installation of three air source heat pumps (Approved) 3/2024/0756: Listed Building Consent for proposed alterations to wall adjacent to driveway and installation of three air source heat pumps (Refused) 3/2024/0755: Planning Permission for proposed alterations to wall adjacent to driveway and installation of three air source heat pumps (Refused) 3/2024/0789: Approval of details reserved by condition 7 (alterations to west facing elevation of garage/home gym) and 9 written Scheme of Investigation of Listed Building Consent 3/2022/1144 (Approved) 3/2024/0790: Approval of details reserved by condition 5 (alterations to west facing elevation of garage/home gym) and 6 (written Scheme of Investigation) of Planning Permission 3/2022/1143 (Approved) 3/2022/1144: Application for Listed Building Consent for proposed external and internal alterations including the replacement of a conservatory with a new extension. Resubmission of 3/2022/0889 (Approved) 3/2022/1143: Proposed external and internal alterations including the replacement of a conservatory with a new extension. Resubmission of 3/2022/0839 (Approved) 3/2022/0889: Application for Listed Building Consent for proposed external and internal alterations including the replacement of a conservatory with a new extension (Withdrawn) 3/2022/0839: Application for Planning Permission for proposed external and internal alterations including the replacement of a conservatory with a new extension (Withdrawn) 3/1997/0509: Conversion of old service courtyard into indoor swimming pool (modification to 3/96/0011)(listed building consent) (Approved) 3/1997/0508: Conversion of old service courtyard into indoor swimming pool (modification to approved plans 3/96/0010) (Approved) 3/1996/0564: New bay, gable porch and construction of 3 car open cloistered garage and forecourt listed building consent (Approved) 3/1996/0563: Addition of new bay gable and porch to north elevation construction of 3 car open cloistered garage and forecourt (Approved) 3/1996/0011: Change of use from domestic garages into swimming pool, changing rooms etc. (app. For listed building consent) (Approved) 3/1996/0010: Convert existing outbuilding into swimming pool with changing room, steam room etc (Approved) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT OF PROPOSED DEVELOPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Description and Surrounding Area: The application relates to a detached historic property in Waddington. Access to the property is from Clitheroe Road from both a pedestrian access which leads to the principal South-western frontage of the property and a vehicular access which leads into a cobbled driveway area which adjoins the property’s North-eastern elevation. The application property has been subject to some C20th alterations and extensions including the addition of a conservatory extension, three bay garage extension and the formation of a cobbled driveway and landscaped area bound by a modern stone retaining wall. The property sits within a sizeable curtilage area comprised of manicured lawns, hard landscaping, extensive planting and mature trees. The application property lies within the centre of Waddington within the Waddington Conservation Area with the surrounding character of the area being predominantly residential. The wider area comprises a mixture of woodland, agricultural land and open countryside. Waddington Hall holds Grade II Listed Building status with the official Historic England listing description reading as follows: ‘House, possibly early C17th, rebuilt c.1900. Sandstone rubble with stone slate roof. H-plan. At the west the cross-wings project forward to form a courtyard. Both have gables with copings and the 3 walls facing the courtyard are topped by a balustrade with square balusters. The right-hand cross-wing has buttresses. On the ground floor is a 2-light mullioned window with inner hollow chamfer and outer chamfer. On the 1st floor is an early C18th cross window with an inner chamfer and an outer cyma-moulded surround. Above is a 2-light mullioned attic window of c,1900. All the windows of the left-hand cross-wing appear to be of c.1900. On the ground floor is a 5-light mullioned window with Tudor-arched heads to the lights. On the flat floor is a 10-light mullioned and transomed window. The attic has a 2-light mullioned window. The central section has a 2-light mullioned window with inner ovolo and outer rebate. To its right is a one-light window with moulded surround. On the 1st floor are 2 2-light double-chamfered mullioned windows, re-tooled and with cinquefoiled ogee heads to the lights. These are probably re-used from elsewhere. Across the angle with the right-hand cross-wing is a moulded doorway with Tudor-arched head. Above is a carved coat of arms under a shaped coping. At the rear all the windows appear to be of c.1900, in keeping. The south cross-wing has a 2-storey canted bay window. The south wall of this cross-wing has a C17th mullioned window of 4 lights with inner ovolo and outer chamfer. Interior not fully inspected, but rear of hall has wooden doorway with ogee head, probably C17th.’ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed Development for which consent is sought: Listed Building Consent is sought for the installation of underfloor heating at the property’s ground floor level. The proposed underfloor heating would be installed within the property’s monks room, morning room, main reception hall, drawing room and secondary reception hall. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principle of Development: The principle statutory duty under the Planning (Listed Building and Conservation Areas) Act 1990 (as amended by s.58B (1) of Levelling-up and Regeneration Act 2023) is to preserve or enhance the special character of heritage assets, including their setting. LPAs should, in coming to decisions, consider the principle Act which states the following; Listed Buildings – Section 66(1) (as amended by s.58B of Levelling-up and Regeneration Act 2023) In considering whether to grant planning permission for development which affects a listed building or its setting, the local planning authority shall have special regard to the desirability of preserving or enhancing the building or its setting. Under s.58B (2) this includes preserving or enhancing any feature, quality or characteristic of the asset or setting that contributes to the significance of the asset. Listed buildings - Section 16 (2) (as amended by s.58B of Levelling-up and Regeneration Act 2023) In considering whether to grant listed building consent for any works to a listed building the local planning authority shall have special regard to the desirability of preserving or enhancing the building. Under s.58B (2) this includes preserving or enhancing any feature, quality or characteristic of the asset or setting that contributes to the significance of the asset. Chapter 16 of the National Planning Policy Framework sets out expectations with regards to conserving and enhancing the historic environment. Applicants are required to describe the significance of any heritage assets affected, including any contribution made by their setting. Local Planning Authorities should consider any loss of historic fabric to constitute harm, but to make an assessment as to the significance of the asset and apply weight to its conservation accordingly. Accordingly, the proposed works to the Listed Building will be carefully assessed with respect to the duties above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Impact upon Listed Building: Paragraph 212 of the NPPF states: ‘When considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). This is irrespective of whether any potential harm amounts to substantial harm, total loss or less than substantial harm to its significance.’ In addition, Key Statement EN5 of the Ribble Valley Core Strategy states: ‘There will be a presumption in favour of the conservation and enhancement of the significance of heritage assets and their settings.’ Furthermore, Policy DME4 of the Core Strategy states: ‘Alterations or extensions to Listed Buildings or buildings of local heritage interest, or development proposals on sites within their setting which cause harm to the significance of the heritage asset will not be supported.’ Heritage impact is considered to be the potential level of harm upon the significance of a heritage asset caused by development proposals. The NPPF defines significance as ‘the value of a heritage asset to this and future generations because of its heritage interest’. Such interest can be archaeological, architectural, artistic or historic. Statements Of Heritage Significance, Historic England (2019) defines these as follows: ‘Archaeological Interest: There will be archaeological interest in a heritage asset if it holds, or potentially holds, evidence of past human activity worthy of expert investigation at some point.’ ‘Architectural And Artistic Interest: Interests in the design and general aesthetics of a place. They can arise from conscious design or fortuitously from the way the heritage asset has evolved. More specifically, architectural interest is an interest in the art or science of the design, construction, craftsmanship and decoration of buildings and structures of all types. Artistic interest is an interest in other human creative skills, like sculpture’. ‘Historic Interest: An interest in past lives and events (including pre-historic). Heritage assets can illustrate or be associated with them. Heritage assets with historic interest not only provide a material record of our nation’s history but can also provide meaning for communities derived from their collective experience of a place and can symbolise wider values such as faith and cultural identity.’ National Planning guidance requires applicants to describe the significance of any heritage assets affected, including any contribution made by their setting in order to allow the LPA to come to a judgment about the level of impact on that significance and therefore on the merits of the proposal. A heritage statement has been provided in support of the application which summarises the significance of the application building as follows: ‘The archaeological interest of the building is derived from its extant historic fabric and its evidence of historical alteration…the architectural and artistic interest of the building is derived from both its external and internal appearance…the external appearance has been altered to accommodate architectural embellishments such as the eaves balustrading, verge copings, the two-storey rear canted bay window and new window and door openings, as well as additions including the front entrance porch and the addition to the north of the building. The building is an imposing and impressive structure within the site and the wider village streetscape and its stylistic appearance is evidence of its high status and its historical development since its origins…the building’s associations with its past occupants appear to be well documented, particularly, its association with Henry VI, who was briefly hosted at the building.’ Accordingly, the above observations indicate that the significance of the application building is largely underpinned by its architectural interest (architectural embellishments referenced above) along with some additional archaeological interest (extant historic fabric) and historic interest (past associations with royal figures). The Institute Of Historic Building Conservation (2021) provides the following guidance with regards to proposals involving works to floorspaces within historic buildings: ‘Service routes should avoid the need to disturb part or all of a particular floor wherever possible…great care will be needed to permit the installation, upgrading or repair of services without damage to the floor covering.’ Similarly, Historic England guidance Making Changes to Heritage Assets (2016) advises: ‘The junction between new work and the existing fabric needs particular attention…historic flooring materials will often be of interest in themselves. Additional care is needed on lower floors to ensure that archaeological interest below the finished surface is not adversely affected by proposed works…new services, both internal and external, can have a considerable, and often cumulative, impact on the significance of a building and can affect significance if added thoughtlessly. The impact of necessary services can be minimised by avoiding damage to decorative features, by carefully routeing and finishing.’ In this instance, the proposal seeks to install underfloor heating to the ground floor level of the application property. These works would be facilitated through the removal of existing floor finishes (timber floorboards and flags) and the part removal of existing floor build up. Levelling screed would then be added to the remnant floor build up with insulation, underfloor heating boards and a new floor finish subsequently added above. It is understood that the existing timber floorboards and flags to be removed are non-historic floor finishes and numerous trial holes have also been made in preparation for the proposed works which have revealed a floor build up predominantly comprised of non-historic concrete slab along with some bitumen / terracotta build up. As such, no historic floor fabric would be disturbed or lost through the installation of the proposed underfloor heating. In addition, a damp proof membrane is to be installed below the new levelling screed within the property’s dining room and secondary reception hall which in turn would protect the remnant historic floor build up within each of these rooms. Furthermore, installation of the underfloor heating system would allow for a reduction in the use of larger modern radiators whilst improving energy efficiency within the property which in turn would deliver a minor visual enhancement to and optimise the ongoing maintenance of the heritage asset respectively. The initial response received from Lancashire County Council’s Archaeology team raised some concern with regards to potential difficulties in partially retaining the existing concrete slab to support the new floor build up as proposed however an appropriate methodology for carrying out these works has since been provided in correspondence from the applicant’s structural engineer, along with commentary to demonstrate that the remnant non-historic floor build up would be capable of supporting the load from the new floor build up and finish. This correspondence has subsequently been subject to further review from LCC Archaeology who have raised no further issues with the proposal subject to the imposition of a condition to secure a programme of archaeological monitoring and recording work. Taking account of all of the above, it is considered that the installation of the proposed underfloor heating system would preserve the significance of the heritage asset, with no harm arising to the heritage asset from the works proposed. The proposed development would therefore satisfy the requirements of Section 16 of the Listed Buildings and Conservation Areas) Act 1990, Paragraph 212 of the NPPF and Key Statement EN5 and Policy DME4 of the Ribble Valley Core Strategy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations/Consideration of Matters Raised/Conclusion: The proposed works would preserve the significance of the heritage asset, with no harm arising to the heritage asset from the works proposed. The proposed development would therefore satisfy the requirements of Section 16 of the Listed Buildings and Conservation Areas) Act 1990, Paragraph 212 of the NPPF and Key Statement EN5 and Policy DME4 of the Ribble Valley Core Strategy. As such, for the above reasons and having regard to all material considerations and matters raised that the application is recommended for approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATION: |  |  | That Listed Building Consent be granted subject to the imposition of conditions. |  |  |  |  |  |  |  |  |  |  |  |